3GPP TSG RAN WG2 #104	 R2-1817401

Spokane, US, 12 – 16 November, 2018

Agenda Item:	12.3.3 Handover robustness improvements

Source: 	Ericsson

Title:  	Repetition of RRC messages at handover

Document for:	Discussion and Decision

# Introduction

In the work item for Even further Mobility Enhancements in E-UTRAN [1], one objective is to improve the robustness at handover. This contribution proposes enhancements related to the handover robustness.

# Discussion

## Transmission of HO Command and Measurement Report

Handovers are normally triggered when the UE experiences poor radio conditions. If the UE enters poor radio conditions quickly the conditions may already be so poor that the actual handover procedure may be hard to execute. One reason for handover failures is problems with transmission of the RRC message containing the HO Command. In poor radio conditions the message is more often segmented, which increases the risk of retransmissions with an increased risk that the message doesn’t reach the UE in time. Failed transmission of Handover Command is a common reason for unsuccessful handovers.

If the RRC message containing the HO Command could be transmitted multiple times by the network without waiting for acknowledgement from the UE, it would increase the likelihood that the UE would receive the whole message faster. Therefore, mechanisms for repeating the message could be considered.

## Use of RLC UM for RRC messages

One existing mechanism which could be used for transmission of a message multiple times without waiting for acknowledgement is RLC UM. RLC UM is currently not used for transmission of RRC messages in LTE, but for the handover case it may be beneficial to use it. Especially when the message is segmented, the UE could receive one segment from one transmission, another segment from another transmission and add up the segments to a complete message without having to wait for an RLC retransmission of the complete message.

RLC AM is dependent on that RLC poll and status signalling is successful. The scheduler doesn’t know that the poll and status is waiting to be transmitted and there may be a timeout due to that the critical poll and status signalling fails. For RLC UM there is no dependency to poll and status signalling. The number of transmissions can also be determined without having to reconfigure the UE.

Taking these aspects into account the full message may reach the UE faster with RLC UM and the UE could go to the new cell faster. Such a procedure would most likely increase the success rate of handovers. The cost is that more radio resources are used for a short period of time, but the benefit is a more robust handover procedure.

UE

Serving gNB

Target gNB

HO decision

UP data

1. Measurement report

2. HO Request

Accept HO, build RRC config

3. HO Ack (incl. RRC config)

4. 3xCHO command

UE accesses the new cell

5. Synchronization and random access

6. HO confirm

7. HO completed

UP  data

RAN2 is therefore asked to consider allowing the use of RLC UM for transmission of RRCConnectionReconfiguration in certain cases. Also allowing the use of RLC UM for the UL MeasurementReport preceeding the HO command may be beneficial. The use of RLC UM doesn’t collide with other robustness improvements for handover, but can be used in addition to other improvements.

Proposal 1: Study the use of RLC UM for transmission of RRCConnectionReconfiguration.

Proposal 2: Study the use of RLC UM for transmission of MeasurementReport.

Reconfiguration of the SRB before transmission of HO Command is not considered an option, as it would add one RRC procedure to the signalling flow. However, four SRBs are currently defined in the RRC specification and one of them could possibly be configured with RLC UM and used for this purpose, if not already used for other purposes.

## Other types of repetitions

RLC UM is one way to transmit the RRC messages using repetitions. There could however be other possibilities to repeat the messages. It is proposed to study mechanisms that could be used for repeating RRCConnectionReconfiguration and MeasurementReport.at handover.

Proposal 3: Study the use of repetitions for transmission of RRCConnectionReconfiguration.

Proposal 4: Study the use of repetitions for transmission of MeasurementReport.

# Summary

RAN2 is kindly asked to discuss the following observations and proposals:

Proposal 1: Study the use of RLC UM for transmission of RRCConnectionReconfiguration.

Proposal 2: Study the use of RLC UM for transmission of MeasurementReport.

Proposal 3: Study the use of repetitions for transmission of RRCConnectionReconfiguration.

Proposal 4: Study the use of repetitions for transmission of MeasurementReport.

# References

- RP-171489, Work Item on Even further Mobility Enhancements in E-UTRAN, China Telecom, RAN#81, September 2018