3GPP TSG-RAN WG2 #104	R2-1817556

Spokane, USA, 12th November – 16th November 2018

Agenda Item: 12.2.7

Source:          Ericsson

Title:               Supporting Presence of NRS on a non-anchor carrier for paging in NB-IoT

Document for:	Discussion, Decision

# Introduction

In the Rel-16 work item description (WID) of Additional enhancements for NB-IoT, one of the objectives is to improve the multi-carrier operation as follows [1].

- Specify signalling to indicate on a non-anchor carrier for paging a set of subframes which will contain NRS even when no paging NPDCCH is transmitted [RAN1, RAN2, RAN4]

In RAN1#94, the followings were agreed in RAN1

“

Agreement

The presence of NRS on subframes which will contain NRS even when no paging NPDCCH is transmitted is enabled by broadcast signalling.

•	Above applies only for non-anchor carriers

Agreement

For the objective on presence of NRS on a non-anchor carrier for paging, consider the following use case:

•	Early termination of NPDCCH / WUS

Above is not intended to restrict the study of other use cases in other WGs which may or may not have RAN1 specification impact

”

In this contribution, we discuss the RAN2 specification impacts based upon RAN1 agreement.

# Discussion

One option to capture RAN1 agreement to broadcast the presence of NRS on subframes which will contain NRS even when no paging NPDCCH is transmitted in 36.331 [2] is as below.

DL-CarrierConfigCommon-NB information elements

-- ASN1START

DL-CarrierConfigCommon-NB-r14 ::=	SEQUENCE {

dl-CarrierFreq-r14					CarrierFreq-NB-r13,

downlinkBitmapNonAnchor-r14			CHOICE {

useNoBitmap-r14						NULL,

useAnchorBitmap-r14					NULL,

explicitBitmapConfiguration-r14		DL-Bitmap-NB-r13

},

dl-GapNonAnchor-r14					CHOICE {

useNoGap-r14						NULL,

useAnchorGapConfig-r14				NULL,

explicitGapConfiguration-r14		DL-GapConfig-NB-r13

},

inbandCarrierInfo-r14				SEQUENCE {

samePCI-Indicator-r14				CHOICE	{

samePCI-r14							SEQUENCE {

indexToMidPRB-r14					INTEGER (-55..54)

},

differentPCI-r14					SEQUENCE {

eutra-NumCRS-Ports-r14				ENUMERATED {same, four}

}

}	OPTIONAL,		-- Cond anchor-guardband-or-standalone

eutraControlRegionSize-r14			ENUMERATED {n1, n2, n3}

}	OPTIONAL,		-- Cond non-anchor-inband

nrs-PowerOffsetNonAnchor-r14		ENUMERATED {dB-12, dB-10, dB-8, dB-6,

dB-4, dB-2, dB0, dB3}	DEFAULT dB0,

...

[[	dl-GapNonAnchor-v1530				DL-GapConfig-NB-v1530	OPTIONAL 	-- Cond TDD

]],

[[

nrs-SubFrames-r16             BIT STRING (SIZE (10))	OPTIONAL

]]

}

-- ASN1STOP

DL-CarrierConfigCommon-NB field descriptions

Nrs-SubFrames Indicates presence of NRS in a SubFrame. Each bit corresponds to a subframe. Bit value 0 indicates nrs is absent whereas bit balue 1 indicates NRS is present

Above extension of DL-CarrierConfigCommon-NB information elements can be used to indicate NRS Subframe.

The non-anchor carrier configured for paging should be updated to contain this information. However, the exact subframes will be known only after RAN4 concludes their study. It would depend upon RAN4 as they may decide to use a fixed subframe or first x subframes etc.

Further, depending upon RAN1 discussion, there may be a need to define a capability bit for Early termination of NPDCCH / WUS. This would then impact 36.306 and UE capability part for 36.331.

There may be a need to update 36.306 and UE capability part for 36.331 depending upon RAN1 discussion.

Wait for RAN1 and RAN4 to progress on this and decide upon the exact details of subframes needed to transmit NRS.

NRS on a non-anchor carrier may not be transmitted all the time. Further, it may not be available in all the paging carrier. In such case, it may not be suitable for RRM measurements. If NRS is to be used for RRM measurements, RAN4 needs to be consulted.

NRS may not be suitable for RRM purpose.

# Conclusion

In section 2 we made the following observations:

Observation 1	Above extension of DL-CarrierConfigCommon-NB information elements can be used to indicate NRS Subframe.

Observation 2	There may be a need to update 36.306 and UE capability part for 36.331 depending upon RAN1 discussion.

Observation 3	NRS may not be suitable for RRM purpose.

Based on the discussion in section 2 we propose the following:

Proposal 1	Wait for RAN1 and RAN4 to progress on this and decide upon the exact details of subframes needed to transmit NRS.

# References

RP-181674, “WID revision: Additional enhancements for NB-IoT, Source Huawei, 3GPP TSG RAN Meeting #81, Gold Coast, Australia, September 2018.

36.331 v15.3.0