3GPP TSG-RAN WG2 #104	R2-1817557

Spokane, USA, 12th November – 16th November 2018

Agenda Item:	12.1.6

Source:	Ericsson

Title:	Support of quality report in Msg3 for MTC

Document for:	Discussion, Decision

# Introduction

In the Rel-16 work item description (WID) on “Additional MTC enhancements for LTE”, the following objectives are listed as improvements of DL transmission efficiency and/or UE power consumption [1].

- Specify support for mobile-terminated (MT) early data transmission (EDT) [RAN2, RAN3]

- Specify quality report in MSG3 at least for EDT [RAN1, RAN2]

- Specify MPDCCH performance improvement by using CRS at least for connected mode [RAN1, RAN2, RAN4]

- Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4]

In this paper we discuss the support of Msg3 quality reporting in Msg3 at least for EDT.

# Discussion

Recently for NB-IoT the support for anchor carrier measurement reporting in Msg3 has been specified. 4 bits have been used as below.

-- ASN1START

CQI-NPDCCH-NB-r14 ::=	ENUMERATED {

noMeasurements, candidateRep-A, candidateRep-B, candidateRep-C,

candidateRep-D, candidateRep-E, candidateRep-F, candidateRep-G,

candidateRep-H, candidateRep-I, candidateRep-J, candidateRep-K,

candidateRep-L}

-- ASN1STOP

Further, a short CQI reporting is also supported in RRCConnectionReestablishmentRequest-NB as below.

-- ASN1START

CQI-NPDCCH-Short-NB-r14 ::=	ENUMERATED {

noMeasurements, candidateRep-1, candidateRep-2, candidateRep-3}

-- ASN1STOP

It is expected that similar measurement reporting as above would be required for MTC. However, it is up to RAN1 and RAN4 to decide.

From RAN2 perspective, the below messages, for example would be impacted for EDT and non-EDT case respectively [2].

RRCEarlyDataRequest message

-- ASN1START

RRCEarlyDataRequest-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

rrcEarlyDataRequest-r15				RRCEarlyDataRequest-r15-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCEarlyDataRequest-r15-IEs ::=	SEQUENCE {

s-TMSI-r15						S-TMSI,

establishmentCause-r15			ENUMERATED {	mo-Data-r15, delayTolerantAccess-r15},

dedicatedInfoNAS-r15				DedicatedInfoNAS,

nonCriticalExtension				SEQUENCE {}			OPTIONAL

}

-- ASN1STOP

-- ASN1START

RRCConnectionRequest ::=			SEQUENCE {

criticalExtensions					CHOICE {

rrcConnectionRequest-r8				RRCConnectionRequest-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionRequest-r8-IEs ::=		SEQUENCE {

ue-Identity							InitialUE-Identity,

establishmentCause					EstablishmentCause,

spare								BIT STRING (SIZE (1))

}

InitialUE-Identity ::=				CHOICE {

s-TMSI								S-TMSI,

randomValue							BIT STRING (SIZE (40))

}

EstablishmentCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1}

-- ASN1STOP

It should be possible to extend the above message using non-critical extension so that UE can include the measurement results. However, the parameters (granularity and number of bits required) should be provided by RAN1/RAN4.

The EDT Msg3 may have more flexibility because the UL grant may be high as data also needs to be included.

However, legacy Msg3 for MTC may have some issue with the existing RRC msg3 size limitation. It should be possible to extend the RRC msg3 size and thus should be possible to have granular DL quality reporting.

From 36.213 below tables, it is possible to have large TBS for Msg3.

Table 3: TBS table for Msg3 in CE mode A

| IMCS | NPRB |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 6 |
| 0 | 16 | 32 | 56 | 152 |
| 1 | 24 | 56 | 88 | 208 |
| 2 | 32 | 72 | 144 | 256 |
| 3 | 40 | 104 | 176 | 328 |
| 4 | 56 | 120 | 208 | 408 |
| 5 | 72 | 144 | 224 | 504 |
| 6 | 328 | 176 | 256 | 600 |
| 7 | 104 | 224 | 328 | 712 |

Table 4: TBS table for Msg3 in CE mode B

| ITBS | NPRB |  |
| --- | --- | --- |
|  | 1 | 2 |
| 0 | 56 | 152 |
| 1 | 88 | 208 |
| 2 | 144 | 256 |
| 3 | 176 | 328 |

There are enough bits in Msg3 for channel quality reporting in both the non-EDT case and the EDT case.

In RAR or System Info Broadcast, when the network has set a bit to send DL Quality Report; UE can then send RRC Connection Request with Rel-16.

-- ASN1START

RRCConnectionRequest ::=			SEQUENCE {

criticalExtensions					CHOICE {

rrcConnectionRequest-r8				RRCConnectionRequest-r8-IEs,

c1			CHOICE {

rrcConnectionRequest-r16			RRCConnectionRequest-r16-IEs

criticalExtensionsFuture            SEQUENCE

}

}

}

RRCConnectionRequest-r8-IEs ::=		SEQUENCE {

ue-Identity							InitialUE-Identity,

establishmentCause					EstablishmentCause,

spare								BIT STRING (SIZE (1))

}

RRCConnectionRequest-r16-IEs ::=		SEQUENCE {

ue-Identity							InitialUE-Identity,

establishmentCause					EstablishmentCause,

dl-MeasurementReport				DL-MeasurementReport

spare								BIT STRING (SIZE (FFS))

}

InitialUE-Identity ::=				CHOICE {

s-TMSI								S-TMSI,

randomValue							BIT STRING (SIZE (40))

}

EstablishmentCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1}

-- ASN1STOP

Wait for RAN1 and RAN4 to progress on this to decide upon the exact number of bits and granularity needed to report the measurement quality and update the above messages accordingly.

# Conclusion

In section 2 we made the following observations:

Observation 1	It is expected that similar measurement reporting as above would be required for MTC. However, it is up to RAN1 and RAN4 to decide.

Observation 2	There are enough bits in Msg3 for channel quality reporting in both the non-EDT case and the EDT case.

Based on the discussion in section 2 we propose the following:

Proposal 1	Wait for RAN1 and RAN4 to progress on this to decide upon the exact number of bits and granularity needed to report the measurement quality and update the above messages accordingly.

# References

RP-181878, “Revised WID: Additional MTC enhancements for LTE”, Ericsson, RAN #81, Gold Coast, Australia, September 2018.

3gpp 36.331, RRC v15.3.0