3GPP TSG-RAN WG2 #104	R2-1817630

Spokane, US, 12th – 16th November 2018	(re-submission of R2-1814448)

Agenda Item:	10.4.1.4.1

Source:	Ericsson

Title:	Measurement related actions upon handover

Document for:	Discussion, Decision

# Introduction

In LTE, a set of measurement related actions are specified when the UE performs handover and re-establishment. During email discussion [99bis#20][NR] RRM (Ericsson) that was captured as an open issue in the TP draft, and as discussions have progressed in RAN2#AdHoc NR 2018-1, that was agreed not to be supported.

However, although the exact actions proposed in LTE were agreed not to be supported in NR, we have identified another potential issue related to measurements during re-establishments and handovers.

# Discussion

## Measurement reporting on handover execution

In LTE, the UE shall always perform RSRP and RSRQ measurements for each configured serving cell. Network provides a measurement object for each serving frequency the UE is expected to measure. According to 36.331, E-UTRAN applies measurement configuration to ensure that, whenever the UE has a measConfig, it includes a measObject for each serving frequency. And, for serving frequencies, set the EARFCN within the corresponding measObject according to the band as used for reception/ transmission.

The list of measurement objects, including the measurement objects associated to serving frequencies, can be provided to the UE within measObjectToAddModList, which is part of measConfig (possibly transmitted in RRCConnectionReconfiguration and RRCConnectionResume). Hence, upon handovers and re-establishments, the network should ensure that a measObjectId corresponding to each target serving frequency is configured (either via full or delta RRC re-configuration). And, when changing the band while the physical frequency remains unchanged, E-UTRAN releases the measObject corresponding to the source frequency and adds a measObject corresponding to the target frequency (i.e. it does not reconfigure the measObject). Upon that, in addition, the UE removes existing measurement reporting entries, stops periodical reporting timer or timer T321 (whichever one is running), as well as associated information (e.g. timeToTrigger) for all configured measurements and release activated measurement gaps, if any.

In LTE, upon handovers and re-establishment, the UE removes existing measurement reporting entries, stops periodical reporting timer or timer T321 (whichever one is running), as well as associated information (e.g. timeToTrigger) for all configured measurements and release activated measurement gaps, if any.

One of the reasons the UE shall delete previously configured measurements is that the target node might already have RSRP/RSRQ/SINR measurements as source might have sent them to target via HandoverPreparationInformaiton, as part of the RRM config, within CandidateCellInfoList. And, as in LTE the quality of these cells might not change dramatically from the time the UE sends the latest measurement reports to source (that would be sent to target) until the time the handover execution is performed.

In NR, as in LTE, target may also benefit in knowing previously performed measurements configured by source. And, it has been agreed that the source can send both cell and beam measurements in HandoverPreparationInformation, as part of the RRM config, as agreed in RAN2#99 Berlin:

2.2	As in LTE, the HandoverPreparationInformation to be transmitted from the source gNB to the target gNB can include the AS configuration, the RRM configuration and the AS context (including information necessary to handle handover failures). The details of the content of each IE are FFS.

4.2: As in LTE, to support CA case, the RRM configuration can include the list of best cells on each frequency for which measurement information is available.

5	Available beam measurement information can be part of the RRM configuration of the HandoverPreparationInformation message if beam measurement information (i.e. beam indexes and optionally measurement results) have been configured by the source gNodeB to be reported by a UE. That information is not a mandatory part of the HandoverPreparationInformation message

6	The RRM configuration can include both beam measurement information (for layer 3 mobility) associated to SS Block(s) and CSI-RS(s) for the reported cell (or cells depending on outcome of FFS above) if both types of measurements are available.

…

In NR, as in LTE, the source can provide beam and cell measurements information to target as part of RRM-config in Handover Preparation Information.

The main purpose of the cell measurements is to enable the target to possibly setup DC and/or CA, while beam measurements can be used in target to allocate contention free resources per beam or optimize handover decisions (e.g. by prioritizing cells with more stable beams). In addition, these beam measurements could also be used by target as input for the beam management configuration(s) to be provided to the UE e.g. to limit the amount of L1 beam reporting after the handover, a limited amount of CSI-RS resources are configured, and QCL with a subset of SS/PBCK blocks.

However, while previously performed cell measurements can be considered stable from the handover preparation until the handover execution, it has been acknowledged that beam measurements might not i.e. the best beams at the moment the UE sends the first measurement reports that may trigger a handover to the moment the UE finally receives the handover command may be different. And, for that reason, a RACH fallback mechanism has been agreed where the UE can use common resources when beam selection upon handovers lead to a beam that does not have contention free resources associated.

In NR, best beams of target cell may change from the time the target receives the HO preparation to the time the UE performs HO execution. Hence, a RACH fallback mechanism was agreed in RAN2.

When that RACH fallback is used, the UE most likely has updated beam measurements and, it could be the case that beam management configurations provided in the handover command, possibly based on previous measurements, might not be the most optimized one. Hence, it seems natural to say that the target could benefit from beam measurements performed between the time the UE has send the last measurement report until the time it has performed a handover execution.

In NR, the target can benefit of knowing updated beam measurements performed between the time the UE has send the last measurement report until the time the UE has performed a handover execution.

One aspect to note is that random access has a beam selection component i.e. upon receiving the preamble during HO execution, the network knows which DL beam has been selected by the UE. And, thanks to the agreed absolute threshold, the network also knows that its beam quality shall be above the configured threshold. Hence, to certain extent, upon handover execution the target cell obtains an update beam measurement information in the sense that it knows at least one suitable beam to send the RAR and to estimate whether its previously provided beam management configuration is optimal or not.

In NR, random access provides the target with some beam measurement information provided by the UE, thanks to the preamble being mapped to a DL beam.

While that beam information is sufficient to enable the network to send the RAR in a suitable DL beam, so the UE can complete the handover, that might not be sufficient to enable the target to confirm whether the beam management configuration, provided in the HO command, is still optimized or not. Hence, RAN2 should further discuss whether beam measurements associated to the target cell could be provided to the target upon handover execution to enable the target to decide whether the beam management configuration provided in the handover command is still suitable or whether that needs to be re-configured. Hence, we propose the following:

Discuss whether the UE can report beam measurements upon handover execution.

# Conclusion

In section 2 we made the following observations:

Observation 1	In LTE, upon handovers and re-establishment, the UE removes existing measurement reporting entries, stops periodical reporting timer or timer T321 (whichever one is running), as well as associated information (e.g. timeToTrigger) for all configured measurements and release activated measurement gaps, if any.

Observation 2	In NR, as in LTE, the source can provide beam and cell measurements information to target as part of RRM-config in Handover Preparation Information.

Observation 3	In NR, best beams of target cell may change from the time the target receives the HO preparation to the time the UE performs HO execution. Hence, a RACH fallback mechanism was agreed in RAN2.

Observation 4	In NR, the target can benefit of knowing updated beam measurements performed between the time the UE has send the last measurement report until the time the UE has performed a handover execution.

Observation 5	In NR, random access provides the target with some beam measurement information provided by the UE, thanks to the preamble being mapped to a DL beam.

Based on the discussion in section 2 we propose the following:

Proposal 1	Discuss whether the UE can report beam measurements upon handover execution.