3GPP TSG-RAN WG2 Meeting #104                                	R2-1818246

Spokane, USA, 12th - 16th November 2018

Agenda item:	11.6.3.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on timer impacts in NTN

Document for:	Discussion and Decision

Introduction

In the RAN plenary #80 meeting, a new RAN3 SI on “solutions evaluation for NR to support Non Terrestrial Network” was approved [1]. One of the objectives is related to the propagation delay which is stated as follows:

- Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2]

During RAN2#130bis meeting, the impacts of propagation delay on user plane are discussed and some general agreements are achieved as follows.

UP Impacts to study 1. DRX 2. HARQ 3. Random access response 4. RLC/PDCP reordering (e.g. timers and SN space) 5. SDAP => no impact

In this contribution, we will further discuss the impact of high propagation delay in Non Terrestrial Network (NTN) based on the above agreements.

Discussion

The relation between the altitude of satellite and propagation delay is showed in the following tables [2].

Table 1 Propagation delays for different NGEO satellites (altitude and payload types)

|  |  | LEO at 600 km |  | LEO at 1500 km |  | MEO at 10000 km |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Elevation angle | Path | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) |
| UE: 10° | satellite - UE | 1932.24 | 6,440 | 3647.5 | 12,158 | 14018.16 | 46.727 |
| GW: 5° | satellite - gateway | 2329.01 | 7.763 | 4101.6 | 13.672 | 14539.4 | 48.464 |
| 90° | satellite - UE | 600 | 2 | 1500 | 5 | 10000 | 33.333 |
| Bent pipe satellite |  |  |  |  |  |  |  |
| One way delay | Gateway-satellite_UE | 4261.2 | 14.204 | 7749.2 | 25.83 | 28557.6 | 95.192 |
| Round Trip Delay | Twice | 8522.5 | 28.408 | 15498.4 | 51.661 | 57115.2 | 190.38 |
| Regenerative satellite |  |  |  |  |  |  |  |
| One way delay | Satellite -UE | 1932.24 | 6.44 | 3647.5 | 12.16 | 14018.16 | 46.73 |
| Round Trip Delay | Satellite-UE-Satellite | 3864.48 | 12.88 | 7295 | 24.32 | 28036.32 | 93.45 |

Table 2 Propagation delays for GEO satellite at 35786 km

|  | GEO at 35786 km |  |  |
| --- | --- | --- | --- |
| Elevation angle | Path | D (km) | Time (ms) |
| UE :10° | satellite - UE | 40586 | 135.286 |
| GW : 5° | satellite - gateway | 41126.6 | 137.088 |
| 90° | satellite - UE | 35786 | 119.286 |
| Bent Pipe satellite |  |  |  |
| One way delay | Gateway-satellite_UE | 81712.6 | 272.375 |
| Round trip Time | Twice | 163425.3 | 544.751 |
| Regenerative Satellite |  |  |  |
| One way delay | Satellite -UE | 40586 | 135.286 |
| Round Trip Time | Satellite-UE-Satellite | 81172 | 270.572 |

As shown in above tables, the round trip delay (RTD) of MEO at 10000 km is up to 93.45ms while the round trip delay for GEO satellite arrives at 544.751ms. The overly large RTD has negligible impacts to the timers in the communication system.

We think it is a bit early to make conclusion on every timer at this stage, since some enhancements/modifications maybe needed in NTN and these enhancements/modifications may impact timers. For example, if HARQ is not supported then there is no need to have some timers at all, e.g. retransmission timer, reordering timer, etc.

But we think it is possible to conclude on some general principles on timer impacts in NTN. Considering the impacts due to long RTD, we think in general there are 2 types of timers may be impacted:

Type 1: Timers which need to cover the RTD(s).

This kind of timers are mainly used to control the UE to perform (no perform) some procedures, which may cover the entire RTD(s). Thus, they have to be running for the entire RTD(s).

For example:

- T300 and T301 in RRC: T300 and T301 are used to avoid too many random access re-attempts when the network is overloaded or the wireless environment is extremely bad. Both of them should cover the whole random access procedure, including Msg1 – Msg4 transmissions and several possible re-attempts. Backoff also needs to be considered. Apparently they need to cover more than one RTDs.

- sr-ProhibitTimer in MAC: During the SR transmission, no extra SR is expected to be triggered. Thus, sr-ProhibitTimer needs to cover at least one RTD to allow the gNB to provide UL grant.

Considering that RTD in NTN is longer than Rel-15 NR system, this kind of timers should be extended. In the following table, some more timers needing extension are listed. Note that the table may not cover all the timers but only a portion.

Table 1: Timers needing extension

| MAC timers | RLC timers | PDCP timers | RRC timers |
| --- | --- | --- | --- |
| timeAlignmentTimer sr-ProhibitTimer phr-ProhibitTimer drx-HARQ-RTT-TimerDL (if HARQ is supported[3]) … | t-Reassembly t-PollRetransmit t-StatusProhibit … | t-Reordering (if HARQ is supported[3]) … | T300 T310 T311 … |

Proposal 1: RAN2 to consider extension for timers covering RTD(s) in NTN.

Type 2: Timers using to control monitoring of PDCCH.

This kind of timers (most of them are MAC timers) are used to control the UE to monitor the PDCCH. During the timers are running, the gNB has opportunities to schedule the UE via PDCCH. The maximum value of this kind of timers mainly depends on SCS (slot length, symbol length, etc.). If there is no change on SCS in NTN, seems no need to extend this kind of timers.

But due to the long RTD in NTN, the triggering/starting point of these timers may need to be changed. For example:

- RAR window (not a timer but very similar): The UE needs to monitor PDCCH for RAR within the window after preamble transmission. Due to long RTD in NTN, the offset between preamble transmission and the first possible PDCCH occasion (starting of RAR window) is longer.

Considering the RTD is enlarged, UE doesn’t have to start this kind of timers right away but can wait for a suitable time (to cover RTD) before starting the timers. In other words, an appropriate offset is needed before the starting point of the timers.

Table 2: Timers needing offset before starting

| MAC timers | RLC timers | PDCP timers | RRC timers |
| --- | --- | --- | --- |
| RAR window configuredGrantTimer drx-RetransmissionTimerDL/UL (if HARQ is supported[3]) … | … | … | … |

Proposal 2: RAN2 to consider introducing offset for timers related to PDCCH monitoring in NTN.

Conclusion

In this contribution, we discussed the timer impacts of high propagation delay in Non Terrestrial Network (NTN), and we get the following proposals:

Proposal 1: RAN2 to consider extension for timers covering RTD(s) in NTN.

Proposal 1: RAN2 to consider extension for timers covering RTD(s) in NTN.

References

[1]		RP-181370, Study on solutions evaluation for NR to support Non Terrestrial Network.

[2]		TR 38.811, Study on New Radio (NR) to support non terrestrial networks.

[3]		R2-1818247, Further discussion on MAC impacts in NTN, Huawei, HiSilicon