3GPP TSG-RAN WG2 Meeting #104	R2-1818293

Spokane, USA, 12th - 16th November 2018

______________________________________________________________________Agenda item: 9.10.2

Source: LG Electronics, ITL, Ericsson

Title: Discussion on Cell-specific SLSS_TxDisabled

Document for: Discussion and decision

- Introduction

This contribution discusses the UE behavior according to the slss-TxDisabled parameter.

- Discussion

In RAN2 #103, TS36.331 CR for eV2X endorsement was approved [1]. New SIB for eV2X (SIB26) and related procedures have been added, and synchronization configuration and procedure for sidelink CA has been also added. Here, we discuss slss-TxDisabled related UE behavior. In RAN1 # 92bis, RAN1 has sent an LS [2] for RRC parameter to RAN2, and RAN1 was indicating that it is UE specific for SL_SLSS_Tx_Enabled as follows,

| Parameter name in specification | New or existing parameter | Parameter name in text | Description | Value range | Default value | UE specific/Cell specific |
| --- | --- | --- | --- | --- | --- | --- |
| SL_SLSS_Tx_Enabled | New |  | Indication of non-sync carrier with configured synchronization resources | {0.1} |  | UE specific |

Currently, in TS36.331, SL_SLSS_Tx_Enabled has been renamed to slss-TxDisabled by a description of backward operation, and we understand that this parameter can be used via RRCConnectionReconfiguration or SL-V2X-Preconfiguration as follows,

5.10.7.2 in TS36.331

2> if syncFreqList and slss-TxMultiFreq are included in RRCConnectionReconfiguration or in SystemInformationBlockType26; and if the UE has selected a frequency other than the concerned frequency as the synchronisation carrier frequency; and if slss-TxDisabled corresponding to the concerned frequency is not configured in RRCConnectionReconfiguration; and if the concerned frequency has been selected for V2X sidelink communication transmission as specified in TS 36.321 [6] and is included in syncFreqList; and if UE is capable of SLSS/PSBCH transmission on the concerned frequency: <omitted> 3> if syncFreqList and slss-TxMultiFreq are included in SL-V2X-Preconfiguration, and if the UE has selected a frequency other than the concerned frequency as the synchronisation carrier frequency; and if slss-TxDisabled corresponding to the concerned frequency is not configured in SL-V2X-Preconfiguration; and if the concerned frequency has been selected for V2X sidelink communication transmission as specified in TS 36.321 [6] and included in syncFreqList; and if the UE is capable of SLSS/PSBCH transmission on the frequency:

However, in our understanding, slss-TxDisabled is also signaled in the SIB. In the current specification implementation, slss-TxDisabled is added to SL-SyncConfig in sidelink information elements, and SLSS-TxDisabled is signaled to SIB 21 via SL-SyncConfigListV2X-r14. It is desirable that slss-TxDisabled is signaled in the SIB for RRC idle UE behavior. If slss-TxDisabled is signaled only in RRCConnectionReconfiguration, only RRC connected UEs will perform slss-TxDisabled-related operations. In other words, RRC idle UEs (i.e. mode 4 UE) in coverage may transmit SLSS/PSBCH on unnecessary carriers because slss-TxDisabled related operation is not defined. Since SLSS-TxDisabled is also signaled in the SIB in the current specification implementation as above mentioned, specification needs to be changed from the procedure perspective so that slss-TxDisabled can be used by RRC idle UEs.

This misbehavior is caused by that RAN1 indicated that slss-TxDisabled only for UE specific. According to previous RAN1 meeting [4], RAN1 conclude that UE behaviour related to parameter slss-TxDisabled should be discussed in RAN2. Hence, RAN2 can handle this issue by ourselves.

Our discussion can be summarized as follows,

Observation 1: In current TS36.331, UE behaviour related to slss-TxDisabled for RRC idle UE is not defined.

Observation 2: In current TS36.331 specification implementation, slss-TxDisabled is already signaled via SIB 21.

Observation 3: If we configured slss-TxDisabled for cell specific, there is no TS36.331 specification and ASN.1 impact.

Proposal 1:  It is necessary that slss-TxDisabled can be configured either UE specific or cell specific.

Proposal 2: For slss-TxDiabled operation for RRC idle UE, let’s reflect the CR [3].

- Conclusion

This contribution discussed UE behavior according to the slss-TxDisabled parameter. The discussions can be summarized as follows:

Observation 1: In current TS36.331, UE behaviour related to slss-TxDisabled for RRC idle UE is not defined.

Observation 2: In current TS36.331 specification implementation, slss-TxDisabled is already signaled via SIB 21.

Observation 3: If we configured slss-TxDisabled for cell specific, there is no TS36.331 specification and ASN.1 impact.

Proposal 1:  It is necessary that slss-TxDisabled can be configured either UE specific or cell specific.

Proposal 2: For slss-TxDiabled operation for RRC idle UE, let’s reflect the CR [3].

Reference

[1] R2-1813200, Introduction on eV2X in TS36.331

[2] R1-1805620, LS on RRC parameters for V2X Phase 2

[3] R2-1818319, Configuration for cell-specific SLSS_TxDisabled

[4] RAN1 chairman’s Notes in 94bis