3GPP TSG-RAN WG2 #104	R2-1818364

Spokane, USA, 12th – 16th November 2018

Agenda Item:	10.4.1.4.1

Source:	Ericsson

Title:	On the inclusion of candidate cells in the measurement report

Document for:	Discussion, Decision

# Introduction

For RAN2#103bis meeting some companies expressed their views on the possibility of UE reporting the cells that are not in the triggered cells list but also other cells to be part of the measurement report [1]. In this contribution, we discuss what has already been discussed in this area in NR in the past and propose not such reporting of ‘non triggered cells’ in the measurement report at least in rel-15.

# Discussion

In one of the previous contribution from Ericsson [2], it was expressed that the network could benefit from knowing the additional information related to cells that have not yet satisfied the measurement report triggering criterion but could be close to doing so. In the contribution, it was proposed to use the remainder of the ‘maxReportCells’ which are not filled by triggered-cells with cells from the ‘non-triggered cells’. The excerpts from the contribution are provided below.

The cells to be included in the measurement report, as mentioned before, the PCell, SCell and cells in the cellsTriggeredList are included in the measurement report.

5: For event triggered reporting:

•	PCell and SCells cell quality are always included in the measurement report

•	Include cells in the cellsTriggeredList in the measurement report (same as LTE)

- Also, during the email discussion that was part of the preparation for the RAN2#98AH at least 8 companies expressed the views that measurement results associated to non-triggered cells could also be included in measurement reports, as long as the maxReportCells is not filed up with the triggered cells.

- Though, the measurement report triggering responsible cell (in an A3 like event) can have a very high probability of being chosen as the target cell of the handover, there are scenarios wherein the cell that triggered the measurement report and the cell to which the serving cell decides to handover the UE is different. Such scenarios are more often seen in areas with multi-cell overlapping border that are candidates for handover to wrong cell scenarios. Hence, to further aid the network in taking the correct decisions related to best handover candidates in such scenarios, it may be beneficial to configure the UE to include the report of non-triggered cells whose measurements are available.

- Observation-1: The serving cell could potentially use the information available at the UE regarding the non-triggered cells to improve the handover related decisions.

- In the previous meetings, some companies have expressed the need to have a measurement report triggering criterion based on the ‘number of good beams’. The intention was to use this information to handover the UE to a cell that has good coverage from multiple beams at that location. Instead of adding one more measurement report triggering criterion, one could enhance the measurement report content for the existing measurement reporting triggering criterion by adding cells with multiple ‘good beams’ to be part of the Non-TriggeredCellsList in the measurement report. By doing so, the serving cell will be notified of the presence of a third cell whose cell level measurements have not triggered the measurement report however, has multiple good beams at the current location of the UE. The ‘good beams’ evaluation could be based on the relative threshold from the best beam of the serving cell or the cell that has triggered the measurement report or from the best beam amongst all the audible beams to the UE. If a non-triggered cell has many ‘good beams’, then the network could use this information to select the handover candidate optimally. For example, the cell with many good beams can be perceived as the most robust cell to handover a UE to however at the cost of reduced throughput in some scenarios. This will allow the network to choose the appropriate decision using appropriate region of the network using a SON function to control the handover decision.

- Proposal-1: Network can configure the UE to include the cells in the cellsTriggeredList until maxReportCells is filled up.

- Proposal-2: If the number of cells in the cellsTriggeredList is less than maxReportCells, include other cells also in the measurement report until maxReportCells is filled up.

    - It is FFS as to how the cells other than the ones in the cellsTriggeredList are selected.

It is FFS as to whether network can configure the UE to include more than just the cellsTriggeredList in the measurement report.

Based on the above, it is clear that there are use cases where the inclusion of non triggered cells also in the measurement report will aid the network. However, in RAN2#99 meeting in Berlin, it was agreed to include only the triggered cells in the measurement report.

Agreements

1:	Cells in the cellsTriggeredList are reported (i.e. TTT has expired and leaving condition has not been met, same as LTE)

2:	Measurement quantity to be reported for cell quality can be same as trigger quantity or both RSRP/RSRQ. Measurement quantity to be reported for beam measurements  can be same as (cell) trigger quantity or both RSRP/RSRQ.

3:	For measurement events based on NR-SS, in each cell the best SS block is reported and up to x-1 next highest measured SS blocks above the absolute threshold. Threshold is the same as that used for cell quality derivation.

4	For measurement events based on CSI-RS, in each cell the best CSI-RS is reported and up to y-1 next highest measured CSI-RS above the absolute threshold. Threshold is the same as that used for cell quality derivation.

5:	Cell quality derived from NR-SS for the same cell is not included in a measurement report triggered based on CSI-RS.

6:	Cell quality derived from CSI-RS for the same cell is not included in a measurement report triggered based on NR-SS.

1:	For cells to be included for event triggered MR, number of cells reported is up to, maxCellMeas (as in LTE)

Based on this agreement, we propose that we do not consider the further enhancement of measurement report with non-triggered cells’ inclusion in rel-15 but bring it to the table in rel-16 mobility enhancements SI+WI.

It is already agreed that only the cells in the triggeredCellsList is included in the measurement report in NR rel-15.

RAN2 to confirm that only the cells in the triggeredCellsList is included in the measurement report in NR rel-15.

NonTriggeredCellsList’s inclusion/exclusion are to be studied in Rel-16 Mobility enhancements SI/WI.

# Conclusion

In this contribution, the following observations were captured:

Observation 1	It is already agreed that only the cells in the triggeredCellsList is included in the measurement report in NR rel-15.

In this contribution, the following proposals were captured:

Proposal 1	RAN2 to confirm that only the cells in the triggeredCellsList is included in the measurement report in NR rel-15.

Proposal 2	NonTriggeredCellsList’s inclusion/exclusion are to be studied in Rel-16 Mobility enhancements SI/WI.

# References

- R2-1814140, Ensuring that all candidate cells for handovers are reported, Huawei, HiSilicon, RAN2#103bis meeting, Chengdu, China.

- R2-1708584, Selection of cells and beams to be included in the measurement report, Ericsson, RAN2#99, Berlin, Germany.