# 3GPP TSG RAN WG2 Meeting #104      			                           R2-1818533
Spokane, US, 12th - 16th Nov. 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0784 | rev | 2 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.331 on RNTI-Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In latest TS 38.331, the IE RNTI-Value can take any value in the range INTEGER (0..65535): RNTI-Value ::= INTEGER (0..65535) However, it is not aligned with TS 38.321 where value of 0 (0000 hex) can’t be configured to UE: Table 7.1-1: RNTI values (from TS 38.321) Value (hexa-decimal); RNTI. 0000; N/A. 0001–FFEF; RA-RNTI, Temporary C-RNTI, C-RNTI, MCS-C-RNTI, CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI, and SP-CSI-RNTI. FFF0–FFFD; Reserved. FFFE; P-RNTI. FFFF; SI-RNTI. It will cause misalignment between 38.331 and 38.321, and confuse people. Therefore, we think it is necessary to clarify that the value 0 is not applicable in this version of the specification. |
| Summary of change: |  | In the description of IE RNTI-Value, add the following clarification: “The value 0 is not applicable in this version of the specification.” it applies to both NSA and SA. It is backward compatible change. Impact analysis Impacted functionality: RNTI value Inter-operability: 1. If the UE is implemented according to this CR but the network is not, UE may be indicated invalid RNTI-value 0 (0000 hex); 2. If the network is implemented according to this CR but the UE is not, no inter-operability issues are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will cause misalignment between 38.331 and 38.321, and confuse people whether value of 0 (0000 hex) can be configured to UE as valid RNTI-Value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

#### –	RNTI-Value

The RNTI-ValueIE represents a Radio Network Temporary Identity. The value 0 is not applicable in this version of the specification.

RNTI-Value information element

-- ASN1START

-- TAG-RNTI-VALUE-START

RNTI-Value ::=						INTEGER (0..65535)

-- TAG-RNTI-VALUE-STOP

-- ASN1STOP

End of change