3GPP TSG-RAN WG2#104	R2-1818579

Spokane, Washington, USA, 12-16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0122 | rev | 1 | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Release and Redirect in 2-step procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Rapporteur (Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-11-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding support of release with redirect in the 2-step resume procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is described that release and redirect is supported in the 2-step resume procedure. Impact Analysis Impacted functionality: 2-step resume procedure Inter-operability: If the CR is implemented by the UE but not by the eNB, no interoperability issues are foreseen. If the CR is implemented by the eNB but not by the UE, the UE may not expect to receive redirect information in the release message in the 2-step resume procedure, which may cause unexpected UE behaviour. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear whether release with re-direct is supported in the 2-step resume procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS 38.304 CR 0056 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 9.2.2.5	RNA update

The following figure describes the UE triggered resume involving context retrieval over Xn followed by an RRC Release without entering RRC_CONNECTED. The procedure is used for e.g. RNA update in which case the procedure may be triggered when the UE moves out of the configured RNA, or at the expiry of a periodic RNA Update timer.

Figure 9.2.2.5-1: Resume Request followed by Release procedure with UE context relocation

1.	The UE resumes from RRC_INACTIVE, providing the I-RNTI allocated by the last serving gNB and appropriate cause value, e.g., RAN notification area update.

2.	The gNB, if able to resolve the gNB identity contained in the I-RNTI, requests the last serving gNB to provide UE Context, providing the cause value received in step 1.

3.	The last serving gNB provides UE context.

4.	The gNB may move the UE to RRC_CONNECTED (and the procedure follows step 4 of Figure 9.2.2.4.1-1), or send the UE back to RRC_IDLE (in which case an RRCRelease message is sent by the gNB and the procedure ends), or send the UE back to RRC_INACTIVE as assumed in the following.

5.	If loss of DL user data buffered in the last serving gNB shall be prevented, the gNB provides forwarding addresses.

6./7. The gNB performs path switch.

8.	The gNB moves the UE back to RRC_INACTIVE state by sending RRCRelease with suspend configuration.

9.	The gNB triggers the release of the UE resources at the last serving gNB.

The following figure describes the procedure for the case when the last serving gNB decides not to relocate the UE context:

Figure 9.2.2.5-2: Resume Request followed by Release without UE context relocation

1.	The UE resumes from RRC_INACTIVE, providing the I-RNTI allocated by the last serving gNB and appropriate cause value, e.g., RAN notification area update.

2.	The gNB, if able to resolve the gNB identity contained in the I-RNTI, requests the last serving gNB to provide UE Context, providing the cause value received in step 1.

3.	The last serving gNB responds to the gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRC Connection Release message. The RRC message includes suspend configuration, if the last serving gNB decides to keep the UE in RRC_INACTIVE.

4.	The gNB forwards the RRC Connection Release message to the UE.

In both cases, the network may respond an RRCResumeRequest with an RRCRelease in the case the network wants to redirect the UE to another NR frequency or another RAT.

END OF CHANGES