3GPP TSG-RAN WG2 Meeting #104	R2-1818780

Spokane, USA, 12 - 16 November 2018

Title:	LS on Intra-UE Prioritization/Multiplexing

Response to:	-

Release:	Release 16

Work Item:	FS_NR_IIOT

Source:	Nokia, Nokia Shanghai Bell [TSG RAN WG2]

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Ping-Heng (Wallace) Kuo

E-mail Address:	ping-heng.kuo@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has discussed and identified a few targeted Intra-UE prioritization/multiplexing scenarios for further study. It is RAN2 understanding that RAN1 should be involved in the study of all these scenarios, since the mechanisms such as pre-emption and the relevant UE behavior should be examined by RAN1. The five prioritized scenarios include the following:

- Scenario 1: Intra-UE DL Prioritization

This scenario considers a case where a UE has sequentially received two DL assignments with overlapping radio resources in time. RAN2 assumes that by the later DL assignment has priority over the earlier DL assignment, considering that in principle the gNB will only give an assignment that overlaps with previous assignment for higher priority traffic. Based on such assumption, RAN1 should study solutions for prioritizing later received DL assignments.

- Scenario 2: Intra-UE UL Prioritization – Resource Conflict between Configured and Dynamic Grant

This scenario considers a case where the UL radio resource associated to a configured grant overlaps with a dynamic grant in time. A joint RAN2/RAN1 study should be initiated to handle such issue. In particular, RAN2 should consider LCP and grant handling priority (i.e. if a configured grant can override a dynamic grant), while RAN1 should study the details of related mechanisms for prioritizing configured grant PUSCH over dynamic grant PUSCH.

- Scenario 3: Intra-UE UL Prioritization – Resource Conflict between Dynamic Grants

This scenario considers a case where the UL radio resource associated to a dynamic grant overlaps with another dynamic grant in time. It is RAN2 understanding that traffics with different priorities could be distinguished by for example explicit L1 signaling of priority level per grant, or by other prioritization rule (for example, allowing a later grant to override the previous grant). Both RAN1 and RAN2 should further study this topic.

- Scenario 4: Intra-UE UL Prioritization – Resource Conflict between Control Channel and Control Channel

This scenario considers a case where the resources of uplink control transmission overlaps in time with other uplink control transmission relating to another, higher priority traffic. It is RAN2 understanding that this scenario should be mainly studied by RAN1, but RAN2 should be involved for analyzing the cases relating to uplink control transmission relating to SR.

- Scenario 5: Intra-UE UL Prioritization – Resource Conflict between Control Channel and Data Channel

This scenario considers a case where the resources of uplink control transmission overlaps in time with uplink data transmission relating to another traffic with either higher or lower priority. It is RAN2 understanding that this scenario should be mainly studied by RAN1, but RAN2 should be involved for analyzing the cases relating to uplink control transmission relating to SR.

Based on the scenarios identified above, RAN2 would like to request RAN1 to study solutions for the scenarios mentioned above.

In addition, RAN2 has also identified following two scenarios that RAN1 may consider to study:

- Scenario 6: Intra-UE UL Prioritization – CA-based Concurrent transmission with power limitation

In cases wherein both eMBB and URLLC data are exchanged between the UE and gNB and corresponding data or control transmissions simultaneously occur on different serving cells, prioritization may have to occur due to transmit power limitation.

- Scenario 7: Intra-UE UL Prioritization – Power control for traffics with different priorities

The UE may need to dynamic change its power control loop to ensure the URLLC transmission.

In RAN2’s understanding, both of these two additional scenarios have only impacts to RAN1.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to take the above information into account and study solutions for intra-UE traffic prioritization for the following five scenarios:

  - Scenario 1: Intra-UE DL Prioritization

  - Scenario 2: Intra-UE UL Prioritization – Resource Conflict between Configured and Dynamic Grant

  - Scenario 3: Intra-UE UL Prioritization – Resource Conflict between Dynamic Grants

  - Scenario 4: Intra-UE UL Prioritization – Resource Conflict between Control Channel and Control Channel

  - Scenario 5: Intra-UE UL Prioritization – Resource Conflict between Control Channel and Data Channel

In addition, RAN1 may also consider and provide feedback on whether the solutions for the following scenarios will be studied:

  - Scenario 6: Intra-UE UL Prioritization – CA-based Concurrent transmission with power limitation

  - Scenario 7: Intra-UE UL Prioritization – Power control for traffics with different priorities

3. Date of Next TSG-RAN WG2 Meetings:

3GPPRAN2#105		25 Feb - 01 Mar 2018		Athens, Greece