3GPP TSG-RAN WG2 Meeting #104	R2-1818800

Spokane, USA, 12 - 16 November 2018

Title:	[Draft] LS on PHR procedure in dual-connectivity

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Qualcomm Inc [TSG RAN WG2]

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Linhai He

E-mail Address:	linhaihe@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has discussed and agreed to the following change to LTE PHR procedure for EN-DC:

After a PHR is triggered, if dualConnectivityPHR is configured, UE needs to report power headroom information of all activated LTE and NR serving cells.  For a NR serving cell, whether UE reports its PH value based on real transmission or reference format is determined based on UL transmissions that have been scheduled or configured until 4 ms prior to the TTI in which the PHR MAC CE is transmitted.

In addition, RAN2 has discussed and agreed to the following change to both LTE and NR PHR procedure:

For a band combination in which a UE is not capable of dynamic power sharing, the UE may omit reporting power headroom information for serving cells in the other MAC entity.

2. Actions:

RAN2 respectfully asks RAN1 to take the above agreements into account and update RAN1 specification(s) accordingly if needed.

3. Date of Next TSG-RAN WG2 Meetings:

3GPPRAN2#105		25 Feb - 01 Mar 2018		Athens

3GPPRAN2#105bis		08 April – 12 April 2018		China