3GPP TSG-RAN WG2#104	R2-1817534

Spokane, USA, 12th – 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0084 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on RRC transfer procedure in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-11-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The text in RRC transfer procedure is not clear and can be measleading. In fact, such procedure is used to exchange RRC messages between the MN and SN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 10.10 RRC Transfer - Description has been updated - Added reference to TS 36.423 Impact analysis If the network implements the CR and the UE does not, it is not clear what generic container to use for reporting the NR measurements or NR failure information to the SN via the MN. If the UE implements the CR and the UE does not, it is not clear what generic container to use for reporting the NR measurements or NR failure information to the SN via the MN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Since the text is measleading, the procedure could be wrongly interpreded and and some EN-DC procedure may not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.10 RRC Transfer |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[3]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage 2".

[4]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[5]	3GPP TS 38.423: "NG-RAN; Xn application protocol (XnAP)".

[6]	3GPP TS 38.425: "NG-RAN; NR user plane protocol".

[7]	3GPP TS 38.401: "NG-RAN; Architecture description".

[8]	3GPP TS 38.133: "NG-RAN; Requirements for support of radio resource management".

[9]	3GPP TS 36.423: “Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)”

[10]	3GPP TS 36.331: “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”

END OF CHANGES

START OF CHANGES

## 10.10	RRC Transfer

### 10.10.1	EN-DC

The RRC Transfer procedure is used to deliver an RRC message, encapsulated in a PDCP PDU between the MN and the SN (and vice versa) so that it may be forwarded to/from the UE using split SRB. The RRC transfer procedure is also used for providing a NR measurement report or NR failure information from the UE to the SN via the MN. Additional details of the RRC transfer procedure are defined in TS 36.423 [9].

Split SRB:

Figure 10.10.1-1: RRC Transfer procedure for the split SRB (DL operation)

Figure 10.10.1-1 shows an example signaling flow for the DL RRC Transfer in case of the split SRB:

1.	The MN, when it decides to use the split SRBs, starts the procedure by initiating the RRC Transfer procedure. The MN encapsulates the RRC message in a PDCP-C PDU and ciphers with own keys.

NOTE:	The usage of the split SRBs shall be indicated in the Secondary Node Addition procedure or Modification procedure.

2.	The SN forwards the RRC message to the UE.

3.	The SN may send PDCP delivery acknowledgement of the RRC message forwarded in step 2.

Figure 10.10.1-2: RRC Transfer procedure for the split SRB (UL operation)

Figure 10.10.1-2 shows an example signaling flow for the UL RRC Transfer in case of the split SRB:

1.	When the UE provides response to the RRC message, it sends it to the SN.

2.	The SN initiates the RRC Transfer procedure, in which it transfers the received PDCP-C PDU with encapsulated RRC message.

NR measurement report or NR failure information:

Figure 10.10.1-3: RRC Transfer procedure for NR measurement report or NR failure information

Figure 10.10.1-3 shows an example signaling flow for RRC Transfer in case of the forwarding of the NR measurement report or NR failure information from the UE:

1.	When the UE sends a measurement report or NR failure information, it sends it to the MN in a container called ULInformationTransferMRDC as specified in TS 36.331 [10]..

2.	The MN initiates the RRC Transfer procedure, in which it transfers the received NR measurement report or NR failure information as an octet string.

END OF CHANGES