3GPP TSG-RAN WG2#104	R2-1818885

# Spokane, USA, 12th – 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3764 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.331 on corrections related to inter-RAT CGI reporting towards NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections related to inter-RAT CGI reporting towards NR in E-UTRAN, specifically the CGI reproting only applies if the SIB 1 is broadcasted in SIB1 but the procedure text does not reflect it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes related to CGI reporting: 5.5.3.1: Procedural text is extended to cover the situation when the UE realizes that the NR cell does not bradcast SIB1. Impact Analysis Impacted functionality: If UE is not implemented according to this CR, there will be some ambiguities in the case that a UE is configured with inter-RAT CGI reporting towards an NR cell which does not broadcast SIB1 since the current procedure text does not specifically state the condition that CGI reporting should be performed only when SIB 1 is broadcasted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not correctly reflect the ANR functionality developments in 3GPP and it would be unclear that the CGI reporting applied to NR cell only when SIB 1 is broadcasted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.1 General |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- ************************************* Beginning of changes *********************************************************

### 5.5.3	Performing measurements

#### 5.5.3.1	General

For all measurements, except for UE Rx–Tx time difference measurements, RSSI, UL PDCP Packet Delay per QCI measurement, channel occupancy measurements, CBR measurement, sensing measurement and except for WLAN measurements of Band, Carrier Info, Available Admission Capacity, Backhaul Bandwidth, Channel Utilization, and Station Count, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria or for measurement reporting. When performing measurements on NR carriers, the UE derives the cell quality as specified in 5.5.3.3 and the beam quality as specified in 5.5.3.4.

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell as follows:

2>	for the PCell, apply the time domain measurement resource restriction in accordance with measSubframePatternPCell, if configured;

2>	if the UE supports CRS based discovery signals measurement:

3>	for each SCell in deactivated state, apply the discovery signals measurement timing configuration in accordance with measDS-Config, if configured within the measObject corresponding to the frequency of the SCell;

1>	if the UE has a measConfig with rs-sinr-Config configured, perform RS-SINR (as indicated in the associated reportConfig) measurements as follows:

2>	perform the corresponding measurements on the frequency indicated in the associated measObject using available idle periods or using autonomous gaps as necessary;

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the purpose for the associated reportConfig is set to reportCGI:

3>	if si-RequestForHO is configured for the associated reportConfig:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using autonomous gaps as necessary;

3>	else:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using available idle periods or using autonomous gaps as necessary;

NOTE 1:	If autonomous gaps are used to perform measurements, the UE is allowed to temporarily abort communication with all serving cell(s), i.e. create autonomous gaps to perform the corresponding measurements within the limits specified in TS 36.133 [16]. Otherwise, the UE only supports the measurements with the purpose set to reportCGI only if E-UTRAN has provided sufficient idle periods.

3>	try to acquire the global cell identity of the cell indicated by the cellForWhichToReportCGI in the associated measObject by acquiring the relevant system information from the concerned cell;

3>	if an entry in the cellAccessRelatedInfoList includes the selected PLMN, acquire the relevant system information from the concerned cell;

3>	if the cell indicated by the cellForWhichToReportCGI included in the associated measObject is an E-UTRAN cell:

4>	try to acquire the CSG identity, if the CSG identity is broadcast in the concerned cell;

4>	try to acquire the trackingAreaCode in the concerned cell;

4>	try to acquire the list of additional PLMN Identities, as included in the plmn-IdentityList, if multiple PLMN identities are broadcast in the concerned cell;

4>	if cellAccessRelatedInfoList is included, use trackingAreaCode and plmn-IdentityList from the entry of cellAccessRelatedInfoList containing the selected PLMN;

4>	if the includeMultiBandInfo is configured:

5>	try to acquire the freqBandIndicator in the SystemInformationBlockType1of the concerned cell;

5>	try to acquire the list of additional frequency band indicators, as included in the multiBandInfoList, if multiple frequency band indicators are included in the SystemInformationBlockType1of the concerned cell;

5>	try to acquire the freqBandIndicatorPriority, if the freqBandIndicatorPriority is included in the SystemInformationBlockType1of the concerned cell;

4>	if cellAccessRelatedInfoList-5GC is broadcast in the concerned cell and the UE is E-UTRA/5GC capable:

5>	try to acquire the cellAccessRelatedInfoList-5GC;

NOTE 2:	The 'primary' PLMN is part of the global cell identity.

3>	if the cell indicated by the cellForWhichToReportCGI included in the associated measObject is a UTRAN cell:

4>	try to acquire the LAC, the RAC and the list of additional PLMN Identities, if multiple PLMN identities are broadcast in the concerned cell;

4>	try to acquire the CSG identity, if the CSG identity is broadcast in the concerned cell;

3>	if the cell indicated by the cellForWhichToReportCGI included in the associated measObject is a GERAN cell:

4>	try to acquire the RAC in the concerned cell;

3>	if the cell indicated by the cellForWhichToReportCGI included in the associated measObject is a CDMA2000 cell and the cdma2000-Type included in the measObject is typeHRPD:

4>	try to acquire the Sector ID in the concerned cell;

3>	if the cell indicated by the cellForWhichToReportCGI included in the associated measObject is a CDMA2000 cell and the cdma2000-Type included in the measObject is type1XRTT:

4>	try to acquire the BASE ID, SID and NID in the concerned cell;

3>	if the cell indicated by the cellForWhichToReportCGI included in the associated MeasObject is an NR cell:

4> if the indicated cell is broadcasting SIB1 (see TS 38.213 [88], section 13):

5>	try to acquire the plmn-IdentityInfoList including plmn-IdentityList, trackingAreaCode (if available), ran-AreaCode (if available) and cellIdentity for each entry of the plmn-IdentityInfoList;

5>	try to acquire the frequencyBandList, if multiple frequency bands are broadcasted in the concerned cell;

2>	if the ul-DelayConfig is configured for the associated reportConfig:

3>	ignore the measObject;

3>	configure the PDCP layer to perform UL PDCP Packet Delay per QCI measurement;

2>	else:

3>	if a measurement gap configuration is setup; or

3>	if the UE does not require measurement gaps to perform the concerned measurements:

4>	if s-Measure is not configured; or

4>	if s-Measure is configured and the PCell RSRP, after layer 3 filtering, is lower than this value; or

4>	if the associated measObject concerns NR; or

4>	if measDS-Config is configured in the associated measObject:

5>	if the UE supports CSI-RS based discovery signals measurement; and

5>	if the eventId in the associated reportConfig is set to eventC1 or eventC2, or if reportStrongestCSI-RSs is included in the associated reportConfig:

6>	perform the corresponding measurements of CSI-RS resources on the frequency indicated in the concerned measObject, applying the discovery signals measurement timing configuration in accordance with measDS-Config in the concerned measObject;

6>	if reportCRS-Meas is included in the associated reportConfig, perform the corresponding measurements of neighbouring cells on the frequencies indicated in the concerned measObject as follows:

7>	for neighbouring cells on the primary frequency, apply the time domain measurement resource restriction in accordance with measSubframePatternConfigNeigh, if configured in the concerned measObject;

7>	apply the discovery signals measurement timing configuration in accordance with measDS-Config in the concerned measObject;

5>	else:

6>	perform the corresponding measurements of neighbouring cells on the frequencies and RATs indicated in the concerned measObject as follows:

7>	for neighbouring cells on the primary frequency, apply the time domain measurement resource restriction in accordance with measSubframePatternConfigNeigh, if configured in the concerned measObject;

7>	if the UE supports CRS based discovery signals measurement, apply the discovery signals measurement timing configuration in accordance with measDS-Config, if configured in the concerned measObject;

4>	if the ue-RxTxTimeDiffPeriodical is configured in the associated reportConfig:

5>	perform the UE Rx–Tx time difference measurements on the PCell;

4>	if the reportSSTD-Meas is set to true or pSCell in the associated reportConfig:

5>	perform SSTD measurements between the PCell and the PSCell;

4>	if the reportSFTD-Meas is set to pSCell in the associated reportConfig:

5>	perform SFTD measurements between the PCell and the NR PSCell;

4>	if the reportSFTD-Meas is set to neighborCells in the associated reportConfig:

5>	perform SFTD measurements between the PCell and NR cell(s) on the frequency indicated in the associated measObject;

4>	if the measRSSI-ReportConfig is configured in the associated reportConfig:

5>	perform the RSSI and channel occupancy measurements on the frequency indicated in the associated measObject;

2>	perform the evaluation of reporting criteria as specified in 5.5.4;

The UE capable of CBR measurement when configured to transmit non-P2X related V2X sidelink communication shall:

1>	if in coverage on the frequency used for V2X sidelink communication transmission as defined in TS 36.304 [4, 11.4]; or

1>	if the concerned frequency is included in v2x-InterFreqInfoList in RRCConnectionReconfiguration or in v2x-InterFreqInfoList within SystemInformationBlockType21 or SystemInformationBlockType26:

2>	if the UE is in RRC_IDLE:

3>	if the concerned frequency is the camped frequency:

4>	perform CBR measurement on the pools in v2x-CommTxPoolNormalCommon and v2x-CommTxPoolExceptional if included in SystemInformationBlockType21;

3>	else if v2x-CommTxPoolNormal or v2x-CommTxPoolExceptional is included in v2x-InterFreqInfoList for the concerned frequency within SystemInformationBlockType21 or SystemInformationBlockType26:

4>	perform CBR measurement on pools in v2x-CommTxPoolNormal and v2x-CommTxPoolExceptional in v2x-InterFreqInfoList for the concerned frequency in SystemInformationBlockType21 or SystemInformationBlockType26;

3>	else if the concerned frequency broadcasts SystemInformationBlockType21:

4>	perform CBR measurement on pools in v2x-CommTxPoolNormalCommon and v2x-CommTxPoolExceptional if included in SystemInformationBlockType21 broadcast on the concerned frequency;

2>	if the UE is in RRC_CONNECTED:

3>	if tx-ResourcePoolToAddList is included in VarMeasConfig:

4>	perform CBR measurements on each resource pool indicated in tx-ResourcePoolToAddList;

3>	if the concerned frequency is the PCell's frequency:

4>	perform CBR measurement on the pools in v2x-CommTxPoolNormalDedicated or v2x-SchedulingPool if included in RRCConnectionReconfiguration, v2x-CommTxPoolExceptional if included in SystemInformationBlockType21 for the concerned frequency and v2x-CommTxPoolExceptional if included in mobilityControlInfoV2X;

3>	else if v2x-CommTxPoolNormal, v2x-SchedulingPool or v2x-CommTxPoolExceptional is included in v2x-InterFreqInfoList for the concerned frequency within RRCConnectionReconfiguration:

4>	perform CBR measurement on pools in v2x-CommTxPoolNormal, v2x-SchedulingPool, and v2x-CommTxPoolExceptional if included in v2x-InterFreqInfoList for the concerned frequency in RRCConnectionReconfiguration;

3>	else if the concerned frequency broadcasts SystemInformationBlockType21:

4>	perform CBR measurement on pools in v2x-CommTxPoolNormalCommon and v2x-CommTxPoolExceptional if included in SystemInformationBlockType21 for the concerned frequency;

1>	else:

2>	perform CBR measurement on pools in v2x-CommTxPoolList in SL-V2X-Preconfiguration for the concerned frequency;

The UE capable of sensing measurement, with commTxResources set to scheduled, shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if measSensing-Config is configured in the associated measObject

3>	perform the sensing measurement in accordance with TS 36.213 [23] on the pools of v2x-SchedulingPool and also indicated in tx-ResourcePoolToAddList in the associated measObject, using sensingSubchannelNumber, sensingPeriodicity, sensingResourceCounter and sensingPriority.

NOTE 3:	The s-Measure defines when the UE is required to perform measurements. The UE is however allowed to perform measurements also when the PCell RSRP exceeds s-Measure, e.g., to measure cells broadcasting a CSG identity following use of the autonomous search function as defined in TS 36.304 [4].

NOTE 4:	The UE may not perform the WLAN measurements it is configured with e.g. due to connection to another WLAN based on user preferences as specified in TS 23.402 [75] or due to turning off WLAN.

- ************************************* End of changes ****************************************************************