3GPP TSG-RAN2 Meeting #104 	R2-1819190

Spokane, USA, November 12th – 16th 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0395 | rev | 3 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inter-band EN-DC Configured Output Power requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN#81 approved way forward for inter-band EN-DC configured output power in RP-182034 and sent LS to RAN4/RAN2. In addition, RAN4 is discussing to use same parameter for intra-band EN-DC. Way forward instructs RAN4 to continue to improving requirement in TS38.101-3 in the RAN4 meetings to prevent unnecessary NR dropping for dynamic power sharing EN-DC UEs when there is ’real’ power left from LTE UL. According to the way forward, the UE is allowed to drop NR only if the power scaling applied to NR means that the difference between scaled and unscaled NR UL power is more than XdB. In other cases the UE does power scaling of NR UL. - X dB is RRC configured parameter with 4 fixed values and X is [0, 2, 4 or 6] dB. - The UE has to be able to support all these 4 configurable X values. - This threshold X dB does not limit the UE performance but only defines the UE minimum performance (i.e. UE can perform better than the minimum performance). Handling of multiple NR UL carriers needs to be addressed (including possibility to leave this up to the UE implementation). Later on, RAN4 endorsed way forward in R4-1814168 to use draft CR R4-1813844 as baseline indicating that X_scale can have values [0, 6]. Thus, there is need to introduce a new paramater in 38.331 to support RAN4 defined use cases. Impact analysis: If the NW supports this CR and UE does not support, for dynamic power sharing, the UE does not understand the signalling and there is mismatch in the power in the NR. If the UE supports this CR and NW does not support, no interoperability issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a parameter for difference between scaled and unscaled NR UL power. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### –	PhysicalCellGroupConfig

The IE PhysicalCellGroupConfig is used to configure cell-group specific L1 parameters.

PhysicalCellGroupConfig information element

-- ASN1START

-- TAG-PHYSICALCELLGROUPCONFIG-START

PhysicalCellGroupConfig ::=         SEQUENCE {

harq-ACK-SpatialBundlingPUCCH       ENUMERATED {true}                                                   OPTIONAL,   -- Need S

harq-ACK-SpatialBundlingPUSCH       ENUMERATED {true}                                                   OPTIONAL,   -- Need S

p-NR-FR1                            P-Max                                                               OPTIONAL,   -- Need R

pdsch-HARQ-ACK-Codebook             ENUMERATED {semiStatic, dynamic},

tpc-SRS-RNTI                        RNTI-Value                                                          OPTIONAL,   -- Need R

tpc-PUCCH-RNTI                      RNTI-Value                                                          OPTIONAL,   -- Need R

tpc-PUSCH-RNTI                      RNTI-Value                                                          OPTIONAL,   -- Need R

sp-CSI-RNTI                         RNTI-Value                                                          OPTIONAL,   -- Cond SP-CSI-Report

cs-RNTI                             SetupRelease { RNTI-Value }                                         OPTIONAL,    -- Need M

...,

[[

mcs-C-RNTI                          RNTI-Value                                                          OPTIONAL,   -- Need R

p-UE-FR1                            P-Max                                                               OPTIONAL    -- Cond MCG-Only

]],

[[

xScale								ENUMERATED	{dB0, dB6, spare2, spare1}								OPTIONAL	-- Cond SCG-Only

]]

}

-- TAG-PHYSICALCELLGROUPCONFIG-STOP

-- ASN1STOP

PhysicalCellGroupConfig field descriptions

cs-RNTI RNTI value for downlink SPS (see SPS-Config) and uplink configured grant (see ConfiguredGrantConfig).

harq-ACK-SpatialBundlingPUCCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUCCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling is disabled. Corresponds to L1 parameter 'HARQ-ACK-spatial-bundling' (see 38.213, section FFS_Section)

harq-ACK-SpatialBundlingPUSCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUSCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling is disabled. Corresponds to L1 parameter 'HARQ-ACK-spatial-bundling' (see 38.213, section FFS_Section)

mcs-C-RNTI RNTI to indicate use of qam64LowSE for grant-based transmissions. When the MCS-C-RNTI is configured, RNTI scrambling of DCI CRC is used to choose the corresponding MCS table.

p-NR-FR1 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 1 (FR1). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR1 (configured total for all serving cells operating on FR1).

p-UE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR1 (configured for the cell group).

pdsch-HARQ-ACK-Codebook The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to both CA and none CA operation. Corresponds to L1 parameter 'HARQ-ACK-codebook' (see 38.213, section FFS_Section)

sp-CSI-RNTI RNTI for Semi-Persistent CSI reporting on PUSCH (see CSI-ReportConfig). Corresponds to L1 parameter 'SPCSI-RNTI' (see 38.214, section 5.2.1.5.2)

tpc-PUCCH-RNTI RNTI used for PUCCH TPC commands on DCI. Corresponds to L1 parameter 'TPC-PUCCH-RNTI' (see 38.213, section 10).

tpc-PUSCH-RNTI RNTI used for PUSCH TPC commands on DCI. Corresponds to L1 parameter 'TPC-PUSCH-RNTI' (see 38.213, section 10)

tpc-SRS-RNTI RNTI used for SRS TPC commands on DCI. Corresponds to L1 parameter 'TPC-SRS-RNTI' (see 38.213, section 10)

xScale The UE is allowed to drop NR only if the power scaling applied to NR results in a difference between scaled and unscaled NR UL of more than xScale dB (see 38.101-3). If the value is not configured for dynamic power sharing, the UE assumes default value of 6 dB

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need R, in the PhysicalCellGroupConfig of the MCG. It is absent otherwise. |
| SP-CSI-Report | The field is mandatory present, Need R, when at least one CSI-ReportConfig with reportConfigType set to semiPersistentOnPUSCH is configured; otherwise it is optionally present, need M. |
| SCG-Only | This field is optionally present, Need S, in the PhysicalCellGroupConfig of the SCG in case of EN-DC as defined in RAN4 38.101-3. It is absent otherwise. FFS for NR-NR DC and multiple NR uplinks. |