3GPP TSG-RAN WG2 Meeting #105                                                 	         R2-1900214

Athens, Greece, 25th Feb – 1st Mar 2019

Source:	CATT

Title:	Discussion on requirements for fast recovery enhancements

Agenda Item:	11.10.5

Document for:	Discussion and Decision

1	Introduction

A new WI on DC and CA enhancements was approved in RAN#81. One objective of the WI is to enhance the radio link failure recovery procedure.

Fast recovery: Support fast recovery of MCG link e.g. by utilizing the SCG link and split SRBs for recovery during MCG failure while operating under MR-DC. [RAN2, RAN3]

  - This objective applies to MR-DC and NR-NR DC

In this contribution, we look in to a set of basic guidelines to be considered for improvement of radio link recovery procedure.

2. Discussion

Split SRB was introduced in Rel-15 as improvement of robustness of RRC messages. Additionally SRB3 was introduced for the transmission of RRC messages between the UE and the SN. Upon detection of S-RLF, the UE informs the MN of the occurrence of S-RLF. Communication between the UE and the SN is suspended while the communication between the UE and the MN is maintained upon S-RLF. The MN takes necessary actions for the S-RLF. This seems appropriate given that the MN is the master which maintains the primary UE connection to the network. When M-RLF occurred, the communication between the UE and the network is suspended and the UE initiates the RRC connection re-establishment procedure to recover form M-RLF. A good quality radio link toward the SN could be utilised in recovering from M-RLF, however it should be handle with care considering that the primary link with the UE and the network has gone through a failure.

RRC connection Re-establishment is triggered in the following cases.

Case 1: Upon radio link failure of the MCG

There are three triggers for RLF. 1).Expiry of a timer started after indication of radio problems from the physical layer (if radio problems are recovered before the timer is expired, the UE stops the timer); 2).Random access procedure failure; 3).RLC failure.

Case 2: Upon re-configuration with sync failure of the MCG

Case 3: Upon mobility from NR failure

Case 4: Upon integrity check failure indication from lower layers concerning SRB1 or SRB2

Case 5: Upon an RRC connection reconfiguration failure

Case 1 (detection of radio link failure) is directly related to the link quality of the MCG. Therefore, a good link quality towards the SN could be utilised to improve failure recovery. Case 2 to Case 5 may or may not occur only due to the radio link failure towards the MN. However, enhancement for fast recovery should consider all 5 failure cases. Ideally, one solution should be targeted for all 5 re-establishment triggers.

Proposal 1: Fast recovery enhancements should consider all cases for RRC connection re-establishment and ideally one solution should be targeted for all cases.

Case 1: Upon radio link failure of the MCG

Case 2: Upon re-configuration with sync failure of the MCG

Case 3: Upon mobility from NR failure

Case 4: Upon integrity check failure indication from lower layers concerning SRB1 or SRB2

Case 5: Upon an RRC connection reconfiguration failure

Considering MR-DC deployment scenarios, it is possible that the MN may go through M-RLF while radio link towards the SN is good. For example in NE-DC, LTE SN link is on low frequency carrier while NR MN is on a high frequency carrier. Thus, high frequency MN carrier may go through RLF while low frequency SN has a good quality link. As indicated in the WI, the good quality SN link could be utilised for fast recovery of M-RLF as the SN link is still good to provide communication link between the UE and the network.

The control plane signalling are conducted over the SRBs. Rel-15 provides options of configuring split SRB (for SRB1 and SRB2), configuring SRB3 and configuring non-split SRBs (MN terminated SRBs) for control plane signalling. Having a good quality link towards the SN doesn’t depend on type of SRB which is configured for the communication between the UE and the network. Therefore the enhancements which utilise the good quality radio link of the SN between the UE and the network should not be limited to the configured SRB type.

Proposal 2: Fast recovery enhancements should utilise the good quality radio link of the SN between the UE and the network. However the enhancements should not be limited by the configured SRB type (split SRB, non-split SRB or SRB3).

The WI already indicated that the fast recovery enhancement should be applied to MR-DC and NR-NR DC. Even though there are some differences in deployment aspects of different MR-DC and differences in type of SRB supported in difference MR-DC options, accessibility of good quality radio link of the SN is presence in all cases. Also, fast recovery from M-RLF would equally benefit all MR-DC options. A single (harmonized) fast recovery solution addressing all MR-DC and NR-DC deployment is preferable. A harmonized solution may also reduce the standardisation effort of the solution in different MR-DC options.

Proposal 3: RAN2 should aim for a single (harmonized) fast recovery solution(s) addressing all MR-DC and NR-DC deployment options.

There are a number of cases (case 1 to case 5 above) where RRC connection re-establishment could be triggered. Regardless of what triggers the failure, this is an indication of failure /problem of the communication link between the UE and the Master Node. Current behaviour of the failure handling is to stop any communication between the UE and the MN upon trigger of RRC re-establishment. While attempting for fast recovery of the communication link, the UE must stop/suspend any radio transmission/reception using the failed radio link between the UE and the MN at the earliest detection of a link failure between the UE and the MN.

Proposal 4: While attempting for fast recovery, the UE must stop/suspend any radio transmission/reception using the failed radio link at the earliest detection of a link failure between the UE and the MN.

3	Conclusion

In this contribution, we have discussed a set of guidelines for fast recovery enhancements which to be investigated under the new WI. And the following high-level proposals were made.

Proposal 1: Fast recovery enhancements should consider all cases for RRC connection re-establishment and ideally one solution should be targeted for all cases.

Case 1:  Upon radio link failure of the MCG

Case 2:  Upon re-configuration with sync failure of the MCG

Case 3: Upon mobility from NR failure

Case 4: Upon integrity check failure indication from lower layers concerning SRB1 or SRB2

Case 5: Upon an RRC connection reconfiguration failure

Proposal 2: Fast recovery enhancements should utilise the good quality radio link of the SN between the UE and the network. However the enhancements should not be limited by the configured SRB type (split SRB, non-split SRB or SRB3).

Proposal 3: RAN2 should aim for a single (harmonized) fast recovery solution(s) addressing all MR-DC and NR-DC deployment options.

Proposal 4: While attempting for fast recovery, the UE must stop/suspend any radio transmission/reception using the failed radio link at the earliest detection of a link failure between the UE and the MN.