3GPP TSG-RAN WG2 Meeting #105	R2-1900431

Athens, Greece, 24 Feb - 1 Mar 2019

Agenda item:	11.12.2

Source:	Nokia, Nokia Shanghai Bell

Title:	TP for QoS optimisation via MDT

WID/SID:	FS_LTE_NR_data_collect - Release 16

Document for:	Discussion and Decision

# 1	Introduction

The contribution consists a Text Proposal to TR37.816 aiming at clarification that MDT is not a use case by itself, but a solution that realizes QoS optimisation. RAN2 target solutions for MDT under RNA DCU should target the use case.

# 2	TR37.816 Text proposal

5.7	MDT

### 5.7.1	Use cases description

#### 5.7.1.2 Use case: QoS Optimisation via MDT

User throughput is an important factor in quality of service verification (QoS). End-user often observes perceived DL throughput or a service delay to judge network performance. This aspect is important also in the initial deployment of a new radio access technology, in order to check if the quality of service experienced by the end user is in line with the performance target defined in the planning strategy and more in general to test the overall performance of the technology along the subsequent deployment phases. Accordingly, information about the end-user achieved throughput and detection of any QoS problems (e.g. low throughput, long delays) is essential for network planning as well as backend management activities, such as network planning, dimensioning and optimization.

The verification of the QoS has become a crucial use case in LTE MDT campaigns. The possibility to collect QoS measurements collectively: from terminals in the field and RAN entities has become enabler to gather effective and reliable performance metrics for faster optimisation cycle resulting in higher customer satisfaction. With planned roll-outs of the new 5G deployments (NSA, SA, DC), the need for continuous check of network QoS significantly increases and motivates collecting feedback from end-user as well as RAN entities.

The concept of MDT data reported from UEs and the RAN for QoS troubleshooting should allow QoS issues monitoring and detection, assess user experience from RAN perspective, and enable to assist network capacity extension. In particular, examples of target situations include:

-	Traffic Location

-	User QoS Experience

### 5.7.2	Solution description

Editor Note: Capture the solutions for the use case, including the procedure for configuration and collection of measurements, necessary procedures and information exchange required for the solution, as well as comparison and evaluation on potential alternative solutions.

### 5.7.3	Conclusion