3GPP TSG-RAN WG2 Meeting #105												R2-1900505

Athens, Greece, 25th February - 1st March 2019

Source: 	vivo

Title:  	Discussion on the RLF and HOF during the DC handover

Agenda Item:	12.3.2

Document for:	Discussion and Decision

# Introduction

According to the email discussion report [1], the solution direction would be to use the non-split bearer option for setting up the connection simultaneously between the source cell and target cell. In this contribution, we discuss the details on how to determine the handover failure, radio link failure and the connection failure during the DC handover procedure.

# Discussion

## Stages of the DC handover

According to [1], the non-split bearer solution includes the following two options:

- Option 1: A single protocol stack active at a time

- Option 2: Dual protocol stacks active at a time

For Option 1, companies consider that “the RLM on source link maintains until UE successfully accesses the target cell, and then the RLM on target link starts”. For Option 2, companies consider that “the simultaneous RLM is required for simultaneous connectivity.” However according to the descriptions of the two Options, both Options needs to receive packets from both the source link and the target link after the successful RACH and before releasing the source connection. For example, Option 2 is saying that “the connection to the source cell is maintained until the UE receives the first downlink data packet from target eNB”. Then we consider that the handover procedure of both Option 1 and Option 2 can be divided into the following stages as illustrated in Figure 1.

Figure 1: DC handover stages

Stage 1: After the reception of the handover command, the UE keeps the source connection and starts the cell searching and the RACH procedure to the target cell.

Stage 2: After the successful RACH and before releasing the source connection, the UE keeps data reception via two links.

Stage 3: After releasing the source connection, the UE only keeps the target cell connection.

## Handover failure and radio link failure during DC handover

As a comparison, the failure detection of the legacy handover is given in the Annex, and we have the following observations:

Observation 1: In the legacy handover, the UE starts RLM immediately on the target PCell when the UE is downlink synchronized to the target PCell, but does not start T310 on the target PCell during the handover (i.e. when T304 is running).

Observation 2: In the legacy handover, when the UE receives the random access problem indication during the handover (i.e. when T304 is running), the UE does not trigger RLF.

Observation 3: In the legacy handover, there is no case that the UE receives the RLC status report from the target cell of the SRB during the handover (i.e. when T304 is running).

From our understanding, to determine the handover failure or RLF, we need firstly decide when to stop the T304, i.e. how to determine the successful DC handover. Then we can discuss the detailed UE behaviors when the T304 is running or stopped. Firstly we would RAN2 to confirm that the T304 starts immediately when the UE receives the handover command, and the expiry of the T304 is used to determine the failure of the DC handover.

Proposal 1: The expiry of the T304 which starts at the reception of the handover command is used to determine the handover failure of the DC handover, as the legacy handover.

Proposal 2: The T304 of the DC handover is stopped at the successful RACH to the target cell, as the legacy handover.

According to the email discussion, it seems that most companies consider that the RLM of the source connection should be kept before releasing the source connection.

Proposal 3: The UE starts T310 upon receiving N310 consecutive “out-of-sync” indication of the source PCell before releasing the source connection during DC handover.

Regarding the target cell RLM, we consider that the UE should start the RLM immediately after downlink synchronized to the target cell as the legacy handover. And the UE should not start the T310 of the target PCell while T310 is running as the legacy handover.

Proposal 4: The UE starts the RLM immediately after downlink synchronized to the target PCell, as the legacy handover.

Proposal 5: The UE does not start the T310 of the target PCell while T304 is running, as the legacy handover.

Regarding the RACH failure (i.e. the RRC receives the random access problem indication from the MAC) to the target cell, we consider that the UE should keep RACH procedure before the handover failure (i.e. the expiry of the T304).

Proposal 6: The UE does not trigger RLF when the RRC receives the random access problem indication from the MAC while T304 is running, as the legacy handover.

Then after the successful RACH (i.e. Stage 2), the UE will have two connection to both the source cell and the target cell. Here we consider that if only one link fails (i.e. PHY/RLC/RACH failure), the UE does not trigger RLF, but report the failure information via another link. If both link fails, the UE triggers the RLF (i.e. RRC connection re-establishment).

Proposal 7: After the successful RACH to the target cell, if only one link fails, the UE does not trigger RRC connection re-establishment, but report the failure information via another link.

Proposal 8: After the successful RACH to the target cell, if both link fails, the UE triggers the RRC connection re-establishment.

# Conclusions

Based on the analysis given above, we have the following Observations and Proposals：

Observation 1: In the legacy handover, the UE starts RLM immediately on the target PCell when the UE is downlink synchronized to the target PCell, but does not start T310 on the target PCell during the handover (i.e. when T304 is running).

Observation 2: In the legacy handover, when the UE receives the random access problem indication during the handover (i.e. when T304 is running), the UE does not trigger RLF.

Observation 3: In the legacy handover, there is no case that the UE receives the RLC status report from the target cell of the SRB during the handover (i.e. when T304 is running).

Proposal 1: The expiry of the T304 which starts at the reception of the handover command is used to determine the handover failure of the DC handover, as the legacy handover.

Proposal 2: The T304 of the DC handover is stopped at the successful RACH to the target cell, as the legacy handover.

Proposal 3: The UE starts T310 upon receiving N310 consecutive “out-of-sync” indication of the source PCell before releasing the source connection during DC handover.

Proposal 4: The UE starts the RLM immediately after downlink synchronized to the target PCell, as the legacy handover.

Proposal 5: The UE does not start the T310 of the target PCell while T304 is running, as the legacy handover.

Proposal 6: The UE does not trigger RLF when the RRC receives the random access problem indication from the MAC while T304 is running, as the legacy handover.

Proposal 7: After the successful RACH to the target cell, if only one link fails, the UE does not trigger RRC connection re-establishment, but report the failure information via another link.

Proposal 8: After the successful RACH to the target cell, if both link fails, the UE triggers the RRC connection re-establishment.

# Reference

[1] Report from [104#61][LTE/feMOB]  Solution directions for minimizing user data interruption for UL/DL (Nokia)

# Annex

### 5.3.11 Radio link failure related actions #### 5.3.11.1 Detection of physical layer problems in RRC_CONNECTED The UE shall: 1> upon receiving N310 consecutive "out-of-sync" indications for the PCell from lower layers while neither T300, T301, T304 nor T311 is running: 2> start timer T310; 1> upon receiving N313 consecutive "out-of-sync" indications for the PSCell from lower layers while T307 is not running: 2> start T313; NOTE: Physical layer monitoring and related autonomous actions do not apply to SCells except for the PSCell.

#### 5.3.11.3 Detection of radio link failure The UE shall: 1> upon T310 expiry; or 1> upon T312 expiry; or 1> upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or 1> upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB: 2> consider radio link failure to be detected for the MCG i.e. RLF; 2> except for NB-IoT, store the following radio link failure information in the VarRLF-Report by setting its fields as follows: 3> clear the information included in VarRLF-Report, if any; 3> set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN); 3> set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure; 3> set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows; 4> if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA; 4> if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA; 4> if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN; 4> if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000; 4> for each neighbour cell included, include the optional fields that are available; NOTE 1: The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported. 3> if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs; 3> if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons; 3> if detailed location information is available, set the content of the locationInfo as follows: 4> include the locationCoordinates; 4> include the horizontalVelocity, if available; 3> set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected; 3> set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected; 3> if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure: 4> if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover: 5> include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received; 5> set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo; 4> if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO: 5> include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received; 5> set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo; 3> if the UE supports QCI1 indication in Radio Link Failure Report and has a DRB for which QCI is 1: 4> include the drb-EstablishedWithQCI-1; 3> set the connectionFailureType to rlf; 3> set the c-RNTI to the C-RNTI used in the PCell; 3> set the rlf-Cause to the trigger for detecting radio link failure; 2> if AS security has not been activated: 3> if the UE is a NB-IoT UE: 4> if the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation: 5> initiate the RRC connection re-establishment procedure as specified in 5.3.7; 4> else: 5> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure'; 3> else: 4> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'; 2> else: 3> initiate the connection re-establishment procedure as specified in 5.3.7; In case of DC, the UE shall: 1> upon T313 expiry; or 1> upon random access problem indication from SCG MAC; or 1> upon indication from SCG RLC, which is allowed to be sent on PSCell, that the maximum number of retransmissions has been reached for an SCG or split DRB: 2> consider radio link failure to be detected for the SCG i.e. SCG-RLF; 2> initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure; In case of CA PDCP duplication, the UE shall: 1> upon indication from an RLC entity, which is restricted to be sent on SCell only, that the maximum number of retransmissions has been reached: 2> initiate the failure information procedure as specified in 5.6.21 to report RLC failure of type duplication; The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected, upon power off or upon detach.