3GPP TSG-RAN WG2 Meeting #105	R2-1900632

Athens, Greece, 25 Feb - 01 Mar 2019

Title:	[DRAFT] Reply LS on TSN integration in the 5G System

Response to:	S2-189051

Release:	Release 16

Work Item:	FS_NR_IIOT

Source:	Nokia [RAN2]

To:	SA2

Cc:	RAN1, RAN3, SA1, RAN

Contact Person:

#### Name:	Dawid KOZIOL

E-mail Address:	dawid.koziol@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

In S2-189051 SA2 requested RAN WGs to analyze whether KPIs relating to latency, reliability and synchronization accuracy set for TSN use cases and captured in TR 22.804 can be met.

ACTION: SA2 would like to ask RAN1, RAN2 and RAN3 whether using the existing 3GPP defined synchronisation, prioritisation and scheduling mechanisms, potentially with some enhancements within RAN, can fulfil the performance requirements defined in clause 8.1 of TR 22.804.

RAN2 responded tentatively to S2-189051 in R2-1816043. Additionally, in S1-163707, SA1 further clarified some of the TSN requirements and informed that after finalizing the related normative work, requirements of TSN use cases captured in TS 22.104 take precedence over those captured in TR 22.804.

RAN WGs have now completed the evaluation related to the request from SA2 and would like to provide a reply as below.

With respect to synchronization requirements:

- The following feedback has been already provided to SA2:

  - Synchronization accuracy achievable over the Uu interface was analyzed by RAN1 and the feedback is provided in R1-1901470

  - Synchronization accuracy between TSN GM clock and gNB was analyzed by RAN3 and the feedback is provided in R3-187252

- Additionally, RAN2 agreed that the granularity of timing information provided via RRC signalling (e.g. SIB or unicast RRC message) can be as low as 10 ns. Thus, considering this and information from RAN1 and RAN3 the overall synchronization accuracy error between a clock source and UE clock, in the worst-case scenario would be equal to 645 ns (540ns (air interface accuracy) + 100ns (network interface accuracy) + 5 ns (granularity/2)). RAN2 would like to point out that most UEs should achieve smaller errors.

- RAN2 would like to point out that the normative work for reference time information provision from RAN to UE has not yet started

With respect to latency/reliability requirements:

- RAN1 has provided their feedback on the latency and reliability achievable on Uu interface in R1-1901470

- RAN2 would like to point out that, as indicated previously in R2-1816043, further gains with respect to reliability can be achieved by using PDCP duplication, e.g. if two links operate with 10-4 reliability in PHY/MAC layer, then it is possible to achieve 10-8 reliability for a transmission on PDCP layer. PDCP duplication requires Carrier Aggregation or Dual-Connectivity support by the network and the UEs.

- With respect to latency on RAN interfaces, RAN3 feedback is provided in R3-187252

In general, it can be concluded that NR, with some additional enhancements introduced in Rel-16, can meet the requirements in TS 22.104 with respect to latency, reliability and synchronization accuracy, as discussed above, for some network configurations and deployments, but the performance depends on such aspects as utilized subcarrier spacing, FDD or TDD network type, backhaul type, cell density etc. RAN2 would like to indicate that the full analysis of latency/reliability and synchronization accuracy requirements for TSN is captured in TR 38.825, section 6.3.

2. Actions:

To SA2 group.

ACTION: 	RAN2 respectfully asks SA2 to take the information provided above into account in their related work.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#105bis		08 - 12 Apr 2018		China

3GPP RAN2#106		13 - 17 May 2018		USA