3GPP TSG-RAN WG2 Meeting #105	R2-1901378

Athens, Greece, 25 Feb - 1 Mar 2019

Agenda item:	12.3.3

Source:	China Telecom

Title: 	Configuration of CHO

Document for:	Discussion and decision

# 1.	Introduction

One new Rel-16 LTE mobility enhancement work item [1] was approved at RAN#80. The objectives of which are to identify and specify the solutions aiming to reduce user data interruption and increase robustness during handover.  During last RAN2, it is agreed that conditional handover will be considered by RAN2 with the following detailed agreements made:

Agreements

1	RAN2 will consider a conditional handover: This is defined as UE having network configuration for initiating access to a target cell based on configured condition(s).

2	Usage of conditional handover is decided by network. UE evaluates when the condition is valid.

=>	FFS on the exact details of the procedures

Agreements

1	Support configuration of one or more candidate cells for conditional handover.

=>	FFS how many candidate cells (UE and network impacts should be clarified).

In this paper, we provide our views on the data forwarding for CHO.

# 2.	Discussion

Conditional handover has be agreed to be considered by RAN2 as solution to improve handover robustness, and the next step is to work on details of the procedure. During last meeting, it is also agreed to support configuration of one or more candidate cells for conditional handover. It is not clear how many candidate cells can be configured, and it is not clear if multiple candidate cells can be configured in one CHO command or multiple CHO commands.

With regard to low threshold measurement procedure, it is possible multiple candidate cells trigger the low threshold event at the same time, thus one measurement report consists of multiple cells is sent by UE, as source eNB cannot tell which one can be the final target cell, it is reasonable to support configuring multiple candidate cells in one RRC reconfiguration message to the UE.

Proposal 1: It is supported to configure more than one candidate cell in one CHO command.

After the first CHO configuration message is sent to UE, new cells can trigger the low threshold event some time later, to configure the new cells as candidate target cell, source eNB should send another CHO command to UE.

Proposal 2: It is proposed that more than one CHO command can be sent to UE.

With the location change of mobile UE, before handover happens, it is possible that configured target cell is not valid any more, as conditional target cell occupies network resources, another issue would be how to release this invalid conditional target cell. Three possible options can be considered:

- Timer based

One timer is sent to UE together with conditional handover, and conditional target cell is considered as invalid once a configured timer is expired. However, UE autonomous release candidate cell by timer would cause state mismatches between eNB and UE, as timer synchronize between UE and eNB is hard to achieve. For instance, UE decided to handover to one candidate cell, while the eNB has already release the corresponding resource belong to the selected cell.

Besides, it will be difficult to set the timer value efficiently, e.g. too short timer will bring the issue of signalling increase as the already released candidate cell may be configured again, while too long timer cannot achieve the benefit of network resource saving.

- Condition based

The conditional target cell is considered as invalid once the condition is met, e.g. when signalling strength of the cell is below a configured threshold. Since UE and network behaviour has to be synchronised, UE should send back indication to eNB on which cell has been released in order to release the reserved resources.

- Network controlled

In this option, instead of UE releasing the candidate cell itself, the network sent explicit RRC message to UE to release conditional target cell based on measurement report from the UE and other factors like a timer in the eNB. The reserved resource can be released once source eNB received acknowledgment of the successful reception of RRC message from the UE.

Same issue is discussed in [2], network controlled solution is proposed as baseline approach, other solutions, e.g. timer based, can be discussed further. From our point of view, network controlled option is the most straightforward and flexible solution and lead to no confusion between eNB and LTE. The network can also exit condition handover mode by releasing all the configured candidate UE and removing corresponding measurement configuration.

Proposal 3: Network controlled solution should be applied to release invalid conditional target cell.

# 3.	Conclusion

In this paper, our view on the configuration of CHO is provided, and the following proposals are given:

Proposal 1: It is supported to configure more than one candidate cell in one conditional HO command.

Proposal 2: It is proposed that more than one conditional HO command can be sent to UE.

Proposal 3: Network controlled solution should be applied to release invalid conditional target cell.

# 4.	References

[1]	RP-181544, New Work Item on even further Mobility enhancement in E-UTRAN, China Telecom.

[2]	R2-1900613, Basic details of Conditional Handover in E-UTRAN.