3GPP TSG-RAN WG2#105	R2-1901415

Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fast MCG recovery in NE-DC and NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-02-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | One of the objectives of the Rel-16 WI on CA & DC enhancements is to add support for fast recovery of MCG link e.g. by utilizing the SCG link and split SRBs for recovery during MCG failure while operating under MR-DC. One way to achieve this objective for UEs in NE-DC or NR-DC is to introduce a procedure in 38.331 for MCG failure information to recover from MCG RLF for UEs configured with split SRB. In case of MCG RLF, such UE suspends MCG transmissions and sends MCG Failure Information using the SCG RLC bearer. In the case the SCG is already suspended e.g. due to SCG RLF, the UE falls back to the RRC connection re-establishment procedure and does not try to report MCG Failure Information to SCG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.7.x is added to introduce the necessary functionality to report MCG failure information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work item objective is not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.x MCG failure information |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.7.x	MCG failure information

#### 5.7.x.1	 General

Figure 5.7.x.x-x: MCG failure information

The purpose of this procedure is to inform NR MN about an MCG failure the UE has experienced i.e. MCG radio link failure, or failure of MCG reconfiguration with sync.

#### 5.7.x.2 Initiation

If MCG transmissions are not already suspended, a UE initiates the procedure to report MCG failures when SCG transmissions are not suspended and when one of the following conditions is met:

1>  upon detecting radio link failure of the MCG, in accordance with 5.3.10.3; or

1>  upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3;

Upon initiating the procedure, the UE shall:

1>	suspend MCG transmission for all SRBs and DRBs;

1>	reset MCG-MAC;

1>	stop T304, if running;

#### 5.7.x.3 Actions related to transmission of MCGFailureInformation message

The UE shall set the contents of the MCGFailureInformation message as follows:

1>	if the UE initiates transmission of the MCGFailureInformation message to provide MCG radio link failure information:

2>	include failureType and set it to the trigger for detecting MCG radio link failure;

1>	set the measResultListNR to include for each MCG cell that is configured, if any, within measResultNR the quantities of the concerned PCell, if available according to performance requirements in [x];

1>	for each MCG serving frequency included in measResultListNR, include within measResultNR the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	set the measResultNeighCells to include the best measured cells on non-serving NR frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

2>	if the UE was configured to perform measurements for one or more non-serving NR frequencies and measurement results are available, include the measResultListNR;

2>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

1>	if SRB1 is configured as Split SRB;

2>	if the primaryPath is associated with the MCG;

3> the UE shall change the primaryPath to SCG;

The UE shall submit the MCGFailureInformation message to lower layers for transmission.

END OF CHANGES