3GPP TSG-RAN WG2 #105									R2-1901527

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.8.2

Source:	Ericsson

Title:	Study on Location Server Functionality in RAN

Document for:	Discussion, Decision

# Introduction

RAN2 has been tasked to study the impacts of having Location exposure functionality in RAN (gNB). Broadly, there are two possibilities where by a RAN node (gNB) could be aware of the target device position.

- Being a client and fetching the location information from location server

- Being a server and calculating the UE position with its own means

RAN2 should study the impacts before concluding on the selection of either of the above two options or to select both options or to reject both.

In this paper, we present the study done for server functionality, we highlight some of the issues that may arise when gNB has server functionality, and we compare the server functionality with the client functionality in terms of latency.

# Discussion on Server Functionality

Moving the location server functionality to gNB may sound attractive that it may reduce latency and new positioning methods based upon gNB being the sever could be realized. However, it is also important to consider the complexity in terms of specification and implementation it will bring and if it is really worth the effort. Importantly, how much latency would be reduced.

## gNB Processor Load Increase

Positioning of UE/target device is a computationally heavy task. In the current architecture, a separate node entity (E-SMLC/LMF) exists to perform this task, offloading the base station from this computation. The gNB has multitude of tasks to perform including resource management, mobility, interactions with Core Network and UEs etc. Having the location server functionality in gNB is obviously going to increase the gNB processing load.

## Mobility (Handover)

A gNB capable of server functionality (local LMF) may handle the positioning of UEs camping in cells of the gNB. However, a mobile UE needs to perform handover to other gNBs. In such cases, the decision should be made as what should be done to the position computed or still being computed by previous gNB. Should there be transfer of location context over XnAP interface or should it be deleted as a new location context may need to be initiated in new gNB/L-LMF as the computation from the old one may not be applicable? There can be also the possibility that a central LMF takes over. It should also be noted that XnAP protocol as such is mainly used for handover support so RAN3 should decide if it is possible to be used for positioning needs.  Moreover, it should be noted that there would not be any possibility to transfer “half-cooked” computation as the local-LMF in different gNBs may use different algorithms or different versions of algorithm.

## Central and L-LMF

For some of the positioning methods, it would still be required that a central location server is present. For instance, for high accuracy GNSS Positioning method with broadcasted assistance data (e.g. RTK), the assistance data is collected by a central server and will be distributed using NRPPa to gNBs. But, with L-LMF, there could be need of distributed RTK server as well. For cost and simplicity, it may require that both central and L-LMF need to co-exist.

Further, for unicast distributed GNSS Assistance Data, AD is normally retrieved by LMF from a WARN (Wide Area Reference NW) and transported to UE using LPP. The solution works with central location Server. With L-LMF this would require dedicated transport to be setup between WARN and each gNB.

## Handling of dual Protocol

Not all positioning methods would be supported by gNB. For instance, GPS based positioning method would still be based upon LMF mainly for reasons specified above in section 2.3. Thus, there would be both central and local LMF that needs to be available.

In the current architecture, LPP messages are carried over NAS and transparently transferred by the gNB to the UE. When gNB acts as local LMF, majority of the companies’ view is to use an RRC container to transfer the LPP message to the UE.

Considering a scenario where both GMLC and gNB would like to obtain UE location; GMLC may have invoked location fetching via the traditional LMF. LMF could then initiate GPS based positioning method, thus necessary Assistance data may be carried by LPP which is then carried on top of NAS and RRC. gNB on the other hand may use RRC and its own positioning method. Thus, a UE may need to simultaneously handle LPP messages originating from gNB and Location server. This would increase complexity on the UE to handle two parallel LPP sessions, further power consumption in UE may also increase.

## gNB as Positioning Initiator

For a positioning request originating from GMLC, AMF may perform the selection among L-LMFs or central Location server. However, if there is a positioning need in gNB, it should be able to invoke the server functionality in order to obtain UE position. If a gNB being a server is not able to initiate or if an initial external trigger is needed, then it is better suited for gNB to be a client .

## Container based solution for LPP

The container-based solution need not as such have any major spec impacts. However, for a gNB to be L-LMF, it needs to basically implement the LPP ASN.1 and understand which message to select before the LPP message can be shipped in an RRC container. This would basically mean duplicating the protocol and procedure implementation between gNB (local-LMF) and cental-LMF. If any change is done in the LPP specs, the same change would be required to be duplicated in LMF and gNB.

## Resource handling in gNB

As the location server becomes one of the core entities of gNB functionality (as part of L-LMF); gNB scheduler and other RRM entities should then basically provide resources for both traffic handling and positioning estimations. gNB should have well established rules on how to share the resources between traffic handling and positioning calculations (prioritization/preemptions of procedures).

## Co-located L-LMF

One option to have the server functionality in gNB is through deployment; thus, having Location server co-located with gNB and sort of making the NRPPa protocol a “local/short-circuit” between the gNB and the LS. In this regard a server functionality would not be required and could be simply solved by the deployment. Thus, there is as such no need to specify the server functionality when it can be easily solved by deployment especially if the gain is mainly in terms of latency.

# Latency Aspects

The proponents of gNB to be equipped with server functionality mainly point out the benefit of minimizing the delay by having the server functionality closer to UE. Further, the thinking is that the gNB with client functionality need not be able to meet the commercial latency requirement.

We have done some analysis from the operator’s network where we see the latency occurred in S1AP Round Trip Time (including Processing in the Core Network Node; UEContextReleaseRequest<->UEContextReleaseCommand) interface is around 35ms. The actual message transfer time is less than 5ms, since it is all IP based.

Thus, the latency difference between client and server is not in large magnitude as the message transfer delay does not have any significant figure and it is very marginal.

gNB may not be suitable for hosting Positioning server functionality. There is not much gain in terms of latency as compared to client.

RAN2 should not select server functionality at least for Rel-16 and go with simpler solution; LCS client.

The below text proposal be captured in the TR 38.555

# Text Proposal

## 9.3	Architecture aspects

### 9.3.1	Discussion on Server Functionality

Moving the location server functionality to gNB may sound attractive that it may reduce latency and new positioning methods based upon gNB being the sever could be realized. However, it is also important to think the complexity that it will bring and if it is really worth the effort. Importantly, how much latency would be reduced.

## 9.3.1.1	gNB Processor Load Increase

Positioning of UE/target device is computationally heavy task. In current architect, a separate node entity (E-SMLC/LMF) exist to perform this task. Thus, offloading the base station from this computation. gNB has multitude of task to perform from resource management, mobility, interactions with Core Network and UE etc. Having the location server functionality in gNB is obviously going to increase the gNB processing load.

## 9.3.1.2		Mobility (Handover)

A gNB capable of server functionality (local LMF) may handle the positioning of UEs camping in cells of the gNB. However, a mobile UE needs to perform handover to other gNBs. In such cases, the decision should be made as what should be done to the position computed or still being computed by previous gNB. Should there be transfer of location context over XnAP interface or should it be deleted as a new location context may need to be initiated in new gNB/L-LMF as the computation from the old one may not be applicable? There can be also the possibility that a central LMF takes over. It should also be noted that XnAP protocol as such is mainly used for handover support so RAN3 should decide if it is possible to be used for positioning needs.

## 9.3.1.3	       Central and L-LMF

For some of the positioning methods, it would still be required that a central location server is present. For instance, for high accuracy GNSS Positioning method with broadcasted assistance data (e.g. RTK), the assistance data is collected by a central server and will be distributed using NRPPa to gNBs. But, with L-LMF, there could be need of distributed RTK server as well. For cost and simplicity, it may require that both central and L-LMF needs to co-exist.

Further, for unicast distributed GNSS Assistance Data, AD is normally retrieved by LMF from a WARN (Wide Area Reference NW) and transported to UE using LPP. The solution works with central location Server. With L-LMF; this would dedicated transport needs to be setup between  WARN and each gNB.

## 9.3.1.4	Handling of dual Protocol

Not all positioning method would be supported by gNB. For instance, GPS based positioning method would still be based upon LMF. Thus, there would be both central and local LMF that needs to be available.

In current architect, LPP message are carried over NAS and transparently shipped by the gNB to the UE. When gNB acts as local LMF, majority of the companies’ view is to use a RRC container to ship the LPP message to the UE.

Considering a scenario where both GMLC and gNB would like to obtain UE location; GMLC may have invoked location fetching via the traditional LMF. LMF could then initiate GPS based positioning method, thus necessary Assistance data may be carrier by LPP which is then carried on top of NAS and RRC. gNB on the other hand may use RRC and its own positioning method. Thus, UE at a time may simultaneously needs to handle LPP message originating from gNB and Location server.

## 9.3.1.5		gNB as Positioning Initiator

For a positioning request originating from GMLC, AMF may perform the selection among L-LMFs or central Location server. However, if there is a positioning need in gNB, it should be able to invoke the server functionality in order to obtain UE position. If gNB being a server is not able to initiate or if initial external trigger is needed, then it is better suited for gNB to be a client .

## 9.3.1.6		Container based solution for LPP

The container-based solution may not as such have any spec impacts. However, for a gNB to be L-LMF, it needs to basically implement the LPP ASN.1 before it can be shipped in a RRC container. This would basically mean duplicating the protocol implementation. If any change is done in the LPP specs, the same change would be required to be duplicated in LMF and gNB.

## 9.3.1.7		Resource handling in gNB

As location sever becomes one of the core entity of gNB functionality (as part of L-LMF); gNB scheduler and other RRM entity should then basically provide resources for both traffic handling and positioning estimations. gNB should have well established rules on how to share the resources between traffic handling and positioning calculations (prioritization/preemptions of procedures).

## 9.3.1.7		Latency Aspects

The proponents of gNB to be equipped with server functionality mainly point out the benefit of minimizing the delay by having the server functionality closer to UE. Further, the thinking is that the gNB with client functionality may not be able to meet the commercial latency requirement.

We have done some analysis from the operator’s network where we see the latency occurred in S1AP Round Trip Time (including Processing in the Core Network Node; UEContextReleaseRequest<->UEContextReleaseCommand) interface is around 35ms. The actual message transfer time is less than 5ms, since it is all IP based.

Thus, the latency difference between client and server is not in large magnitude as the message transfer delay does not have any significant figure and it is very marginal.

## 9.3.1.8	      Co-located L-LMF

One option to have the server functionality in gNB is through deployment; thus, having Location server co-located with gNB and sort of making the NRPPa protocol a “local” protocol between gNB and LS. In this regard a server functionality would not be required and could be simply solved by the deployment.

gNB may not be suitable for hosting Positioning server functionality. There is not much gain in terms of latency as compared to client.

RAN2 should not select server functionality at least for Rel-16 and go with simpler solution; LCS client.

# Conclusion

In section 2 we made the following observations:

Observation 1	gNB may not be suitable for hosting Positioning server functionality. There is not much gain in terms of latency as compared to client.

Observation 2	gNB may not be suitable for hosting Positioning server functionality. There is not much gain in terms of latency as compared to client.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 should not select server functionality at least for Rel-16 and go with simpler solution; LCS client.

Proposal 2	The below text proposal be captured in the TR 38.555

Proposal 3	RAN2 should not select server functionality at least for Rel-16 and go with simpler solution; LCS client.

# References

SA2, 23.731 “Study on Enhancement to the 5GC Location Services”