3GPP TSG RAN WG2#105										R2-1901533

Athens, Greece, 25th February – March 1st, 2019

Agenda Item:	11.8.3

Source:	Ericsson

Title:		Co-ordination among UE, gNB and Location Server for PRS Configuration   and Feedbacks

Document for:	Discussion, Decision

# 1	Introduction

With every new generation of mobile communication, there is objective and motivation to improve the functionality and services. Moving from LTE to NR, there are new commercial use cases such I-IOT and need to further improve location accuracy of existing use cases such as Aerials, V2X, NB-IoT etc. OTDOA based positioning method is expected to be one of the important positioning methods in NR similar to in LTE. Though the exact reference signal to be used is still unknown for OTDOA in NR, in this paper a generic name NR Positioning Reference Signal (PRS) has been assumed. The physical reference signal could very well be CRS, TRS, PSS/SSS etc.

# 2	Discussion

In LTE, Location server obtains PRS configuration from eNB and provides the PRS configuration to UE using LPP signalling. There is opportunity to provide feedback on the quality of the measurement especially in terms of OTDOA error resolution and the target device's best estimate of the uncertainty of the OTDOA (or TOA) measurement.

Moving forward to NR, there is a scope to further improve the co-ordination between the target device and Network entity. The NR technology is expected to provide a more dynamic PRS configurations than its predecessor LTE. A fundamental difference between LTE and NR is that PRS would be transmitted via beams rather than cells. There could be multiple beams as compared to cell sectors in LTE. UE may be receiving good quality signal from some beams and poor-quality signal from other beams. Dynamic PRS configuration could efficiently utilize the network resources by allowing PRS to be adaptive based upon the UE perceived quality of various beams transmitting the PRS signal. This scheme may also allow gNB to save energy on PRS transmission by filtering out the poor-quality beams from PRS transmission. The improved feedback would in turn help UE in estimating RSTD with low error resolution.

When there are few number of UEs in a cell or gNB that are using positioning session, in such scenario, it is possible to provide a more tailored made (customized) PRS configuration for these UEs. A dynamic, per position occasion, PRS configuration.

Support PRS configuration to be dynamic.

When there are multiple UEs in a cell/beam, it may be difficult to realize dynamic PRS configuration per UE. Instead, it is more effective, the location server collects statistics measurement from a first set of target devices, and applies the new PRS configurations (i.e. second/or third PRS configurations) to the second set of target devices for positioning in later time.

This scheme would allow that the PRS resources are only spent on cells that:

- provide low Geometric Dilution of precision (GDOP)

- have high quality RSTD (e.g. cell close to UE),

- have high probability of being in LOS condition, etc.

Support PRS configuration to be adaptive based upon statistical measurement analysis.

Dynamic/Adaptive PRS configuration would imply

- Increasing PRS configuration in a cell/beam (e.g., increase one or more of: bandwidth, density, number of PRS REs within a RB and/or subframe, number of PRS subframes per positioning occasion, etc.)

- Decreasing PRS configuration in a cell/beam (e.g., decrease one or more of reduce bandwidth, density, number of PRS REs within a RB and/or subframe, number of PRS subframes per positioning occasion, etc.)

- Sorting of the neighbor beam/cell list

- Adding/removing beam/cell from the measurement list

The important aspect in realizing dynamic/adaptive PRS configuration is to be able to obtain UE feedback efficiently. The feedback would mainly be in terms of below.

- PRS quality (e.g., received signal quality and/or strength, Es/Iot, SINR, RSRQ, CSI etc.)

- Location Measurement quality or uncertainty (e.g., for UE-based positioning the UE may still assist the location server or radio network nodes transmitting PRS in optimizing PRS to improve its location estimation)

- GDOP

- Channel and LOS/NLOS characteristics

- Correlation property with a known PRS signal based on a first configuration

- Power Delay Profile

- A statistical measure over time of any of the above, e.g., average, Xth percentile, median,

- An indication of one or more cells contributing to one or more of the above being below a first threshold and/or above a second threshold

Provision to allow UE to provide a more detailed PRS quality feedback including other environment characteristics so that the Network can adapt the PRS configuration accordingly

Another important aspect in dynamic or statistics based PRS configuration is that, gNB may play a more active role in providing the PRS configuration as compared to in LTE. UE can inform gNB directly (over RRC) the PRS quality, allowing gNB to adjust the PRS transmission and later informing the Location server accordingly.

gNB in NR may play more active role in providing the PRS configuration as compared to eNB in LTE.

# Conclusion

In the previous sections we made the following observations:

Observation 1	gNB in NR may play more active role in providing the PRS configuration as compared to eNB in LTE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support PRS configuration to be dynamic.

Proposal 2	Support PRS configuration to be adaptive based upon statistical measurement analysis.

Proposal 3	Provision to allow UE to provide a more detailed PRS quality feedback including other environment characteristics so that the Network can adapt the PRS configuration accordingly

# References

RP-182155, Study on NR positioning support, RAN1, Intel Corporation, Ericsson, June 2019.

3GPP TS 36.355, Stage 3 LPP