3GPP TSG-RAN WG2 #105	Tdoc R2-1901561

Athens, US, 25th February – 1st March 2019

Agenda Item:	11.6.3.2

Source:	Ericsson

Title:	On RLC impacts for NTN

Document for:	Discussion, Decision

# Introduction

In RAN#80, a new SI “Solutions for NR to support Non-Terrestrial Network” was agreed [1]. It is a continuation of the preceding SI “NR to support Non-Terrestrial Networks” (RP-171450), where the objective was to study the channel model for the non-terrestrial networks, to define deployment scenarios, parameters and identify the key potential impacts on NR. The results are summarized in [2]. The new study item has the objective at evaluating potential solutions addressing the minimum necessary identified key impact areas from the previous activity and to study impact on RAN protocols/architecture.

In RAN2#103bis, it is agreed to study the following UP and CP aspects:

UP Impacts to study

- DRX

- HARQ

- Random access response

- RLC/PDCP reordering (e.g. timers and SN space)

- SDAP => no impact

Impacts to study for CP

- Mobility

- TA management and update

In RAN2#104, the following agreements on UP were reached:

RACH Agreements:

-	Discussion on 2-step RACH will be postponed until the procedures are more stable.

HARQ Agreements:

-	Both options (enhancing HARQ and disabling HARQ) will be studied.

RLC Agreements:

-	All RLC modes are supported.

-	Study the need to extend the RLC/PDCP SN and window sizes based on throughput requirements.

In this paper, we discuss RLC for NTN operation.

# Discussion

Some important features of the RLC layer is to offer reliable, in order delivery and error free communication by using ARQ with status reporting, segmentation. Some of these services are making use of timers that might be affected by the long propagations delays of a non-terrestrial network. Extension of timers is typically an easy solution to solve some problems. However, care should be taken so that buffer-limitations and sequence number space is considered.

This contribution is mostly focused on issues relating to RLC AM for NR NTN.

## Supported RLC modes

RLC can be configured in one of three different modes;

- transparent (TM)

- unacknowledged (UM)

- acknowledged mode (AM)

During previous meeting it was discussed regarding which RLC modes that should supported for NTN. In order to ensure that the system remains flexible, we believe that all RLC modes should be supported and we also believe that the specification impact of keeping all RLC modes supported and relevant for NTN is relatively low.

RAN2 to conclude that no RLC mode shall be excluded for NR NTN.

## Reassembly

In a previous contribution [4] and in the e-mail discussion on User plane timers it was mentioned that the t-reassembly need to be extended to ensure that HARQ is able to deliver the transport block before the timer expires, if HARQ is supported. When evaluating the needed time for HARQ to deliver the PDU some factors, mainly maximum number of HARQ retransmissions, and round-trip delay needs to be considered. A rough calculation could be:

t-reassembly = RTT * nrof_HARQ_retrans

Where nrof_HARQ_retrans is a constant that is implementation-dependent constant that could be configurable. Furthermore, to account for HARQ scheduling delays, a minimum value for t-reassembly can be added to the value as:

t-reassembly = RTT * nrof_HARQ_retrans + minimum_timer_value

This would ensure that the HARQ delay can be correctly accounted for in t-reassembly.

RAN2 to consider the above mentioned method for calculating the needed extension of t-reassembly.

## Sequence number space

As discussed in a previous contribution [4], the Sequence Number (SN) space was analysed through initial calculations. We provide some updates on the assumptions in this contribution.

For NR, the Sequence Number space was extended to allow for higher rates. What is different to LTE RLC is that in NR, a new sequence number is generated only for each SDU, rather than for each PDU, but on the other hand the RLC does not support concatenation. For RLC the sequence number space is 6 or 12 bits for RLC UM and 12 or 18 bits for RLC AM. And in the below cases, we consider that HARQ is not limiting the maximum rate, meaning that HARQ is turned off.

The basic supportable RLC bit rate calculation is:

Supportable RLC bit rate = ((RLC SDU size (bits) x 2 ^ (SN length – 1)) / Retransmission time (s).

For selecting reasonable values:

- SN length: Selecting the SN length depends on the application, but for continuous and high-rate applications, the SN length should be chosen to be large.

- RLC SDU size: depends entirely on the specific traffic and it is difficult to give a good estimate for a typical SDU size. For continuous data, it is probably more likely that the RLC SDUs are bigger rather than small. Sizes of 500B and 1500B are considered here.

- Retransmission times: During continuous uplink transmissions, the gNB may decide when to schedule RLC status reports meaning that the retransmission time is likely to be short compared to the downlink case when UE needs uplink resources to send RLC status reports. However, in the case of continuous downlink transmissions the UE should have uplink resources due to configured grant. A reasonable retransmission time is 2 RTTs, but we also consider the case where retransmission time is 6 RTTs.

| RLC SDU size | Retransmission time | SN length | Supportable bit rate |
| --- | --- | --- | --- |
| 500B | 1 | 18 | ~ 524 Mbps |
| 1500B | 1 | 18 | ~ 1.57 Gbps |
| 500B | 3 | 18 | ~ 175 Mbps |
| 1500B | 3 | 18 | ~ 524 Mbps |

Table 1. Supportable bit rates

In the above table the supportable bit rates can be seen. For a SN length of 18 bits, the rates should be enough for even more demanding applications.

Given the above assumptions, the RLC SN space should be enough for the throughput considered for NR NTN.

RAN2 to agree to capture the above calculations in the TR.

# Conclusion

In section 2 we made the following observations:

Observation 1	Given the above assumptions, the RLC SN space should be enough for the throughput considered for NR NTN.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 to conclude that no RLC mode shall be excluded for NR NTN.

Proposal 2	RAN2 to consider the above mentioned method for calculating the needed extension of t-reassembly.

Proposal 3	RAN2 to agree to capture the above in the TR.

# References

RP-181370, New SID on Solutions for NR to support Non Terrestrial Network, Thales, RAN#80, La Jolla, USA, June 2018

TR 38.811, Study on New Radio (NR) to support non terrestrial networks, Release 15

TR 38.821, Solutions for NR to support Non Terrestrial Network

R2-1817767, RLC for NTN, Ericsson, RAN2#104, Spokane, USA, November 2018