3GPP TSG-RAN WG2 Meeting #105	R2-1901772

Athens, Greece, 25th February – 1st March 2019

Agenda Item	: 11.7.2.2 (FS_NR_IIOT)

Source	: LG Electronics Inc.

Title	: TP on Measurement gap enhancement for TSN traffic

Document for	: Decision

1	Introduction

This contribution provides a text proposal to TR 38.825 to capture that measurement gap enhancement is required to support various TSN traffic based on its own requirement.

<START>

6.2	TSN use cases, scenarios and architectures

Editor’s note: RAN3 responsibility

6.3	TSN performance evaluation

6.3.1	Requirements

Editor’s note: RAN2 responsibility

6.3.2	Physical layer aspects

Editor’s note: RAN1 responsibility

6.3.3	Protocol aspects

Editor’s note: RAN2 responsibility

6.3.4	Radio access network aspects

Editor’s note: RAN3 responsibility

6.4	Accurate reference timing provisioning

Editor’s note: RAN2 responsibility with potential network interfaces impacts handled by RAN3

6.5	QoS and scheduling enhancements

Editor’s note: RAN2 responsibility with potential PHY impacts handled by RAN1

According to TR 22.804 [3], TSN traffic needs to be transmitted within bounds, e.g., 1ms. In the meanwhile, the network may configure a UE to perform measurements and report measurement results. According to the TS 38.133, the length of measurement gap could be configured from 1.5ms to 6ms depending on the frequency of the serving and target cell. Even if the measurement gap is set to 1.5ms, the QoS requirements for TSN traffic cannot be met for a UE in gap-assisted scenarios. Thus, it is necessary to enhance the measurement gap related operation in order to satisfy the QoS requirement as well as guarantee the coverage of a UE. For example, a UE can skip the measurement gap or network can indicate to skip the measurement gap when TSN traffic collides with measurement gap.