3GPP TSG-RAN WG2 Meeting #105	R2-1901886

# Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0629 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SR_COUNTER initialization due to RRC reconfiguration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fujitsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | [Background] There is an unclarity on how to set SR_COUNTER when SR configuration to a logical channel is changed from e.g. SR configuration X (i.e. schedulingRequestID = X) to SR configuration Y (i.e. schedulingRequestID = Y) due to RRC reconfiguration w/o MAC reset (The exact procedure is specified in Section 5.3.5.5.5 of TS38.331). For example, SR configuration X has been initially configured to a logical channel and SR is triggered by the logical channel. According to the current SR procedure, the triggered SR is considered as a pending SR to the SR configuration X. With keeping the SR is pending by the logical channel, the logical channel is reconfigured so that the SR configuration becomes schedulingRequestID = Y due to some propoer reasons e.g. using better PUCCH resources, shortage of PUCCH resources in SR configuration X, and so on. In this case, there is a potential problem with initialization of SR_COUNTER of SR configuration Y. [Issue to be addressed] According to the current specification, SR_COUNTER corresponds to a SR configuration is set to 0 when an SR is triggered. If an SR is triggered and there are no other SRs pending corresponding to the same SR configuration, the MAC entity shall set the SR_COUNTER of the corresponding SR configuration to 0.. However, there are no clear behaviour in the above example case i.e. SR configuration X has pending SR which is triggered by the logical channel and the pending SR is taken over to SR configuration Y due to the RRC reconfiguraion w/o MAC reset. As a result, SR configuration Y keeps the pending SR. Here, note that the pending SR kept in SR configuration Y cannot be referred to as triggered SR. Therefore, SR_COUNTER of SR configuration Y cannot be set to 0 if there are no other pending SRs in SR configuration Y because the current spec states that only triggered SR can set SR_COUNTER to 0. When an SR transmission corresponding to the pending SR is instrucuted to the physical layer, SR_COUNTER is incremented based on an unknow value, which can be any values other than 0 becuase the initial value of SR_COUNTER corresponds to SR configuration Y can be any values (depending on operating system on how to set initial values to any valuables or parameters). The increment based on unkonw value would be avoided because SR_COUNTER may immidiately hit sr-TranxMax, which causes to release PUCCH resources, SRS resources, and so on. Typicall, the network will properly configure sr-TransMax to avoid sucu resource releases and reconfiguration after that. Therefore, SR_COUNTER should also propely count the number of SR instructions to PHY from 0 to avoid such resource releases and reconfiguration after that. |
| Summary of change: |  | To address the issue indicated above, it is proposed that SR_COUNTER is also set to 0 when the SR configuration with pending SRs is changed from SR configuration X to SR configuration Y, for which the pending SR is allowed to initialize SR_COUNTER of SR configuration Y. Impact analysis Impacted functionality: SR procedure i.e. SR_COUNTING Inter-operability: - If the UE is implemented according to this CR but the network is not, there is no backward compatibility problem. - If the network is implemented according to this CR but the UE is not, there is no backward compatibility problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SR_COUNTER corresponds to SR configuration Y cannot be initialized when a pending SR is taken over from SR configuration X to SR configuration Y. Consequently, there is difficulity in properly counting the number of SR transmnissions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.4.4	Scheduling Request

The Scheduling Request (SR) is used for requesting UL-SCH resources for new transmission.

The MAC entity may be configured with zero, one, or more SR configurations. An SR configuration consists of a set of PUCCH resources for SR across different BWPs and cells. For a logical channel, at most one PUCCH resource for SR is configured per BWP.

Each SR configuration corresponds to one or more logical channels. Each logical channel may be mapped to zero or one SR configuration, which is configured by RRC. The SR configuration of the logical channel that triggered the BSR (subclause 5.4.5) (if such a configuration exists) is considered as corresponding SR configuration for the triggered SR.

RRC configures the following parameters for the scheduling request procedure:

-	sr-ProhibitTimer (per SR configuration);

-	sr-TransMax (per SR configuration).

The following UE variables are used for the scheduling request procedure:

-	SR_COUNTER (per SR configuration).

If an SR is triggered and there are no other SRs pending corresponding to the same SR configuration, or there are SRs pending corresponding to SR configuration without initialization of SR_COUNTER, the MAC entity shall initialize the SR_COUNTER of the corresponding SR configuration to 0.

(omit descriptions)