3GPP TSG-RAN WG2 #105			    R2-1902175

Athens, Greece, 25th February – 1st March 2019

Agenda Item	: 11.10.4	 (LTE_NR_DC_CA_enh-Core)

Source	: LG Electronics Inc.

Title	: Fast cell setup for MR-DC and CA in RRC inactive

Document for	: Discussion and Decision

1.	Introduction

According to the [1], the objective of this MR-DC and CA enhancement work item is considered as following topics:

- Support of asynchronous and synchronous NR-NR Dual Connectivity [RAN1, RAN2, RAN4] - UE power control [RAN1] - RRC signalling to support of enhanced NR-NR DC [RAN2] - Core requirements to support enhanced NR-NR DC [RAN4] Note: Synchronous DC enhancements in this WID considers only cases not covered in Rel-15 exception sheet for NR WI NR_newRAT-Core. - Early Measurement reporting: Early and fast reporting of measurements information availability from neighbor and serving cells to reduce delay setting up MR-DC and/or CA. [RAN2, RAN4] - This objective applies to MR-DC, NR-NR DC and CA - The objective should consider measurements in IDLE, INACTIVE mode and CONNECTED mode - The impacts on UE power consumption should be minimized - The LTE Rel-15 euCA work should be utilized, when applicable - Efficient and low latency serving cell configuration/activation/setup: Minimizing signalling overhead and latency needed for initial cell setup, additional cell setup and additional cell activation for data transmission. [RAN2, RAN1, RAN4, RAN3] - This objective applies to MR-DC, NR-NR DC and CA - The objective should consider enhancements when starting from IDLE, INACTIVE mode and CONNECTED mode - Fast recovery: Support fast recovery of MCG link e.g. by utilizing the SCG link and split SRBs for recovery during MCG failure while operating under MR-DC. [RAN2, RAN3] - This objective applies to MR-DC and NR-NR DC - Cross-carrier scheduling with different numerologies on the scheduling and scheduled carriers [RAN1, RAN2, RAN4] - This objective applies to CA only. - Target completion by RAN#84.

In this contribution, we would like to investigate how to support fast cell setup for MR-DC and CA.

2.	Discussion

According to RRC specifications, the UE release SCG lower layer configuration when the UE initiate Resume procedure.

In RAN2 #103bis meeting, RAN2 agreed the followings:

Agreements

1:	For NE-DC, NGEN-DC, and NR-DC, the UE stores the PDCP/SDAP configuration when moving to INACTIVE.  When resuming, the UE applies the stored PDCP/SDAP configuration as in NR SA/LTE.

2	A UE releases its lower-layer SCG configuration in RRC_INACTIVE

3	RAN2 understand that discussion on keeping the lower layer SCG configuration in Inactive would fit within the existing scope of the DC/CA enhancements WI

4	RAN2 understand that before suspend or in the resume message, the network should reconfigure any SCG only bearers so that at completion of the resume the UE is not left with any DRB without any RLC bearer (may not need to be captured in the specification)

According to the highlighted part in the agreements, RAN2 need to discuss whether the UE SCG lower layer configuration is kept when the UE initiate Resume procedure in this work item.

Observation 1: RAN2 needs to discuss whether the SCG lower layer configuration is kept when the UE initiate Resume procedure.

If RAN2 agree to release SCG lower layer configuration, two kinds of methods can be considered to support fast cell setup for MR-DC and CA as follow:

- Early measurement reporting in MSG5

- Blind configuration in RRC Resume message

According to current specifications, the network needs to receive measurement report from the UE to setup cells for MR-DC or CA. To support fast setup cells for CA, early measurement reporting (i.e. idle measurement reporting) has been introduced in LTE Rel-15 euCA work. The measurement report can be provided via UEInformationResponse message upon receiving UEInformationRequest message after sending MSG5. It can be faster than pre-Rel-15 but may not be enough. As much faster way, we can consider the UE provides the measurement report in MSG5. Upon receiving it, the network performs PSCell/SCell and send RRCConnectionReconfiguration message including PSCell/SCell configuration. Upon receiving the RRCConnectionReconfiguration message, the UE performs PSCell addition and SCell addition. It may require radio resource and processing to provide measurement results to the network and new cell configuration(s) to the UE. In the network, time/processing to setup cell(s) newly is required. (e.g., much signalling/processing of MN and SN require to add PSCell) Moreover, the UE need to wait the new cell configuration from the network to initiate cell setup.

Observation 2: Early measurement reporting in MSG5 may require signalling and latency as compared to use stored configuration.

The network doesn’t have measurement information sent from the UE when transmitting RRC resume message. Thus, the network don’t know which cell(s) is valid (or suitable) to the UE. Due to the reason, it has been proposed for blind SCG configuration where the network determines SCG configuration blindly (i.e. without measure reporting from the UE) and provides the PSCell/SCell configuration to the UE via RRC resume message. It may be assumed that the network can estimate valid cells of the UE. However, the estimation may not be correct. Then, the UE may not be able to apply the blind configuration. For example, the network provides PSCell/SCell configuration(s) blindly in RRC resume message but some cell(s) in the configuration may not be valid or suitable to the UE (e.g. due to low signalling strength). Then, the UE cannot apply the cell configuration provided by the network. In the worst case, all cell(s) in the blind configuration may not be valid or suitable to the UE. Then, the cell configuration(s) provided in RRC resume message will be wasted. Additional signalling(s) is required to resolve the misconfiguration by the network.

Observation 3: Blind configuration may require additional signallings to resolve misconfiguration if the network provide cell(s) configuration blindly but the cell(s) is not valid to the UE.

If RAN2 agrees to keep SCG lower layer configuration, signalling overhead to provide PSCell/SCell configuration (e.g. in RRCConnectionReconfiguration message) can be reduced. Signalling/Latency/processing to add PSCell and SCell can be also reduced by performing just to resume stored configuration instead of performing new cell addition. Moreover, the UE initiates cell setup fast by using stored cell configuration. For example, the UE initiates uplink transmission and synchronization to the cell as soon as the network identifies the cell is resumed.

Observation 4: If RAN2 agrees to keep SCG lower layer configuration, signalling overhead and latency can be reduced to setup cell(s).

Proposal 1: RAN2 agrees to keep SCG lower layer configuration even if the UE initiate resume procedure.

If SCG lower layer configuration is kept, some cells in the stored configuration may not be valid to the UE because the environment such as UE’s location or TA validity is changed over time. It is not likely that the cell configuration which was stored when the UE enters into RRC inactive mode is valid until the UE initiates the resume procedure. If the network resumes just some of stored cell configuration(s) without information for whether cell(s) is valid, it causes some misconfiguration as blind configuration. The UE can identify whether cells in the stored configuration is valid by measuring signalling strength or checking suitability of the cell(s) while the network cannot. Thus, it is beneficial the UE provides validity information for cell(s) in the stored configuration. Upon receiving the validity information, the network can resume only valid cell(s) to the UE. Obviously, it can avoid misconfiguration.

Observation 5: If UE provides validity information of cells in stored configuration to the network, signalling overhead and latency due to misconfiguration can be reduced.

Proposal 2: RAN2 agrees UE to provide validity information of cells in stored configuration to the network.

3.	Conclusion

In this contribution, we address how to support fast cell setup for MR-DC and CA and our proposals are as follows.

Observation 1: RAN2 needs to discuss whether the SCG lower layer configuration is kept when the UE initiate Resume procedure.

Observation 2: Early measurement reporting in MSG5 may require signalling and latency as compared to use stored configuration.

Observation 3: Blind configuration may require additional signallings to resolve misconfiguration if the network provide cell(s) configuration blindly but the cell(s) is not valid to the UE.

Observation 4: if RAN2 agrees to keep SCG lower layer configuration, signalling overhead and latency can be reduced to setup cell(s).

Proposal 1: RAN2 agrees to keep SCG lower layer configuration even if the UE initiate resume procedure.

Observation 5: If UE provide validity information of cells in stored configuration to the network, signalling overhead and latency due to misconfiguration can be reduced.

Proposal 2: RAN2 agrees UE to provide validity information of cells in stored configuration to the network.

References

[1] RP-182076, WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements.