3GPP TSG-RAN WG2 Meeting #106	R2-1907465

Reno, USA, 8th – 12th May 2019

Agenda Item:	12.2.8

Source:	Ericsson

Title:	Text proposal for adding new SIB in 36.300

Document for:	Discussion, Decision

# Introduction

In order to prepare a NB-IoT device for inter-RAT cell selection, the UE needs to be configured with supported radio access technologies. Cell selection occurs, e.g., at power on. Hence, the information needs to made available already with system information. In this contribution we propose text needed for the SIB. Whether this information is added to existing SIB or to a new SIB is debatable; arguments are presented in the accompanying contribution. This contribution adds specification text in 36.300 needed for a new SIB.

# References

RP-190770. Revised WID: Additional MTC enhancements for LTE 3GPP TSG RAN meeting #83 Shenzhen, China, March 18-21, 2019

RP-190757 WID revision: Additional enhancements for NB-IoT, 3GPP TSG RAN meeting #83, Shenzhen, China, March 18-21, 2019	(revision of RP-182902)

R2-19xxxxx – Adding new system information on IRAT cell selection for NB-IoT

TS 36.300 Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 15)

# Text Proposal

## 7.4	System Information

[text omitted]

System information for NB-IoT is divided into the MasterInformationBlock-NB (MIB-NB) and a number of SystemInformationBlocks-NB (SIBs-NB):

-	MasterInformationBlock-NB defines the most essential information of the cell required to receive further system information;

-	SystemInformationBlockType1-NB contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2-NB contains common radio resource configuration information;

-	SystemInformationBlockType3-NB contains cell re-selection information for intra-frequency, inter-frequency;

-	SystemInformationBlockType4-NB contains neighboring cell related information relevant for intra-frequency cell re-selection;

-	SystemInformationBlockType5-NB contains neighboring cell related information relevant for inter-frequency cell re-selection;

-	SystemInformationBlockType14-NB contains information about access barring;

-	SystemInformationBlockType15-NB contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16-NB contains information related to GPS time and Coordinated Universal Time (UTC);

-	SystemInformationBlockType20-NB contains information related to SC-PTM;

-	SystemInformationBlockType22-NB contains common radio resource configuration information for paging and random access procedure on non-anchor carriers;

-	SystemInformationBlockType23-NB contains common additional radio resource configuration information for random access procedure on anchor and non-anchor carriers.

-	SystemInformationBlockTypeXY-NB contains information about other E-UTRA and GERAN frequencies relevant for cell selection