3GPP TSG-RAN WG2 meeting #106	 R2-1908224

Reno, Nevada, US, 13 – 17 May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3949 | rev | 3 | Current version: | 13.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction in the field description of aperiodicCSI-Trigger |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_enh_b5C-Core |  |  |  |  |  | Date: |  |  | 2019-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction on the description of the usage of aperiodicCSI-Trigger : With the introduction of B5CA, the number of bits used for the aperiodic CSI trigger is increased from 2 to 3 bits. Along with the increase of this field from release-13, the interpretaion of the bits is updated in the field description of IE aperiodicCSI-Trigger. However, the description in TS 36.331 is not clear on which trigger set (trigger1-r10 or trigger1-13) the UE should use in cases the UE is configured with more than 4 SCells or with 4 SCells or less. And in the case where the NW has configured both trigger1-r10 (using aperiodicCSI-Trigger-r10) and trigger1-r13 (using aperiodicCSI-Trigger-v1310), it is not clear which trigger set the UE should use for the 2 bit CSI trigger in the DCI. Since TS 36.213 section 7.2.1 clearly describes the DCI bit-length the UE should use based on the number of SCells the UE is configured with, a correction is needed to the field description in 36.331 which is aligned with this. Correction on the applicability of serving cells for aperiodicCSI-Trigger: PUCCH SCell feature was introduced in release-13 and as part of this, the CSI feedback for some of the serving cells can be sent by the UE on the PUCCH SCell (instead of the PCell). RAN1 specification also reflects such agreements on the CSI feedback procedures such that the serving cells in CA are to be associated with either the PCell group or the PUCCH SCell group depending on the respective serving cell which carries the PUCCH of these serving cells. TS36.213 captures the below: ## 7.2 UE procedure for reporting Channel State Information (CSI) If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group unless stated otherwise - When the procedures are applied for the primary PUCCH group, the terms ‘secondary cell’, ‘secondary cells’, ‘serving cell’, and ‘serving cells’ in this clause refer to secondary cell, secondary cells, serving cell or serving cells belonging to the primary PUCCH group respectively unless stated otherwise. - When the procedures are applied for secondary PUCCH group, the terms ‘secondary cell’, ‘secondary cells’, ‘serving cell’ and ‘serving cells’ in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively unless stated otherwise. The term ‘primary cell’ in this clause refers to the PUCCH-SCell of the secondary PUCCH group. Based on the above, the aperiodic CSI reporting procedure also gets affected in terms of the applicable serving cells that a DCI based CSI trigger can trigger for CSI reporting. If PUCCH SCell is configured, the set of serving cells for which the CSI feedback is requested by aperiodicCSI-Trigger, is limited to the PCell or PUCCH SCell group. However the field description of aperiodicCSI-Trigger does not accurately reflect this. The description still refers to ‘all’ serving cells instead of the group of serving cells which share the same PUCCH (on PCell or the PUCCH SCell). This CR corrects the field description to reflect the RAN1 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UE uses trigger1-r10 (using aperiodicCSI-Trigger-r10) when the number of SCells are 4 or less (2 bit DCI) and that the UE uses trigger1-r13 (using aperiodicCSI-Trigger-v1310) when the number of SCells are more than 4 (3 bit DCI). Clarify that the serving cells that are triggered by aperiodicCSI-Trigger are limited to cell group which share the same PUCCH (of either the PCell or the PUCCH SCell). Impact Analysis: Impacted functionality: E-UTRA CA (B5CA operation) Inter-operability: There is no inter-operability issue since the correction is to align TS 36.331 with TS 36.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is ambiguity at the UE on which trigger set to use, specially when the NW configures both trigger1-r10 (using aperiodicCSI-Trigger-r10) and trigger1-r13 (using aperiodicCSI-Trigger-v1310). Also there is mis-alignment between TS36.213 and TS36.331 on the set of serving cells that can be triggered using aperiodicCSI-Trigger when PUCCH SCell is configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

### 6.3.2	Radio resource control information elements

<<skip irrelevant parts>>

#### –	CQI-ReportConfig

The IE CQI-ReportConfig is used to specify the CQI reporting configuration.

CQI-ReportConfig information elements

-- ASN1START

CQI-ReportConfig ::=				SEQUENCE {

cqi-ReportModeAperiodic			CQI-ReportModeAperiodic	OPTIONAL,			-- Need OR

nomPDSCH-RS-EPRE-Offset				INTEGER (-1..6),

cqi-ReportPeriodic				CQI-ReportPeriodic	OPTIONAL				-- Need ON

}

CQI-ReportConfig-v920 ::=		SEQUENCE {

cqi-Mask-r9						ENUMERATED {setup}		OPTIONAL,		-- Cond cqi-Setup

pmi-RI-Report-r9				ENUMERATED {setup}		OPTIONAL		-- Cond PMIRI

}

CQI-ReportConfig-r10 ::=	SEQUENCE {

cqi-ReportAperiodic-r10				CQI-ReportAperiodic-r10			OPTIONAL,	-- Need ON

nomPDSCH-RS-EPRE-Offset			INTEGER (-1..6),

cqi-ReportPeriodic-r10				CQI-ReportPeriodic-r10			OPTIONAL,	-- Need ON

pmi-RI-Report-r9					ENUMERATED {setup}				OPTIONAL,	-- Cond PMIRIPCell

csi-SubframePatternConfig-r10		CHOICE {

release							NULL,

setup							SEQUENCE {

csi-MeasSubframeSet1-r10			MeasSubframePattern-r10,

csi-MeasSubframeSet2-r10			MeasSubframePattern-r10

}

}																	OPTIONAL	-- Need ON

}

CQI-ReportConfig-v1130 ::=	SEQUENCE {

cqi-ReportPeriodic-v1130			CQI-ReportPeriodic-v1130,

cqi-ReportBoth-r11					CQI-ReportBoth-r11

}

CQI-ReportConfig-v1250 ::=		SEQUENCE {

csi-SubframePatternConfig-r12		CHOICE {

release							NULL,

setup							SEQUENCE {

csi-MeasSubframeSets-r12			BIT STRING (SIZE (10))

}

}																OPTIONAL,	-- Need ON

cqi-ReportBoth-v1250					CQI-ReportBoth-v1250		OPTIONAL,	-- Need ON

cqi-ReportAperiodic-v1250	CQI-ReportAperiodic-v1250		OPTIONAL,	-- Need ON

altCQI-Table-r12			ENUMERATED {

allSubframes, csi-SubframeSet1,

csi-SubframeSet2, spare1}		OPTIONAL		-- Need OP

}

CQI-ReportConfig-v1310 ::=			SEQUENCE {

cqi-ReportBoth-v1310				CQI-ReportBoth-v1310		OPTIONAL,	-- Need ON

cqi-ReportAperiodic-v1310			CQI-ReportAperiodic-v1310	OPTIONAL,		-- Need ON

cqi-ReportPeriodic-v1310			CQI-ReportPeriodic-v1310	OPTIONAL		-- Need ON

}

CQI-ReportConfig-v1320 ::=			SEQUENCE {

cqi-ReportPeriodic-v1320			CQI-ReportPeriodic-v1320	OPTIONAL	-- Need ON

}

CQI-ReportConfigSCell-r10 ::=				SEQUENCE {

cqi-ReportModeAperiodic-r10			CQI-ReportModeAperiodic OPTIONAL,			-- Need OR

nomPDSCH-RS-EPRE-Offset-r10				INTEGER (-1..6),

cqi-ReportPeriodicSCell-r10			CQI-ReportPeriodic-r10			OPTIONAL,	-- Need ON

pmi-RI-Report-r10					ENUMERATED {setup}				OPTIONAL	-- Cond PMIRISCell

}

CQI-ReportPeriodic ::=		CHOICE {

release							NULL,

setup							SEQUENCE {

cqi-PUCCH-ResourceIndex				INTEGER (0..1185),

cqi-pmi-ConfigIndex					INTEGER (0..1023),

cqi-FormatIndicatorPeriodic			CHOICE {

widebandCQI							NULL,

subbandCQI							SEQUENCE {

k									INTEGER (1..4)

}

},

ri-ConfigIndex						INTEGER (0..1023)	OPTIONAL,				-- Need OR

simultaneousAckNackAndCQI			BOOLEAN

}

}

CQI-ReportPeriodic-r10 ::=		CHOICE {

release								NULL,

setup								SEQUENCE {

cqi-PUCCH-ResourceIndex-r10			INTEGER (0..1184),

cqi-PUCCH-ResourceIndexP1-r10		INTEGER (0..1184)				OPTIONAL,	-- Need OR

cqi-pmi-ConfigIndex				INTEGER (0..1023),

cqi-FormatIndicatorPeriodic-r10		CHOICE {

widebandCQI-r10						SEQUENCE {

csi-ReportMode-r10		ENUMERATED {submode1, submode2}		OPTIONAL	-- Need OR

},

subbandCQI-r10						SEQUENCE {

k								INTEGER (1..4),

periodicityFactor-r10				ENUMERATED {n2, n4}

}

},

ri-ConfigIndex					INTEGER (0..1023)		OPTIONAL,				-- Need OR

simultaneousAckNackAndCQI		BOOLEAN,

cqi-Mask-r9						ENUMERATED {setup}		OPTIONAL,				-- Need OR

csi-ConfigIndex-r10				CHOICE {

release							NULL,

setup							SEQUENCE {

cqi-pmi-ConfigIndex2-r10		INTEGER (0..1023),

ri-ConfigIndex2-r10				INTEGER (0..1023)		OPTIONAL		-- Need OR

}

}		OPTIONAL																-- Need ON

}

}

CQI-ReportPeriodic-v1130 ::=	SEQUENCE {

simultaneousAckNackAndCQI-Format3-r11		ENUMERATED {setup}		OPTIONAL,	-- Need OR

cqi-ReportPeriodicProcExtToReleaseList-r11	CQI-ReportPeriodicProcExtToReleaseList-r11	OPTIONAL,	-- Need ON

cqi-ReportPeriodicProcExtToAddModList-r11	CQI-ReportPeriodicProcExtToAddModList-r11	OPTIONAL	-- Need ON

}

CQI-ReportPeriodic-v1310 ::=	SEQUENCE {

cri-ReportConfig-r13			CRI-ReportConfig-r13				OPTIONAL,	-- Need OR

simultaneousAckNackAndCQI-Format4-Format5-r13		ENUMERATED {setup}		OPTIONAL-- Need OR

}

CQI-ReportPeriodic-v1320 ::=	SEQUENCE {

periodicityFactorWB-r13			ENUMERATED {n2, n4}			OPTIONAL		-- Need OR

}

CQI-ReportPeriodicProcExtToAddModList-r11 ::=		SEQUENCE (SIZE (1..maxCQI-ProcExt-r11)) OF CQI-ReportPeriodicProcExt-r11

CQI-ReportPeriodicProcExtToReleaseList-r11 ::=	SEQUENCE (SIZE (1..maxCQI-ProcExt-r11)) OF CQI-ReportPeriodicProcExtId-r11

CQI-ReportPeriodicProcExt-r11 ::=		SEQUENCE {

cqi-ReportPeriodicProcExtId-r11	CQI-ReportPeriodicProcExtId-r11,

cqi-pmi-ConfigIndex-r11			INTEGER (0..1023),

cqi-FormatIndicatorPeriodic-r11	CHOICE {

widebandCQI-r11				SEQUENCE {

csi-ReportMode-r11			ENUMERATED {submode1, submode2}	OPTIONAL	-- Need OR

},

subbandCQI-r11				SEQUENCE {

k							INTEGER (1..4),

periodicityFactor-r11		ENUMERATED {n2, n4}

}

},

ri-ConfigIndex-r11				INTEGER (0..1023)					OPTIONAL,	-- Need OR

csi-ConfigIndex-r11				CHOICE {

release							NULL,

setup							SEQUENCE {

cqi-pmi-ConfigIndex2-r11		INTEGER (0..1023),

ri-ConfigIndex2-r11				INTEGER (0..1023)		OPTIONAL		-- Need OR

}

}																OPTIONAL,		-- Need ON

...,

[[	cri-ReportConfig-r13			CRI-ReportConfig-r13				OPTIONAL	-- Need ON

]],

[[	periodicityFactorWB-r13			ENUMERATED {n2, n4}			OPTIONAL		-- Need ON

]]

}

CQI-ReportAperiodic-r10	::=		CHOICE {

release								NULL,

setup								SEQUENCE {

cqi-ReportModeAperiodic-r10			CQI-ReportModeAperiodic,

aperiodicCSI-Trigger-r10			SEQUENCE {

trigger1-r10					BIT STRING (SIZE (8)),

trigger2-r10					BIT STRING (SIZE (8))

}																	OPTIONAL	-- Need OR

}

}

CQI-ReportAperiodic-v1250	::=		CHOICE {

release								NULL,

setup								SEQUENCE {

aperiodicCSI-Trigger-v1250			SEQUENCE {

trigger-SubframeSetIndicator-r12	ENUMERATED {s1, s2},

trigger1-SubframeSetIndicator-r12	BIT STRING (SIZE (8)),

trigger2-SubframeSetIndicator-r12	BIT STRING (SIZE (8))

}

}

}

CQI-ReportAperiodic-v1310	::=		CHOICE {

release								NULL,

setup								SEQUENCE {

aperiodicCSI-Trigger-v1310			SEQUENCE {

trigger1-r13					BIT STRING (SIZE (32)),

trigger2-r13					BIT STRING (SIZE (32)),

trigger3-r13					BIT STRING (SIZE (32)),

trigger4-r13					BIT STRING (SIZE (32)),

trigger5-r13					BIT STRING (SIZE (32)),

trigger6-r13					BIT STRING (SIZE (32))

}																	OPTIONAL,	-- Need ON

aperiodicCSI-Trigger2-r13		CHOICE {

release							NULL,

setup						SEQUENCE {

trigger1-SubframeSetIndicator-r13	BIT STRING (SIZE (32)),

trigger2-SubframeSetIndicator-r13	BIT STRING (SIZE (32)),

trigger3-SubframeSetIndicator-r13	BIT STRING (SIZE (32)),

trigger4-SubframeSetIndicator-r13	BIT STRING (SIZE (32)),

trigger5-SubframeSetIndicator-r13	BIT STRING (SIZE (32)),

trigger6-SubframeSetIndicator-r13	BIT STRING (SIZE (32))

}

}																	OPTIONAL	-- Need ON

}

}

CQI-ReportAperiodicProc-r11	::=		SEQUENCE {

cqi-ReportModeAperiodic-r11			CQI-ReportModeAperiodic,

trigger01-r11						BOOLEAN,

trigger10-r11						BOOLEAN,

trigger11-r11						BOOLEAN

}

CQI-ReportAperiodicProc-v1310	::=		SEQUENCE {

trigger001-r13						BOOLEAN,

trigger010-r13						BOOLEAN,

trigger011-r13						BOOLEAN,

trigger100-r13						BOOLEAN,

trigger101-r13						BOOLEAN,

trigger110-r13						BOOLEAN,

trigger111-r13						BOOLEAN

}

CQI-ReportModeAperiodic ::=				ENUMERATED {

rm12, rm20, rm22, rm30, rm31,

rm32-v1250, rm10-v1310, rm11-v1310

}

CQI-ReportBoth-r11 ::=			SEQUENCE {

csi-IM-ConfigToReleaseList-r11		CSI-IM-ConfigToReleaseList-r11	OPTIONAL,	-- Need ON

csi-IM-ConfigToAddModList-r11		CSI-IM-ConfigToAddModList-r11	OPTIONAL,	-- Need ON

csi-ProcessToReleaseList-r11		CSI-ProcessToReleaseList-r11	OPTIONAL,	-- Need ON

csi-ProcessToAddModList-r11			CSI-ProcessToAddModList-r11		OPTIONAL	-- Need ON

}

CQI-ReportBoth-v1250 ::=			SEQUENCE {

csi-IM-ConfigToReleaseListExt-r12		CSI-IM-ConfigId-v1250	OPTIONAL,	-- Need ON

csi-IM-ConfigToAddModListExt-r12		CSI-IM-ConfigExt-r12	OPTIONAL	-- Need ON

}

CQI-ReportBoth-v1310 ::=			SEQUENCE {

csi-IM-ConfigToReleaseListExt-r13	CSI-IM-ConfigToReleaseListExt-r13	OPTIONAL,	-- Need ON

csi-IM-ConfigToAddModListExt-r13	CSI-IM-ConfigToAddModListExt-r13	OPTIONAL	-- Need ON

}

CSI-IM-ConfigToAddModList-r11 ::=		SEQUENCE (SIZE (1..maxCSI-IM-r11)) OF CSI-IM-Config-r11

CSI-IM-ConfigToAddModListExt-r13 ::=	SEQUENCE (SIZE (1..maxCSI-IM-v1310)) OF CSI-IM-ConfigExt-r12

CSI-IM-ConfigToReleaseList-r11 ::=		SEQUENCE (SIZE (1..maxCSI-IM-r11)) OF CSI-IM-ConfigId-r11

CSI-IM-ConfigToReleaseListExt-r13 ::=	SEQUENCE (SIZE (1..maxCSI-IM-v1310)) OF CSI-IM-ConfigId-v1310

CSI-ProcessToAddModList-r11 ::=		SEQUENCE (SIZE (1..maxCSI-Proc-r11)) OF CSI-Process-r11

CSI-ProcessToReleaseList-r11 ::=	SEQUENCE (SIZE (1..maxCSI-Proc-r11)) OF CSI-ProcessId-r11

CQI-ReportBothProc-r11 ::=			SEQUENCE {

ri-Ref-CSI-ProcessId-r11			CSI-ProcessId-r11				OPTIONAL,		-- Need OR

pmi-RI-Report-r11					ENUMERATED {setup}				OPTIONAL		-- Need OR

}

CRI-ReportConfig-r13 ::=			CHOICE {

release								NULL,

setup								SEQUENCE {

cri-ConfigIndex-r13					CRI-ConfigIndex-r13,

cri-ConfigIndex2-r13				CRI-ConfigIndex-r13	OPTIONAL	-- Need OR

}

}

CRI-ConfigIndex-r13 ::=				INTEGER (0..1023)

-- ASN1STOP

CQI-ReportConfig field descriptions

altCQI-Table Indicates the applicability of the alternative CQI table (i.e. Table 7.2.3-2 in TS 36.213 [23]) for both aperiodic and periodic CSI reporting for the concerned serving cell. Value allSubframes means the alternative CQI table applies to all the subframes and CSI processes, if configured, and value csi-SubframeSet1 means the alternative CQI table applies to CSI subframe set1, and value csi-SubframeSet2 means the alternative CQI table applies to CSI subframe set2. EUTRAN sets the value to csi-SubframeSet1 or csi-SubframeSet2 only if transmissionMode is set in range tm1 to tm9 and csi-SubframePatternConfig-r10 is configured for the concerned serving cell and different CQI tables apply to the two CSI subframe sets; otherwise EUTRAN sets the value to allSubframes. If this field is not present, the UE shall use Table 7.2.3-1 in TS 36.213 [23] for all subframes and CSI processes, if configured.

aperiodicCSI-Trigger Indicates for which serving cell(s) the aperiodic CSI report is triggered when one or more SCells are configured. trigger1-r10 corresponds to the CSI request field 10 while trigger1-r13 corresponds to the CSI request field 010, trigger2-r10 corresponds to the CSI request field 11 while trigger2-r13 corresponds to the CSI request field 011, trigger3-r13 corresponds to the CSI request field 100, see TS 36.213 [23, table 7.2.1-1A and table 7.2.1-1D], and so on. The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex=1 etc. Each bit has either value 0 (means no aperiodic CSI report is triggered) or value 1 (means the aperiodic CSI report is triggered). At most 5 bits can be set to value 1 in the bit string in aperidociCSI-Trigger-r10 and in aperiodicCSI-Trigger-v1250 and at most 32 bits can be set to value 1 in the bit string in aperiodicCSI-Trigger-v1310. E-UTRAN configures value 1 only for cells configured with transmissionMode set in range tm1 to tm9. One value applies for all serving cells configured with transmissionMode set in range tm1 to tm9 and belonging to the same PUCCH group (the associated functionality is common i.e. not performed independently for each cell).

cqi-Mask Limits CQI/PMI/PTI/RI reports to the on-duration period of the DRX cycle, see TS 36.321 [6]. One value applies for all CSI processes and all serving cells (the associated functionality is common i.e. not performed independently for each cell).

cqi-FormatIndicatorPeriodic Parameter: PUCCH CQI Feedback Type, see TS 36.213 [23, table 7.2.2-1]. Depending on transmissionMode, reporting mode is implicitly given from the table.

cqi-pmi-ConfigIndex Parameter: CQI/PMI Periodicity and Offset Configuration Index ICQI/PMI, see TS 36.213 [23, tables 7.2.2-1A and 7.2.2-1C]. If subframe patterns for CSI (CQI/PMI/PTI/RI) reporting are configured (i.e. csi-SubframePatternConfig is configured), the parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet1 or corresponding to the CSI subframe set 1 indicated by csi-MeasSubframeSets-r12.

cqi-pmi-ConfigIndex2 Parameter: CQI/PMI Periodicity and Offset Configuration Index ICQI/PMI, see TS 36.213 [23, tables 7.2.2-1A and 7.2.2-1C]. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet2 or corresponding to the CSI subframe set 2 indicated by csi-MeasSubframeSets-r12.

cqi-PUCCH-ResourceIndex, cqi-PUCCH-ResourceIndexP1 Parameter for antenna port P0 and for antenna port P1 respectively, see TS 36.213 [23, 7.2]. E-UTRAN does not apply value 1185. One value applies for all CSI processes.

cqi-ReportAperiodic E-UTRAN does not configure CQI-ReportAperiodic when transmission mode 10 is configured for all serving cells. E-UTRAN configures cqi-ReportAperiodic-v1250 only if cqi-ReportAperiodic-r10 and csi-MeasSubframeSets-r12 are configured. E-UTRAN configures cqi-ReportAperiodic-v1310 only if cqi-ReportAperiodic-r10 is configured.

cqi-ReportModeAperiodic Parameter: reporting mode. Value rm12 corresponds to Mode 1-2, rm20 corresponds to Mode 2-0, rm22 corresponds to Mode 2-2 etc. PUSCH reporting modes are described in TS 36.213 [23, 7.2.1]. The UE shall ignore cqi-ReportModeAperiodic-r10 when transmission mode 10 is configured for the serving cell on this carrier frequency. The UE shall ignore cqi-ReportModeAperiodic-r10 configured for the PCell/ PSCell when the transmission bandwidth of the PCell/PSCell in downlink is 6 resource blocks.

CQI-ReportPeriodicProcExt A set of periodic CQI related parameters for which E-UTRAN may configure different values for each CSI process. For a serving frequency E-UTRAN configures one or more CQI-ReportPeriodicProcExt only when transmission mode 10 is configured for the serving cell on this carrier frequency.

cri-ConfigIndex Parameter: cri-ConfigIndex ICRIsee TS 36.213 [23]. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet1. EUTRAN configures the field if subframe patterns for CSI (CQI/PMI/PTI/RI/CRI) reporting are configured (i.e. csi-SubframePatternConfig is configured).

cri-ConfigIndex2 Parameter: cri-ConfigIndex ICRIsee TS 36.213 [23]. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet2 or corresponding to the CSI subframe set 2 indicated by csi-MeasSubframeSets. E-UTRAN configures cri-ConfigIndex2 only if cri-ConfigIndex is configured.

cri-ReportConfig E-UTRAN configures the field only if the UE is configured with eMIMO-Type set to "beamformed" and if multiple references to RS configuration using non-zero power transmission are configured (i.e. if csi-RS-ConfigNZPIdListExt is configured).

csi-ConfigIndex E-UTRAN configures csi-ConfigIndex only for PCell and only if csi-SubframePatternConfig is configured. The UE shall release csi-ConfigIndex if csi-SubframePatternConfig is released.

csi-IM-ConfigToAddModList For a serving frequency E-UTRAN configures one or more CSI-IM-Config only when transmission mode 10 is configured for the serving cell on this carrier frequency.

csi-MeasSubframeSets Indicates the two CSI subframe sets. Value 0 means the subframe belongs to CSI subframe set 1 and value 1 means the subframe belongs to CSI subframe set 2. CSI subframe set 1 refers to CCSI,0 in TS 36.213 [23, 7.2] and CSI subframe set 2 refers to CCSI,1 in TS 36.213 [23, 7.2]. EUTRAN does not configure csi-MeasSubframeSet1-r10 and csi-MeasSubframeSet2-r10 if either csi-MeasSubframeSets-r12 for PCell or eimta-MainConfigPCell-r12 is configured.

csi-MeasSubframeSet1, csi-MeasSubframeSet2 Indicates the CSI measurement subframe sets. csi-MeasSubframeSet1 refers to CCSI,0 in TS 36.213 [23, 7.2] and csi-MeasSubframeSet2 refers to CCSI,1 in TS 36.213 [23, 7.2]. E-UTRAN only configures the two CSI measurement subframe sets for the PCell.

csi-ProcessToAddModList For a serving frequency E-UTRAN configures one or more CSI-Process only when transmission mode 10 is configured for the serving cell on this carrier frequency.

csi-ReportMode Parameter: PUCCH_format1-1_CSI_reporting_mode, see TS 36.213 [23, 7.2.2].

K Parameter: K, see TS 36.213 [23, 7.2.2].

nomPDSCH-RS-EPRE-Offset Parameter: see TS 36.213 [23, 7.2.3]. Actual value = field value * 2 [dB].

periodicityFactor, periodicityFactorWB Parameter: , see TS 36.213 [23, 7.2.2]. EUTRAN configures field periodicityFactorWB only when the UE is configured with eMIMO-Type set to nonPrecoded and with cqi-FormatIndicatorPeriodic set to widebandCQI.

pmi-RI-Report See TS 36.213 [23, 7.2]. The presence of this field means PMI/RI reporting is configured; otherwise the PMI/RI reporting is not configured. EUTRAN configures this field only when transmissionMode is set to tm8, tm9 or tm10. The UE shall ignore pmi-RI-Report-r9/ pmi-RI-Report-r10 when transmission mode 10 is configured for the serving cell on this carrier frequency.

ri-ConfigIndex Parameter: RI Config Index IRI, see TS 36.213 [23, 7.2.2-1B]. If subframe patterns for CSI (CQI/PMI/PTI/RI/CRI) reporting are configured (i.e. csi-SubframePatternConfig is configured), the parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet1.

ri-ConfigIndex2 Parameter: RI Config Index IRI, see TS 36.213 [23, 7.2.2-1B]. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet2 or corresponding to the CSI subframe set 2 indicated by csi-MeasSubframeSets-r12. E-UTRAN configures ri-ConfigIndex2 only if ri-ConfigIndex is configured.

ri-Ref-CSI-ProcessId CSI process whose RI value the UE inherits when reporting RI, in the same subframe, for CSI reporting. E-UTRAN ensures that the CSI process that inherits the RI value is configured in accordance with the conditions specified in TS 36.213 [23, 7.2.1, 7.2.2].

simultaneousAckNackAndCQI Parameter: Simultaneous-AN-and-CQI, see TS 36.213 [23, 10.1]. TRUE indicates that simultaneous transmission of ACK/NACK and CQI is allowed. One value applies for all CSI processes. For SCells except for the PSCell and PUCCH SCell this field is not applicable and the UE shall ignore the value.

simultaneousAckNackAndCQI-Format3 Indicates that the UE shall perform simultaneous transmission of HARQ A/N and periodic CQI report multiplexing on PUCCH format 3, see TS 36.213 [23, 7.2, 10.1.1]. E-UTRAN configures this information only when pucch-Format is set to format3. One value applies for all CSI processes. For SCells except for the PSCell and PUCCH SCell this field is not applicable and the UE shall ignore the value.

simultaneousAckNackAndCQI-Format4-Format5 Indicates that the UE shall perform simultaneous transmission of HARQ A/N and periodic CSI report multiplexing on PUCCH format 4 and format 5, see TS 36.213 [23, 10.1.1]. E-UTRAN configures this information only when pucch-Format is set to format4 or format5. One value applies for all CSI processes. For SCells except for the PSCell and PUCCH SCell this field is not applicable and the UE shall ignore the value.

trigger01 Indicates whether or not reporting for this CSI-process or reporting for this CSI-process corresponding to a CSI subframe set is triggered by CSI request field set to 01, for a CSI request applicable for the serving cell on the same frequency as the CSI process, see TS 36.213 [23, table 7.2.1-1D and 7.2.1-1E].

trigger10, trigger11 Indicates whether or not reporting for this CSI-process or reporting for this CSI-process corresponding to a CSI subframe set is triggered by CSI request field set to 10 or 11, see TS 36.213 [23, table 7.2.1-1B]. EUTRAN configures at most 5 CSI processes, across all serving frequencies within each CG, to be triggered by a CSI request field set to value 10. The same restriction applies for value 11. In case E-UTRAN simultaneously triggers CSI requests for more than 5 CSI processes some limitations apply, see TS 36.213 [23].

trigger001 Indicates whether or not reporting for this CSI-process or reporting for this CSI-process corresponding to a CSI subframe set is triggered by CSI request field set to 001, for a CSI request applicable for the serving cell on the same frequency as the CSI process, see TS 36.213 [23, table 7.2.1-1D and 7.2.1-E].

trigger010, trigger011, trigger100, trigger101, Trigger110, Trigger111 Indicates whether or not reporting for this CSI-process or reporting for this CSI-process corresponding to a CSI subframe set is triggered by CSI request field set to 010, 011, 100, 101, 110 or 111, see TS 36.213 [23, table 7.2.1-1D and 7.2.1-1E].

trigger-SubframeSetIndicator For a serving cell configured with csi-MeasSubframeSets-r12, indicates for which CSI subframe set the aperiodic CSI report is triggered for the serving cell if the aperiodic CSI is triggered by the CSI request field 01 or 001, see TS 36.213 [23, table 7.2.1-1C or table 7.2.1.-1E]. Value s1 corresponds to CSI subframe set 1 and value s2 corresponds to CSI subframe set 2.

trigger1-SubframeSetIndicator If signalled in the aperiodicCSI-Trigger-v1250, indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field 10, see TS 36.213 [23, table 7.2.1-1C] or by the CSI request field 010, see TS 36.213 [23, table 7.2.1-1E].The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex=1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

trigger2-SubframeSetIndicator If signalled in the aperiodicCSI-Trigger-v1250, indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field 11, see TS 36.213 [23, table 7.2.1-1C] or by the CSI request field 011, see TS 36.213 [23, table 7.2.1-1E].The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex=1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

trigger3-SubframeSetIndicator Indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field100, see TS 36.213 [23, table 7.2.1-1E].The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex =1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

trigger4-SubframeSetIndicator Indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field 101, see TS 36.213 [23, table 7.2.1-1E].The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex =1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

trigger5-SubframeSetIndicator Indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field 110, see TS 36.213 [23, table 7.2.1-1E].The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex =1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

trigger6-SubframeSetIndicator Indicates for which CSI subframe set the aperiodic CSI report is triggered when aperiodic CSI is triggered by the CSI request field 111, see TS 36.213 [23, table 7.2.1-1E].The leftmost bit, bit 0 in the bit string corresponds to the cell with ServCellIndex=0 and bit 1 in the bit string corresponds to the cell with ServCellIndex =1 etc. Each bit has either value 0 (means that aperiodic CSI report is triggered for CSI subframe set 1) or value 1 (means that aperiodic CSI report is triggered for CSI subframe set 2).

<End of Change>