3GPP TSG RAN WG2 Meeting #107 	R2-1908844

Prage,CZ, 26-30 Aug 2019

Title:	UE positioning in legacy satellite communication systems

Source: 	Thales

Type:	Discussion

Document for:	Discussion

Agenda Item:	11.6.4.2

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document introduces UE positioning in legacy satellite communication systems.

# UE positioning in NTN

In legacy satellite communication, the objective of UE network positioning is to determine the geographical position of the UE based on time and frequency measurements on the User Link. UE positioning will be provided with a maximum location error (typically < 10km) and a maximum location determination time (typical values < 30 seconds for 90% of the users with 90% cofindence).

Use cases

There are two use cases on which UE positioning is used:

- Geo-location for access control

  - Required to manage access requests

  - To access the network, information used for geo-location are carried in the access request sent by the UE to the satellite, and propagated to the home gateway. UE geo-location is estimated by the network. On the basis of this geo-location, the network grants or not the access to satellite services based on the UE location, service provider and requested service. The network returns the access decision notification to the UE via the satellite.

- Geo-location data to user service

  - Information used for geo-location data is sent to the gateway and processed to estimate SE geo-location in the same way as for Access Control. Geo-location information is then provided to the UE by the Gateway.

Service architecture

The geo-location service is supported by the network on the basis of information provided by the UE, the satellite and Satellite Control System:

- DTOA (Differential Time of Arrival) and DFOA (Differential Frequency of Arrival) are measured on the user link bursts between the UE and the satellite.

- Propagation delays and frequency shifts (along with tie and beam information) are sent to the network by the satellite to estimate the position of the UE.

- Updated satellite ephemeris are also regularly sent to the network by the Satellite Control System.

- The network geo-location algorithm returns a location along with an error probability which if too large, will trigger the network to request additional measurements from the UE.

Figure 1 Legacy satellite communication systems geo-location architecture

Observation 1: legacy satellite communication systems provide geo-location procedures as part of the access and registration process. This process is based on propagation delay measurements, frequency shifts, beam information and satellite ephemeris.

Proposal 1: RAN2 is kindly asked to analyse how NTN UE network positioning for 5G can be performed as an evolution from the solutions supported in legacy satellite communication systems.

# Conclusions

Observation 1: legacy satellite communication systems provide geo-location procedures as part of the access and registration process. This process is based on propagation delay measurements, frequency shifts, beam information and satellite ephemeris.

Proposal 1: RAN2 is kindly asked to analyse how NTN UE network positioning for 5G can be performed as an evolution from the solutions supported in legacy satellite communication systems.