3GPP TSG-RAN WG2 Meeting #107	R2-1908949

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	12.3.3.3

Source: 	MediaTek Inc.

Title:  	RRC Configurations Related to Conditional Handover in LTE

Document for:	Discussion and decision

Introduction

In RAN2#106 meeting, we made some further progress about conditional handover (CHO) for handover robustness improvement in LTE and NR. Agreements that are applicable to both LTE and NR are as follows.

Agreements 1 Separate CHO execution condition(s) can be configured for each individual candidate cells. 2 Define a CHO execution condition by the measurement identity which identifies a measurement configuration. (FFS to be addressed in stage 3 which parts of the measurement configuration are used for the CHO triggering). 3 As a baseline CHO can be triggered based on a condition consisting of a single event, singe quantity. 3.1 The single trigger quantity can be configured to be RSRP, RSRQ or RS-SINR. FFS Whether multiple triggering conditions are required. 4 Deconfiguration of CHO candidates is performed by RRC signalling (we will not introduce timer based mechanism for the UE to deconfiguration of the CHO candidates). 5 Baseline that configuration of all CHO candidates are released after successful (any) handover completion (sending complete message to the target cell). FFS if it might be possible to keep CHO candidates after the HO. 6 UE shall not stop T310 and shall not start T304 when it receives configuration of a CHO candidate 7 The timer T310 is stopped and timer T304-like is started when the UE begins execution of a conditional handover for a target cell. (Stage 3 detail whether we reuse T304 or define a new timer) Working assumption: 8 At RLF the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed. 9 At legacy handover failure (T304 expiry) or failure to access a CHO candidate cell (T304-like expiry), the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed.

In this contribution, we discuss RRC configurations related to CHO in LTE, based on the agreements made in previous meetings.

Discussion

CHO candidate configuration

A UE supporting CHO should keep a list of CHO candidate cells. The network may add new CHO candidate cells or modify existing CHO candidate cells by sending a list for addition and modification. The network may also release one or more CHO candidate cells by sending a list for release. These two lists should be included in the CHO command (e.g. MobilityControlInfoCHO), which is carried by RRCConnectionReconfiguration.

Proposal 1:	The CHO command carries a list for CHO candidate cell addition/ modification, and a list for CHO candidate cell release.

In the following, we discuss the configuration for each CHO candidate cell. Some configurations are applied before CHO execution, while others are applied after CHO execution.

Configurations applied before CHO execution

For each CHO candidate cell, the following information should be configured:

- A candidate index: To identify a cell for future modification or release, an index should be assigned to a CHO candidate cell upon configuration.

- Cell identification information: physical cell ID, carrier frequency, etc.

- T3xx timer: A T304-like timer for CHO; CHO is considered failed if T3xx timer expires.

- CHO execution condition(s): UE periodically evaluates the conditions for each candidate cell, and CHO can be executed once the conditions are met. Notice that the network may configure multiple conditions for a candidate cell, in this case the conditions are configured as a list.

Proposal 2:	For each CHO candidate cell, the following information should be configured: a candidate index, cell identification information, a T3xx timer for CHO failure indication, and CHO execution condition(s).

Configurations applied after CHO execution

In normal HO, the handover command (MobilityControlInfo) carries common radio resource configurations (radioResourceConfigCommon) as well as RACH-related configurations. In addition, other configurations may also be included in the same RRCConnectionReconfiguration message, and UE applies these configurations in the target cell.

In CHO, however, the configurations in the same RRCConnectionReconfiguration but outside the CHO command (MobilityControlInfoCHO) are applied to source cell, since the CHO is not yet executed when these configurations are received. In other words, they are irrelevant to CHO. But the network may also want to send configurations that are applicable once the UE is in target cell. Such configurations can be carried inside the CHO command. We think the following configurations may be included in CHO command:

- fullConfig: An indication for full configuration

- radioResourceConfigDedicated: Dedicated configurations on top of common configurations

- measConfig: Measurement configurations

Among the abovementioned IEs in CHO configurations, common radio resource configurations and security configurations should be mandatory present, while others can be optional present.

Proposal 3:	For each CHO candidate cell, the following information should be configured as mandatory present, and applied once the candidate cell is chosen as target cell for CHO execution: common radio resource configurations.

Proposal 4:	For each CHO candidate cell, the following information can be optionally configured, and applied once the candidate cell is chosen as target cell for CHO execution: a full configuration indicator, dedicated radio resource configurations and measurement configurations.

Example ASN.1 structure

Base on the above discussions, here we provide an example

MobilityControlInfoCho information element

-- ASN1START

MobilityControlInfoCho ::=		SEQUENCE {

cho-CandCellToReleaseList				Cho-CandCellToReleaseList	OPTIONAL,	-- Need ON

cho-CandCellToAddModList				Cho-CandCellToAddModList	OPTIONAL,	-- Need ON

}

CHO-CandCellToAddModList ::=		SEQUENCE (SIZE (1..maxCHO-Cand)) OF CHO-CandCellToAddMod

CHO-CandCellToAddMod ::=		SEQUENCE {

cho-CandCellIndex 				Cho-CandCellIndex,

cellIdentification	{

candPhysCellId					PhysCellId,

carrierFreq						CarrierFreqEUTRA					OPTIONAL,	-- Need ON

carrierBandwidth				CarrierBandwidthEUTRA				OPTIONAL,	-- Need ON

additionalSpectrumEmission		AdditionalSpectrumEmission			OPTIONAL	-- Need ON

}							OPTIONAL, -- Cond cho-CandCellAdd

t3xx								ENUMERATED {TBD},

newUE-Identity						C-RNTI,

radioResourceConfigCommon			RadioResourceConfigCommon,

radioResourceConfigDedicated		RadioResourceConfigDedicated		OPTIONAL, 	-- Need ON

measConfig							MeasConfig							OPTIONAL,	-- Need ON

rach-ConfigDedicated				RACH-ConfigDedicated				OPTIONAL,	-- Need OP

cho-ExecCondList					SEQUENCE (SIZE (1..maxCho-ExecCond)) OF ReportConfig

...

}

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1:	The CHO command carries a list for CHO candidate cell addition/ modification, and a list for CHO candidate cell release.

Proposal 2:	For each CHO candidate cell, the following information should be configured: a candidate index, cell identification information, a T3xx timer for CHO failure indication, and CHO execution condition(s).

Proposal 3:	For each CHO candidate cell, the following information should be configured as mandatory present, and applied once the candidate cell is chosen as target cell for CHO execution: common radio resource configurations.

Proposal 4:	For each CHO candidate cell, the following information can be optionally configured, and applied once the candidate cell is chosen as target cell for CHO execution: a full configuration indicator, dedicated radio resource configurations and measurement configurations.

Reference

- RP-181475, New Work Item on even further Mobility enhancement in E-UTRAN	China Telecom