3GPP TSG-RAN-WG2 Meeting #107	R2-1909393

# Prague, Czech Republic, 26th - 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1147 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security requirement for UE capability enquiry for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation, NTT DoCoMo, Ericsson, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-06-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | GSMA recommended in R2-1908112 and SA3 LS R2-1908668 that network should apply security for UE capability enquiry to prevent man-in-the-middle attack to e.g., downgrade UE AS capability or to snoop on the UE capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a recommendation that network should apply security for UE capability enquiry. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If network does not apply security for UE capability enquiry, it exposes the UE capabilities to tampering by a man-in-the-middle attacker on the radio interface, which can result in degradation of service e.g. downgrading the UE’s maximum throughput. Since the UE capabilities are persistently stored in the network, the impact of the attack can last for weeks, or until the UE is power cycled. Such attacks can have a particularly high impact on unattended IoT devices. It can also be used to trace UEs based on UE capability. Impact analysis: Impacted architecture: SA, NE-DC, NR-DC Impacted functionality: connection establishment, SMC, UE capability enquiry. There is no interoperability issue. The CR only impacts network behaviour and has no UE impact. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.1, 5.6.1, B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First change ****

## 5.3	Connection control

### 5.3.1	Introduction

#### 5.3.1.1	RRC connection control

RRC connection establishment involves the establishment of SRB1. The network completes RRC connection establishment prior to completing the establishment of the NG connection, i.e. prior to receiving the UE context information from the 5GC. Consequently, AS security is not activated during the initial phase of the RRC connection. During this initial phase of the RRC connection, the network may configure the UE to perform measurement reporting, but the UE only sends the corresponding measurement reports after successful AS security activation. However, the UE only accepts a re-configuration with sync message when AS security has been activated.  Network should perform UECapabilityEnquiry procedure only after AS security activation.

Upon receiving the UE context from the 5GC, the RAN activates AS security (both ciphering and integrity protection) using the initial AS security activation procedure. The RRC messages to activate AS security (command and successful response) are integrity protected, while ciphering is started only after completion of the procedure. That is, the response to the message used to activate AS security is not ciphered, while the subsequent messages (e.g. used to establish SRB2 and DRBs) are both integrity protected and ciphered. After having initiated the initial AS security activation procedure, the network may initiate the establishment of SRB2 and DRBs, i.e. the network may do this prior to receiving the confirmation of the initial AS security activation from the UE. In any case, the network will apply both ciphering and integrity protection for the RRC reconfiguration messages used to establish SRB2 and DRBs. The network should release the RRC connection if the initial AS security activation and/ or the radio bearer establishment fails.

The release of the RRC connection normally is initiated by the network. The procedure may be used to re-direct the UE to an NR frequency or an E-UTRA carrier frequency.

The suspension of the RRC connection is initiated by the network. When the RRC connection is suspended, the UE stores the UE Inactive AS context and any configuration received from the network, and transits to RRC_INACTIVE state. If the UE is configured with SCG, the UE releases the SCG configuration upon initiating a RRC Connection Resume procedure. The RRC message to suspend the RRC connection is integrity protected and ciphered.

The resumption of a suspended RRC connection is initiated by upper layers when the UE needs to transit from RRC_INACTIVE state to RRC_CONNECTED state or by RRC layer to perform a RNA update or by RAN paging from NG-RAN. When the RRC connection is resumed, network configures the UE according to the RRC connection resume procedure based on the stored UE Inactive AS context and any RRC configuration received from the network. The RRC connection resume procedure re-activates AS security and re-establishes SRB(s) and DRB(s).

In response to a request to resume the RRC connection, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to initiate NAS level recovery (in this case the network sends an RRC setup message).

**** Next Change ****

### 5.6.1	UE capability transfer

#### 5.6.1.1	General

This clause describes how the UE compiles and transfers its UE capability information upon receiving a UECapabilityEnquiry from the network.

Figure 5.6.1.1-1: UE capability transfer

#### 5.6.1.2	Initiation

The network initiates the procedure to a UE in RRC_CONNECTED when it needs (additional) UE radio access capability information.  Network should perform UECapabilityEnquiry procedure only after AS security activation. If Network had to retrieve the capability before AS security, it does not send them to other network entities and should re-run the UECapabilityEnquiry procedure after AS security activation.

**** Next Change ****

Annex B (informative):	RRC Information

# B.1	Protection of RRC messages

The following list provides information which messages can be sent (unprotected) prior to AS security activation and which messages can be sent unprotected after AS security activation. Those messages indicated "-" in "P" column should never be sent unprotected by gNB or UE. Further requirements are defined in the procedural text.

P…Messages that can be sent (unprotected) prior to AS security activation

A – I…Messages that can be sent without integrity protection after AS security activation

A – C…Messages that can be sent unciphered after AS security activation

NA… Message can never be sent after AS security activation

| Message | P | A-I | A-C | Comment |
| --- | --- | --- | --- | --- |
| CounterCheck | - | - | - |  |
| CounterCheckResponse | - | - | - |  |
| DLInformationTransfer | + | - | - |  |
| FailureInformation | - | - | - |  |
| LocationMeasurementIndication | - | - | - |  |
| MIB | + | + | + |  |
| MeasurementReport | - | - | - | Measurement configuration may be sent prior to AS security activation. But: In order to protect privacy of UEs, MeasurementReport is only sent from the UE after successful AS security activation. |
| MobilityFromNRCommand | - | - | - |  |
| Paging | + | + | + |  |
| RRCReconfiguration | + | - | - | The message shall not be sent unprotected before AS security activation if it is used to perform handover or to establish SRB2 and DRBs. |
| RRCReconfigurationComplete | + | - | - | Unprotected, if sent as response to RRCReconfiguration which was sent before AS security activation. |
| RRCReestablishment | - | - | + | Integrity protection applied, but no ciphering. |
| RRCReestablishmentComplete | - | - | - |  |
| RRCReestablishmentRequest | - | - | + | This message is not protected by PDCP operation. However, a shortMAC-I is included. |
| RRCReject | + | + | + | Justification for A-I and A-C: the message can be sent in SRB0 in RRC_INACTIVE state, after the AS security is activated. |
| RRCRelease | + | - | - | Justification for P: If the RRC connection only for signalling not requiring DRBs or ciphered messages, or the signalling connection has to be released prematurely, this message is sent as unprotected. RRCRelease message sent before AS security activation cannot include deprioritisationReq, suspendConfig, redirectedCarrierInfo, cellReselectionPriorities information fields. |
| RRCRequest | + | NA | NA |  |
| RRCResume | - | - | - |  |
| RRCResumeComplete | - | - | - |  |
| RRCResumeRequest | - | - | + | This message is not protected by PDCP operation. However, a resumeMAC-I is included. |
| RRCResumeRequest1 | - | - | + | This message is not protected by PDCP operation. However, a resumeMAC-I is included. |
| RRCSetup | + | + | + | Justification for A-I and A-C: the message can be sent in SRB0 in RRC_INACTIVE state, after the AS security is activated. |
| RRCSetupComplete | + | NA | NA |  |
| RRCSystemInfoRequest | + | + | + | Justification for A-I and A-C: the message can be sent in SRB0 in RRC_INACTIVE state, after the AS security is activated. |
| SIB1 | + | + | + |  |
| SCGFailureInformation | - | - | - |  |
| SCGFailureInformationEUTRA | - | - | - |  |
| SecurityModeCommand | + | NA | NA | Integrity protection applied, but no ciphering (integrity verification done after the message received by RRC). |
| SecurityModeComplete | - | - | + | The message is sent after AS security activation. Integrity protection applied, but no ciphering. Ciphering is applied after completing the procedure. |
| SecurityModeFailure | + | NA | NA | Neither integrity protection nor ciphering applied. |
| SystemInformation | + | + | + |  |
| UEAssistanceInformation | - | - | - |  |
| UECapabilityEnquiry | + | - | - | Network should perform UECapabilityEnquiry procedure only after AS security activation. If Network had to retrieve the capability before AS security, it does not send them to other network entities and should re-run the UECapabilityEnquiry procedure after AS security activation. |
| UECapabilityInformation | + | - | - |  |
| ULInformationTransfer | + | - | - |  |
| ULInformationTransferMRDC | - | - | - |  |