3GPP TSG-RAN WG2 Meeting #107												R2-1909776

Prague, Czech Republic, 26th – 30th Aug, 2019

Source: 	vivo

Title:  	Discussion on RLM RLF and HOF for non-DC-based solution

Agenda Item:	12.3.2.5

Document for:	Discussion and Decision

# Introduction

According to the LTE mobility WI discussion in the RAN2#106 meeting, RAN2 agreed to introduce the NON-DC-BASED (Dual Active Protocol Stack) solution. According to the NR mobility WI discussion in the RAN2#106 meeting, RAN2 has two solutions (i.e. one is DC-based HO, and another is non-DC-based HO) on the table. As the non-DC-based solution requires two active connections (i.e. one for the source node, and another for the target node), RAN2 needs to consider how the RLF/HOF is handled during the handover procedure. In this contribution, we focus on the handling of RLF/HOF of the non-DC-based solution, and we consider that the handling of RLF/HOF of the non-DC-based solution may potentially be also applied to the DC-based HO. The corresponding agreements from the RAN2#106 meeting are quoted as follows:

RAN2#106 meeting agreements for LTE mobility WI: - Simultaneous UL PUSCH transmission does not need to be supported for the HO interruption solution. - UL PUSCH switches from source to target after reception of the first UL grant from the target eNB. - We will not specify single active protocol stack solution (option 0/1/2). - We will specify dual active with specified capability coordination that does not have to be utilized by the network. FFS how/whether we will specify the rules for UE when capability coordination is not utilized and UE capabilities are exceeded (we may leave this up to UE implementation).

RAN2#106 meeting agreements for NR mobility WI: - PDCP packet duplication does not need to be supported in combination with the HO interruption solution (but doesn't preclude that it might be possible to support it and it may be beneficial in some cases). - Simultaneous UL PUSCH transmission does not need to be supported for the HO interruption solution. - There is a point in time where the UL PUSCH switches from source to target.

# Discussion

## Stages of the non-DC-based handover

Here we consider that the non-DC-based handover procedure can be divided into the following stages as illustrated in Figure 1.

Figure 1: non-DC-based handover stages

Stage 1: After the reception of the handover command, the UE keeps the source connection and starts the cell searching and the RACH procedure to the target cell.

Stage 2: After the successful RACH and before releasing the source connection, the UE keeps data reception via two connections.

Stage 3: After releasing the source connection, the UE only keeps the target cell connection.

## Handover failure and radio connection failure during non-DC-based handover

As a comparison, the failure detection of the legacy handover is given in the Annex, and we have the following observations for the legacy handover procedure:

Observation 1: In the legacy handover, the UE starts RLM immediately on the target PCell when the UE is downlink synchronized to the target PCell, but does not start T310 on the target PCell during the handover (i.e. when T304 is running).

Observation 2: In the legacy handover, when the UE receives the random access problem indication during the handover (i.e. when T304 is running), the UE does not trigger RLF.

Observation 3: In the legacy handover, there is no case that the UE receives the RLC status report from the target cell of the SRB during the handover (i.e. when T304 is running).

### Handover failure detection

From our understanding, to determine the handover failure or RLF, we need firstly decide when to stop the T304, i.e. how to determine the successful handover. Then we can discuss the detailed UE behaviors when the T304 is running or stopped. Firstly we would like to ask RAN2 to confirm that the T304 starts immediately when the UE receives the handover command, and the expiry of the T304 is used to determine the failure of the handover.

Proposal 1: The expiry of the T304 which starts at the execution of the non-DC-based handover command is used to determine the handover failure, as the legacy handover.

Proposal 2: The T304 of the non-DC-based handover is stopped when the MAC successfully completes the RACH to the target cell, as the legacy handover.

According to Proposal 1 and 2, as the T304 is stopped only after the successful RACH to the target cell, the source connection is still alive at the time when the RACH to the target cell succeeds. Thus at the expiry of the T304, as the source connection is not released, we consider that the UE can report the handover failure to the network via the source connection, instead of triggering the RRC connection re-establishment procedure. However if the handover failure and the RLF of the source connection happens at the same time, it is not possible to report the handover failure via the source connection. Then the UE should trigger the RRC connection re-establishment in the case.

Proposal 3: The UE reports the handover failure via the source connection at the expiry of T304 if the UE does not detect the source connection RLF.

### Source connection RLF

For the source connection during the handover procedure, we consider that the legacy RLF detection procedure can still be kept. This means that the RRC layer of UE can still start the T310 of the source PCell and receive the RACH problem and the RLC problem indication from the layer 2. However the UE should not trigger re-establishment but reports the source connection failure to the target cell if the source connection detects the radio connection failure. And the reporting of the source connection RLF via the target link can be applicable only if the target connection does not declare the RLF or HOF. If the target connection RLF/HOF and the source connection RLF happens at the same time, the UE should trigger the RRC connection re-establishment.

Proposal 4: The RLF detection (i.e. physical layer problem detection, RACH failure detection and RLC re-transmission failure detection) of the source connection is kept.

Proposal 5: If the UE detects the source connection RLF, the UE reports the source connection RLF to the target cell if the target connection does not declare the RLF/HOF.

Proposal 6: If the UE detects the source connection RLF and the target connection RLF/HOF at the same time, the UE triggers the RRC connection re-establishment.

### Target connection RLF

For the target connection RLF detection, we consider that the UE behaviors can be divided in to 3 stages as given in Section 2.1.

Stage 1: when the T304 is running.

Regarding the target cell RLM, we consider that the PHY layer of the UE should start the RLM immediately after downlink synchronized to the target cell as the legacy handover. The RLM COUNTER can still count the number of IS and OOS indication(s) from the PHY, and the UE just prohibits the starting of the T310 of the target PCell as the legacy handover, in order to avoid the re-establishment caused by the T310 expiry.

Proposal 7: The PHY layer of the UE starts the target PCell RLM immediately after the reception of the handover command, as the legacy handover.

Proposal 8: The UE does not start the T310 of the target PCell when T304 is running, as the legacy handover.

Regarding the RACH failure (i.e. the RRC receives the random access problem indication from the MAC) to the target cell, we consider that the UE should keep RACH procedure to the target cell before the handover failure (i.e. the expiry of the T304), so as to allow the UE to have more opportunities to access the target cell.

Proposal 9: The UE does not trigger re-establishment when the RRC receives the random access problem indication from the target MAC when T304 is running, as the legacy handover.

Regarding the RLC failure (i.e. RLC AM mode reaches the maximum number of re-transmission.), we consider that while the T304 is running, no RLC status report can be received from the target cell.

Stage 2: when T304 is stopped and the source connection is not released.

After the successful RACH (i.e. Stage 2), the RLF detection of the target connection should be the same as the legacy UE, but the UE will have two connection to both the source cell and the target cell. Here we consider that if only one connection fails (i.e. PHY/RLC/RACH failure), the UE does not trigger RRC connection re-establishment, but reports the failure information via another connection. If both connection fails, the UE triggers the RLF (i.e. RRC connection re-establishment), as given in Proposal 6.

Proposal 10: When the T304 is not running, the RLF detection of the target connection is the same as the legacy UE.

Proposal 11: Before releasing the source connection, the UE reports the target connection RLF via the source connection when the UE detects the target connection RLF.

Stage 3: when T304 is stopped and the source connection is released.

The UE behavior regarding the RLF should be the same as the legacy UE.

## Control plane of the non-DC-based handover

According to the analysis given above, the non-DC-based solution would make the handover procedure more robust than before as only the case that both the target connection and the source connection fails the UE would trigger re-establishment, and the UE can keep the data reception even though one link fails. However this is based on the assumption that the UE can report the failure of one link via another link. Then during the non-DC-based handover (i.e. before releasing the source connection), the UE should have SRBs for both the source connection and the target connection, in order to allow the connection failure reporting via either the source link or the target link.

Observation 4: Keeping SRB1 during the non-DC-based handover helps reducing the impacts caused by the HOF and the target connection failure.

Proposal 12: The SRB1 of the source connection is kept before releasing the source connection.

Proposal 13: The SRB1 of the target connection is established at the execution of the non-DC-based handover command, as the legacy handover.

# Conclusions

Based on the analysis given above, we have the following Observations and Proposals：

Observation 1: In the legacy handover, the UE starts RLM immediately on the target PCell when the UE is downlink synchronized to the target PCell, but does not start T310 on the target PCell during the handover (i.e. when T304 is running).

Observation 2: In the legacy handover, when the UE receives the random access problem indication during the handover (i.e. when T304 is running), the UE does not trigger RLF.

Observation 3: In the legacy handover, there is no case that the UE receives the RLC status report from the target cell of the SRB during the handover (i.e. when T304 is running).

Observation 4: Keeping SRB1 during the non-DC-based handover helps reducing the impacts caused by the HOF and the target connection failure.

Proposal 1: The expiry of the T304 which starts at the execution of the non-DC-based handover command is used to determine the handover failure, as the legacy handover.

Proposal 2: The T304 of the non-DC-based handover is stopped when the MAC successfully completes the RACH to the target cell, as the legacy handover.

Proposal 3: The UE reports the handover failure via the source connection at the expiry of T304 if the UE does not detect the source connection RLF.

Proposal 4: The RLF detection (i.e. physical layer problem detection, RACH failure detection and RLC re-transmission failure detection) of the source connection is kept.

Proposal 5: If the UE detects the source connection RLF, the UE reports the source connection RLF to the target cell if the target connection does not declare the RLF/HOF.

Proposal 6: If the UE detects the source connection RLF and the target connection RLF/HOF at the same time, the UE triggers the RRC connection re-establishment.

Proposal 7: The PHY layer of the UE starts the target PCell RLM immediately after the reception of the handover command, as the legacy handover.

Proposal 8: The UE does not start the T310 of the target PCell when T304 is running, as the legacy handover.

Proposal 9: The UE does not trigger re-establishment when the RRC receives the random access problem indication from the target MAC when T304 is running, as the legacy handover.

Proposal 10: When the T304 is not running, the RLF detection of the target connection is the same as the legacy UE.

Proposal 11: Before releasing the source connection, the UE reports the target connection RLF via the source connection when the UE detects the target connection RLF.

Proposal 12: The SRB1 of the source connection is kept before releasing the source connection.

Proposal 13: The SRB1 of the target connection is established at the execution of the non-DC-based handover command, as the legacy handover.

# Reference

[1] RAN2#106 meeting minutes.

# Annex

### 5.3.11 Radio connection failure related actions #### 5.3.11.1 Detection of physical layer problems in RRC_CONNECTED The UE shall: 1> upon receiving N310 consecutive "out-of-sync" indications for the PCell from lower layers while neither T300, T301, T304 nor T311 is running: 2> start timer T310; 1> upon receiving N313 consecutive "out-of-sync" indications for the PSCell from lower layers while T307 is not running: 2> start T313; NOTE: Physical layer monitoring and related autonomous actions do not apply to SCells except for the PSCell.

#### 5.3.11.3 Detection of radio connection failure The UE shall: 1> upon T310 expiry; or 1> upon T312 expiry; or 1> upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or 1> upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB: 2> consider radio connection failure to be detected for the MCG i.e. RLF; 2> except for NB-IoT, store the following radio connection failure information in the VarRLF-Report by setting its fields as follows: 3> clear the information included in VarRLF-Report, if any; 3> set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN); 3> set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio connection failure; 3> set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio connection failure, and set its fields as follows; 4> if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA; 4> if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA; 4> if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN; 4> if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000; 4> for each neighbour cell included, include the optional fields that are available; NOTE 1: The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported. 3> if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs; 3> if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons; 3> if detailed location information is available, set the content of the locationInfo as follows: 4> include the locationCoordinates; 4> include the horizontalVelocity, if available; 3> set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio connection failure is detected; 3> set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio connection failure is detected; 3> if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure: 4> if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover: 5> include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received; 5> set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo; 4> if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Connection Failure Report for Inter-RAT MRO: 5> include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received; 5> set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo; 3> if the UE supports QCI1 indication in Radio Connection Failure Report and has a DRB for which QCI is 1: 4> include the drb-EstablishedWithQCI-1; 3> set the connectionFailureType to rlf; 3> set the c-RNTI to the C-RNTI used in the PCell; 3> set the rlf-Cause to the trigger for detecting radio connection failure; 2> if AS security has not been activated: 3> if the UE is a NB-IoT UE: 4> if the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation: 5> initiate the RRC connection re-establishment procedure as specified in 5.3.7; 4> else: 5> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure'; 3> else: 4> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'; 2> else: 3> initiate the connection re-establishment procedure as specified in 5.3.7; In case of DC, the UE shall: 1> upon T313 expiry; or 1> upon random access problem indication from SCG MAC; or 1> upon indication from SCG RLC, which is allowed to be sent on PSCell, that the maximum number of retransmissions has been reached for an SCG or split DRB: 2> consider radio connection failure to be detected for the SCG i.e. SCG-RLF; 2> initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio connection failure; In case of CA PDCP duplication, the UE shall: 1> upon indication from an RLC entity, which is restricted to be sent on SCell only, that the maximum number of retransmissions has been reached: 2> initiate the failure information procedure as specified in 5.6.21 to report RLC failure of type duplication; The UE may discard the radio connection failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio connection failure is detected, upon power off or upon detach.