3GPP TSG-RAN2 Meeting #107	R2-1910117

# Prague, Czech Republic, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1189 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on quantity configuration in MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section Sec 5.5.2.8 is speficied that the actions to be performed are "for each RAT for which the received quantityConfig includes parameter(s)”. However, in NR-DC this sentence does not hold since it should be in “each CG” rather than “each RAT”. According to this, a clarification is needed in the procedural text |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 5.5.2.8 Impact analysis Impacted 5G architecture options: SA, (NG)EN-DC, NE-DC, and NR-DC Impacted functionality: Quantity configuration Inter-operability: In the network implements the CR and the UE does not, the UE will receive a quantity configuration for each CG (in NR-DC). If the UE implements the CR and the network does not, the UE will apply the quantity configuration only to one CG (in NR-DC) thus leading to some configuration problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, it would be not clear that the quantity configuration received should be applied for each RAT (in (NG)EN-DC and NE-DC) and for each CG (in NR-DC). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.5.2.8	Quantity configuration

The UE shall:

1>	for each RAT and for each CG for which the received quantityConfig includes parameter(s):

2>	set the corresponding parameter(s) in quantityConfig within VarMeasConfig to the value of the received quantityConfig parameter(s);

1>	for each measId included in the measIdList within VarMeasConfig:

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

START OF CHANGES