3GPP TSG-RAN2 Meeting #107	R2-1910121

# Prague, Czech Republic, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1192 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Measurements handling in SCGFailureInformation(EUTRA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the SCG failure information procedure for NR-DC and NE-DC, it is not clear that measResultFreqList in SCGFailureInformation and SCGFailureInformationEUTRA is configured by a measConfig associated to the MCG. Further, the reporting of beam measurements in SCGFailureInformation(EUTRA) and the way how they are ordered, should follow the same approach as in the measurement reporting section 5.5.5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 5.7.3.4 Setting the content MeasResultSCG-Failure Procedural text has been aligned to what has been decided in the last RAN2#106 meeting for the reporting of measurement configured by the SCG - 5.7.3.5 Actions related to transmission of SCGFailureInformation message Procedural text has been aligned to what has been decided in the last RAN2#106 meeting for the reporting of measurement configured by the MCG -5.7.3a.3 Actions related to transmission of SCGFailureInformationEUTRA message Procedural text has been aligned to what has been decided in the last RAN2#106 meeting for the reporting of measurement configured by the MCG Impact analysis Impacted 5G architecture options: NR-DC, NE-DC Impacted functionality: SCGFailureInformation, SCGFailureInformationEUTRA Inter-operability: If the network implementes the CR and the UE does not, the network may not receive some measurements (e.g., beam level measurements) that may be usefull to decide what actions to perform. If the UE implements the CR and the network does not, there are not inter-operability problem since the fields received by the network do not change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the CR is not approved, will be missing some measurements that may be important for the network to take a decision on the action to perform in case of RLF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.3.4, 5.7.3.5, 5.7.3a.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.7.3.4	Setting the contents of MeasResultSCG-Failure

The UE shall set the contents of the MeasResultSCG-Failure as follows:

1>	for each MeasObjectNR configured on NR SCG for which a measId is configured and measurement results are available:

2>	include an entry in measResultPerMOList;

2>	if the reportConfig associated with the measId includes rsType:

3>	if the serving cell measurements based on the rsType included in the reportConfig are available:

4>	set the measResultServingCell within measResultPerMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig;

2>	else:

3>	if SSB based serving cell measurements are available:

4>	set the measResultServingCell within measResultPerMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

3>	else if CSI-RS based serving cell measurements are available:

4>	set the measResultServingCell within measResultPerMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

2>	if results derived based on SSB are included:

3>	include the ssbFrequency to the value indicated by ssbFrequency as included in the MeasObjectNR;

2>	if results based on CSI-RS are included:

3>	include the refFreqCSI-RS to the value indicated by refFreqCSI-RS as included in the MeasObjectNR;

2>	if the reportConfig associated with the measId includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

3>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

2>	if reportConfig associated with the measId includes reportAddNeighMeas:

3>	if the measObjectNR includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

4>	set the measResultBestNeighCellListNR within measResultPerMOList to include one entry with the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR;

5>	if the reportConfig associated with the measId includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

6>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

3>	for each neighbour cell included:

4>	include the optional fields that are available.

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

#### 5.7.3.5	Actions related to transmission of SCGFailureInformation message

The UE shall set the contents of the SCGFailureInformation message as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message due to T310 expiry:

2>	set the failureType as t310-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as synchReconfigFailure-SCG;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide random access problem indication from SCG MAC:

2>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to SRB3 IP check failure:

2>	set the failureType as srb3-IntegrityFailure;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to Reconfiguration failure of NR RRC reconfiguration message:

2>	set the failureType as scg-reconfigFailure.

1> include and set MeasResultSCG-Failure in accordance with 5.7.3.4;

1>	set the measResultFreqList to include for each NR frequency the UE is configured to measure by a MeasConfig associate with the MCG and for which measurement results are available:

2>	if the reportConfig associated with the measId configured to measured by a MeasConfig associated with the MCG includes rsType:

3>	if the serving cell measurements based on the rsType included in the reportConfig are available:

4>	set the measResultServingCell within measResultFreqList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig;

2>	else:

3>	if SSB based serving cell measurements are available:

4>	set the measResultServingCell within measResultFreqList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

3>	else if CSI-RS based serving cell measurements are available:

4>	set the measResultServingCell within measResultFreqList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

2>	if results for the serving cell derived based on SSB are included:

3>	include the ssbFrequency to the value indicated by ssbFrequency as included in the MeasObjectNR configured by a MeasConfig associated with the MCG;

2>	if results for the serving cell derived based on CSI-RS are included:

3>	include the refFreqCSI-RS to the value indicated by refFreqCSI-RS as included in the MeasObjectNR configured by a MeasConfig associated with the MCG;

2>	if the reportConfig associated with the measId includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

3>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

2>	if reportConfig associated with the measId configured to measured by a MeasConfig associated with the MCG includes reportAddNeighMeas:

3>	if the measObjectNR configured by a MeasConfig associated with the MCG includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

4>	set the measResultBestNeighCellListNR within measResultFreqList to include one entry with the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR;

5>	if the reportConfig associated with the measId configured to measured by a MeasConfig associated with the MCG includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

6>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

NOTE:	Field measResultSCG-Failure is used to report available results for NR frequencies the UE is configured to measure by SCG RRC signalling.

The UE shall submit the SCGFailureInformation message to lower layers for transmission.

END OF CHANGES

START OF CHANGES

#### 5.7.3a.3	Actions related to transmission of SCGFailureInformationEUTRA message

The UE shall set the contents of the SCGFailureInformationEUTRA message as follows:

1>	include failureType within failureReportSCG-EUTRA and set it to indicate the SCG failure in accordance with TS 36.331 [10] clause 5.6.13.3;

1>	include and set measResultSCG-FailureMRDC in accordance with TS 36.331 [10] clause 5.6.13.5;

1>	for each EUTRA frequency the UE is configured to measure by a MeasConfig associated with the MCG for which measurement results are available set the measResultFreqListMRDC as follows:

2>	set the measResultServingCell to include the available measurement quantities that the UE is configured to measure;

2>	if reportConfig associated with the measId configured to measured by a MeasConfig associated with the MCG includes reportAddNeighMeas:

3>	set the measResultFreqListMRDC to include within measResultBestNeighCell the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

NOTE:	Field measResultSCG-FailureMRDC is used to report available results for E-UTRAN frequencies the UE is configured to measure by E-UTRA SCG RRC signalling.

The UE shall submit the SCGFailureInformationEUTRA message to lower layers for transmission.

END OF CHANGE