3GPP TSG-RAN WG2 Meeting #107	R2-1910180

Prague, Czech Republic, 26th - 30th August 2019	revision of R2-1906166

Agenda Item:	12.1.12.1

Source:			Huawei, HiSilicon

Title:	Impact to support user plane optimisations for eMTC and NB-IoT connected to 5GC

Document for:	Discussion and Decision

# Introduction

At RAN#82, revision of the Rel-16 work items on additional enhancements for NB-IoT and MTC enhancements for LTE were agreed to include support of connection to 5GC and further revised at RAN2#84 [1], [2].

One of the common objective is as follows:

Connection to 5GC:

- Specify support for the following features [RAN2, RAN3]

  - Support of User Plane CIoT optimization

  - Support of EDT for Data over NAS and User Plane CIoT optimization

In this paper, we discuss the changes required to support the user plane EDT and user plane optimisations for eMTC/NB-IoT connected to 5GC.

Note this document does not discuss the impact of supporting RRC_INACTIVE for eMTC.

Compared to R2-1906166, this document takes into account the agreements made in SA2#133 and SA2#134 and in RAN2#106:

- UP optimisation solution is supported for both eMTC and NB-IoT connected to 5GC, agreed in RAN2

- RRC_INACTIVE is supported, optional, for eMTC UE connected to 5GC, agreed in RAN2

- Broadcast of whether N3 data transfer is supported or not in system information for NB-IoT agreed in SA2

- Reuse of R15 LTE security key derivation and algorithm agreed in RAN2

# Discussion

## 2.1 Non EDT aspects

### 2.1.1 Support Indications

Network Support indications in SIB

It has been agreed that user plane CIoT 5GS optimisation is supported for eMTC & NB-IoT devices connected to 5GC and that support and use were negotiated between the UE and the AMF at NAS level.

At RAN2#106, RAN2 has agreed that support of User plane CIoT 5GS optimisation is optional for both eMTC and NB-IoT devices connected to 5GC.

UP optimization solution is supported for both eMTC and NB-IoT connected to 5GC with capability signalling.

In Rel-13, there is no AS capability reporting for UP CIoT EPS optimisations, the UE User Plane CIoT Support Indicator IE is provided by the MME in the S1-AP INITIAL UE CONTEXT REQUEST, based on the result of the negotiation between the MME and the UE. The same applies over the N2 interface (SA2 has specified that if the UE and AMF indicate support for User Plane CIoT 5GS Optimisation, the AMF indicates support of User Plane CIoT 5GS Optimisation support for the UE to NG-RAN).

Proposal 1: Support of User Plane CIoT 5GS optimisation is optional for both eMTC and NB-IoT devices connected to 5GC without capability signalling.

In Rel-13, for eMTC, an indication up-CIoT-EPS-Optimisation is broadcast per PLMN to indicate whether the UE is allowed to resume the connection with User plane CIoT EPS Optimisation in the cell. For NB-IoT, even though support of the feature is optional in the network, it was agreed not to signal any indication in system information and have ‘graceful’ handling of a resume request (i.e. fallback to RRC connection establishment) in the eNB when not supported.

In Rel-16, SA2 has agreed that the UE selects the core network type (EPC or 5GC) based on the broadcast indications for both EPC and 5GC, and the UE's (5G) Preferred Network Behaviour. Thus, it is also necessary for NB-IoT, to indicate support of the User plane CIoT 5GS Optimisation in system information.

Also at RAN2#105bis, it was agreed “For NB-IoT/eMTC: SIB-NB/SIB1-BR extended to include per PLMN indication of the supported CIoT 5GS Optimisations”

Proposal 2: For NB-IoT and eMTC, introduce a new parameter up-CIoT-5GS-Optimisation-r16 in SIB1-BR/SIB1-NB to indicate per PLMN support of User plane CIoT 5GS optimisations.

In Rel-13, for eMTC, support of S1-U data transfer without support for User plane CIoT EPS optimisation is a mandatory feature at the UE and the NW. For NB-IoT, distinction between support of S1-U data transfer and User plane CIoT EPS Optimisation was introduced only after the freeze of the release and no indication was introduced to avoid change to the ASN.1.

In Rel-16, for eMTC, following the same approach, support of S1-U data transfer is a mandatory feature at the UE and the NW.

For NB-IoT, SA2 has agreed “For a network that supports NB-IoT, it shall broadcast an indication of whether N3 data transfer is supported or not in system information”.

Proposal 3: For eMTC, support of N3 data transfer is mandatory (UE and NW) hence no broadcast signalling required for this.

Proposal 4: For NB-IoT, support of N3 data transfer is optional (UE and NW), hence introduce a new parameter n3-5GS-DataTransfer-r16 in SIB1-NB to indicate per PLMN support of N3 user plane data transfer.

UE support indication in RRC Connection Setup Complete

In Rel-13, to assist the eNB in the MME selection, the eMTC and NB-IoT UE indicates in RRCConnectionSetupComplete message the CIoT EPS optimisations that it supports.

In Rel-16, as SA2 has agreed that the eNB performs AMF selection based on reported support indications, the same approach should apply for both eMTC and NB-IoT.

Proposal 5: For eMTC, introduce new parameters up-CIoT-5GS-Optimisation-r16 and cp-CIoT-5GS-Optimisation-r16 in RRCConnectionSetupComplete when accessing 5GC.

Proposal 6: For NB-IoT, introduce new parameters up-CIoT-5GS-Optimisation-r16 and n3-5GS-DataTransfer-r16 in RRCConnectionSetupComplete-NB when accessing 5GC.

### 2.1.2 RRC Connection Resume for non-EDT access

RRC Connection Resume Request

In Rel-15, RRC connection resumption when accessing 5GC takes place when the UE is in RRC_INACTIVE, a new resumeIdentity (carrying the full or short I-RNTI) was defined and security is re-activated prior to sending RRCConnectionResumeRequest. Due to the message size limitation and also to distinguish between legacy R13 EPC UP Optimization Resume and R15 5GC RRC_INACTIVE Resume procedure at ng-eNB, a critical extension of RRCConnectionResumeRequest was created to carry the new resumeIdentity. In addition, a UE in RRC_INACTIVE enters RRC_IDLE and releases the UE Inactive AS context when it switches to another CN type, i.e. EPS. Similarly a UE in RRC_IDLE in ‘Suspended’ state in EPS can only initiate RRC Connection Establishment when switching to 5GC type.

For eMTC and NB-IoT connected to 5GC, SA2 has agreed to go ahead with the UP CIoT 5GS optimisation solution, i.e. the RRC connection resumption when accessing 5GS takes place when the UE is in RRC_IDLE with suspend, same as when connected to EPC. It was also agreed at RAN2#106 that RRC_INACTIVE is optionally supported by eMTC UE connected to 5GC.

For eMTC connected to 5GC, for consistent handling of UE Identity when connected to 5GC for both RRC_INACTIVE and UP CIoT 5GS Optimisation, same I-RNTI can be adopted for UP CIoT 5GS Optimisation and this also avoids any RAN3 impact for handling different UE Identity for different features for 5GC connectivity. This allows usage of R15 introduced critical extension version of RRCConnectionResumeRequest for eMTC connected to 5GC. Since ng-eNB uses either UP CIoT 5GC Optimisation or RRC_INACTIVE for a given UE at any one time, there should not be any ambiguity for ng-eNB connected to 5GC.

For NB-IoT connected to 5GC, in order to keep consistent handling of UE Identity when connected to 5GC, I-RNTI can also be used as the UE identity. In addition, in order for ng-eNB to differentiate between legacy R13 UP CIoT EPS optimisation resume request and R16 UP CIoT 5GS optimisation resume request, we propose to introduce critical extension version of RRCConnectionResumeRequest-NB message for 5GC connectivity.

Proposal 7: For NB-IoT and eMTC connected to 5GC, adopt I-RNTI as UE identity for UP CIoT 5GS Optimisation.

Proposal 8: For NB-IoT connected to 5GC, create a critical version of RRCConnectionResumeRequest-NB message for UP CIoT 5GS optimisation, including the I-RNTI, shortResumeMAC-I, resumeCause (same causes as agreed for RRCConnectionRequest-NB in 5GC), and cqi-NPDCCH.

Proposal 9: For eMTC connected to 5GC, adopt R15 introduced critical extension version of RRCConnectionResumeRequest for UP CIoT 5GS optimisation, including the I-RNTI, shortResumeMAC-I, and resumeCause.

In Rel-15 eLTE, UE resumes the AS context and re-activates security prior to sending RRCConnectionResumeRequest.  For UP CIoT 5GS optimisation, we can follow the same security principle as RRC_INACTIVE when connected to 5GC. This will avoid different security behaviour for different features when UE connected to 5GC, e.g. UP optimisation, EDT and RRC_INACTIVE.

Proposal 10: For NB-IoT and eMTC, adopt the same security principle as RRC_INACTIVE when connected to 5GC.

By analogy with Rel-15 eLTE and to avoid the complexity of cross-CN resumption procedure, that would impact also other groups, also for NB-IoT and eMTC, the UE releases the stored UE AS context when reselecting to another CN type.

Proposal 11: For NB-IoT and eMTC, the UE releases the UE AS context when it reselects to another CN type.

RRC Connection Resume

In Rel-15 eLTE, full configuration is allowed during connection resumption to 5GC and a new parameter fullConfig was added in RRCConnectionResume message. This was motivated by the fact that security is reactivated before resumption and that RRCConnectionResume is ciphered.

Proposal 12: For NB-IoT and eMTC, full configuration in RRCConnectionResume(-NB) is supported when connected to 5GC.

RRC Connection Release for suspension to RRC_IDLE

In Rel-15 eLTE, the configuration for next connection resumption is provided in IE RRC-InactiveConfig in RRCConnectionRelease message and the inclusion of the IE triggers transition to RRC_INACTIVE.

As RRC_INACTIVE and UP optimisations are two independent features, it is proposed not to reuse this IE for the UP 5GS optimisation. Still the UE idle mode Identity (I-RNTI) and the next hop chaining counter (NCC) need to be provided.

Proposal 13: For NB-IoT and eMTC connected to 5GC, introduce a new IE up-CIOT-5GS-OptimisationConfig-r16 in RRCConnectionRelease including the full I-RNTI and nextHopChainingCount for UP CIoT 5GS optimisation.

Proposal 14: For eMTC connected to 5GC, also include the short I-RNTI in IE up-CIOT-5GS-OptimisationConfig-r16 in RRCConnectionRelease for UP CIoT 5GS optimisation.

Fallback to RRC connection establishment

In Rel-15 eLTE, upon reselection to another CN type from RRC_INACTIVE, the UE enters RRC_IDLE and discards the UE Inactive AS context (TS 36.331 section 5.3.17.3)..

For the UP optimisations, UE is in RRC_IDLE both in EPC and 5GC. However, it is not possible to resume in a different CN (e.g different security key). Thus the same should apply.

Proposal 15: For NB-IoT and eMTC, when the UE in RRC_IDLE using the UP CIoT optimisation reselects to another CN type, it discards the UE AS context and the UE identity.

### 2.1.3 RRC Connection Re-establishment for the user plane

In Rel-15 eLTE, UE goes to RRC_IDLE and initiates NAS recovery procedure when the UE switches between CN types at RRC Connection re-establishment. This has been motivated by the need of cross-CN reestablishment, with many RAN3 and SA2 impacts.

By analogy with R15 eLTE and for the same reasons, the same should apply to NB-IoT and eMTC.

Proposal 16: For NB-IoT and eMTC, the UE goes to RRC_IDLE and initiates NAS recovery procedure when the UE switches between CN types at RRC Connection re-establishment for the user plane.

## 2.2 EDT aspects

### 2.2.1 Support Indication

Network support Indications in SIBs

At RAN2#105bis, for CP-EDT, the following was agreed:

Working assumption: Introduce a new IE “cp-EDT-5GC-r16” in SIB2-BR/SIB2-NB to indicate ng-eNB connected to 5GC supports CP-EDT optimization.

Rel-15 eMTC/NB-IoT EDT PRACH Configuration parameters can be commonly used for both EPC and 5GC.

For UP-EDT, the same agreements are also valid.

Proposal 17: Introduce a new IE “up-EDT-5GC-r16” in SIB2-BR/SIB2-NB to indicate ng-eNB connected to 5GC supports UP-EDT.

UE support indication in UE capability

In Rel-15 EPS, in order to provide the NCC and drb-ContinueROHC in RRCConnectionRelease message, the eNB needs the knowledge that the UE supports UP-EDT. For EPS, this is realised by the reporting of a UE capability, earlyData-UP-r15.

For connection to 5GC, if proposal 10 is agreed, and 5GS security principles are reused, then NCC will always been provided when the connection is suspended. However, the eNB still needs to know whether EDT is supported in order to provide drb-ContinueROHC. By analogy with EPS and because the UE may have different capabilities when connected to EPC or 5GC, it is proposed to introduce a new capability for support of UP-EDT in 5GC, earlyData-UP-5GC-r16

Proposal 18: Introduce a new UE capability “earlyData-UP-5GC-r16’ in UE-EUTRA-Capability, UE-Capability-NB to indicate support of UP-EDT in 5GC.

### 2.2.2 UP-EDT procedure

The UP-EDT procedure is a pure AS procedure introduced on top of the UP CIoT EPS optimisation and no changes additional to the ones described in section 2.1.3 are expected for NB-IoT and eMTC connected to 5GC.

Proposal 19: The EPS UP-EDT procedure is reused, the changes described in section 2.1.3 apply for UP-EDT when connected to 5GC.

If proposal 10 (security activation) is agreed, changes to EDT fallback are needed so that the security activation and SRB1 is maintained.

Proposal 20: Security should not be deactivated and SRB1 should not be suspended at EDT fallback for UP-EDT when connected to 5GC.

# 3 Conclusion

In this document, we have discussed the changes required to support the User plane CIoT 5GS optimisation and UP-EDT when connected to 5GC. We have made the following proposals:

Proposal 1: Support of User Plane CIoT 5GS optimisation is optional for both eMTC and NB-IoT devices connected to 5GC without capability signalling.

Proposal 2: For NB-IoT and eMTC, introduce a new parameter up-CIoT-5GS-Optimisation-r16 in SIB1-BR/SIB1-NB to indicate per PLMN support of User plane CIoT 5GS optimisations.

Proposal 3: For eMTC, support of N3 data transfer is mandatory (UE and NW) hence no broadcast signalling required for this.

Proposal 4: For NB-IoT, support of N3 data transfer is optional (UE and NW), hence introduce a new parameter n3-5GS-DataTransfer-r16 in SIB1-NB to indicate per PLMN support of N3 user plane data transfer.

Proposal 5: For eMTC, introduce new parameters up-CIoT-5GS-Optimisation-r16 and cp-CIoT-5GS-Optimisation-r16 in RRCConnectionSetupComplete when accessing 5GC.

Proposal 6: For NB-IoT, introduce new parameters up-CIoT-5GS-Optimisation-r16 and n3-5GS-DataTransfer-r16 in RRCConnectionSetupComplete-NB when accessing 5GC.

Proposal 7: For NB-IoT and eMTC connected to 5GC, adopt I-RNTI as UE identity for UP CIoT 5GS Optimisation.

Proposal 8: For NB-IoT connected to 5GC, create a critical version of RRCConnectionResumeRequest-NB message for UP CIoT 5GS optimisation, including the I-RNTI, shortResumeMAC-I, resumeCause (same causes as agreed for RRCConnectionRequest-NB in 5GC), and cqi-NPDCCH.

Proposal 9: For eMTC connected to 5GC, adopt R15 introduced critical extension version of RRCConnectionResumeRequest for UP CIoT 5GS optimisation, including the I-RNTI, shortResumeMAC-I, and resumeCause.

Proposal 10: For NB-IoT and eMTC, adopt the same security principle as RRC_INACTIVE when connected to 5GC.

Proposal 11: For NB-IoT and eMTC, the UE releases the UE AS context when it reselects to another CN type.

Proposal 12: For NB-IoT and eMTC, full configuration in RRCConnectionResume(-NB) is supported when connected to 5GC.

Proposal 13: For NB-IoT and eMTC connected to 5GC, introduce a new IE up-CIOT-5GS-OptimisationConfig-r16 in RRCConnectionRelease including the full I-RNTI and nextHopChainingCount for UP CIoT 5GS optimisation.

Proposal 14: For eMTC connected to 5GC, also include the short I-RNTI in IE up-CIOT-5GS-OptimisationConfig-r16 in RRCConnectionRelease for UP CIoT 5GS optimisation.

Proposal 15: For NB-IoT and eMTC, when the UE in RRC_IDLE using the UP CIoT optimisation reselects to another CN type, it discards the UE AS context and the UE identity.

Proposal 16: For NB-IoT and eMTC, the UE goes to RRC_IDLE and initiates NAS recovery procedure when the UE switches between CN types at RRC Connection re-establishment.

Proposal 17: Introduce a new IE “up-EDT-5GC-r16” in SIB2-BR/SIB2-NB to indicate ng-eNB connected to 5GC supports UP-EDT.

Proposal 18: Introduce a new UE capability “earlyData-UP-5GC-r16’ in UE-EUTRA-Capability, UE-Capability-NB to indicate support of UP-EDT in 5GC.

Proposal 19: The EPS UP-EDT procedure is reused, the changes described in section 2.1.3 apply for UP-EDT when connected to 5GC.

Proposal 20: Security should not be deactivated and SRB1 should not be suspended at EDT fallback for UP-EDT when connected to 5GC.

# 4	Reference

RP-191576 “WID revision: Additional enhancements for NB-IoT”, Huawei, RAN#84, Newport Beach, USA, June 2019

RP-191356 “WID revision: Additional MTC enhancements for LTE”, Ericsson, RAN#84, Newport Beach, USA, June 2019

R2-1908188 “RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”, BlackBerry, RAN2#106, Reno, USA, May 2019

R2-1910184 “Support of Slicing and 5G QoS framework for NB-IoT and eMTC connected to 5GC”, Huawei, HiSilicon, RAN2#107, Prague, Czech Republic. August 2019