3GPP TSG-RAN WG2 Meeting #107     							         R2-1910217

Prague, Czech Republic, 26th - 30th August 2019

Source: 	vivo

Title:  	PC5 SDAP protocol in NR V2X

Agenda Item:	11.4.2

Document for:	Discussion and Decision

Introduction

In last RAN2 meetings, some conclusions about PC5 L2 protocol in NR V2X were agreed. The agreements are as follows.

RAN2#104 Agreements 11: In NR, PC5-C protocol stack includes at least RRC, RLC, MAC and PHY sub-layers. Whether to have PDCP sub-layer depends on whether any new PC5 RRC message other than MIB-SL is introduced (e.g. outcome of [103bis#38]) Agreements on RLC: 8: Segmentation and reassembly of RLC SDUs are supported in NR RLC for NR sidelink broadcast, groupcast and unicast. 9: RLC SDU discard function is supported in NR RLC for NR sidelink broadcast, groupcast and unicast. 10: If SBCCH is used for NR sidelink (dependent on RAN1 decision on synchronization aspect), a NR TM RLC entity is configured to submit/receive RLC PDUs. 11: A NR UM RLC entity is configured to submit/receive RLC PDUs, for user packets of SL broadcast, groupcast and unicast. RLC AM is not supported for broadcast. Agreements on PDCP: 12: Sidelink packet duplication is supported in NR PDCP for NR sidelink broadcast, groupcast. FFS on unicast. 13: Timer based SDU/PDU discard function is supported in NR PDCP for NR sidelink broadcast, groupcast and unicast. RAN2#105 Agreements on groupcast: 5: RLC UM mode is used for groupcast. RLC AM mode for groupcast is not supported. Agreements on QoS: 5: For NR SL unicast, some SLRB configurations need to be informed by the one UE to the peer UE in SL, including at least SN length, RLC mode (related to also Q9) and PC5 QoS profile associated with each SLRB. Other SLRB related parameters are not excluded. 6: SDAP layer is needed at least for NR SL unicast, performing PC5 QoS flow to SLRB mapping. SDAP layer is not needed for per-packet QoS model, e.g. broadcast. 7: RLC AM is supported for NR SL unicast.

In this contribution, we will discuss detailed PC5 SDAP protocol and default parameters for broadcast and groupcast. And our proposals are given.

Discussion

According to newest SA2 LS, SA2 had agreed that per-flow QoS model will be applied to all three types of cast, i.e. unicast, groupcast and broadcast [3].

It was agreed in SA2 to use the bearer based QoS Model (i.e. Per-Flow QoS model) for unicast, groupcast and broadcast operations (please see the attached S2-1904426). SA2 believes that a single PC5 QoS model for all NR sidelink cast type aligned with the Uu QoS model simplifies the overall system behavior. SA2 kindly requests RAN2 to take this information into account in their work.

That is to say, all three types of SL cast, i.e. unicast, groupcast and broadcast, need SDAP layer to perform PC5 QoS flow to SLRB mapping.

Proposal 1: All three types of SL cast, i.e. unicast, groupcast and broadcast, need SDAP layer to perform PC5 QoS flow to SLRB mapping.

In current NR Uu, SDAP sublayer is established, configured and released by RRC signalling. For NR V2X, RRC signalling is available in unicast case, where similar RRC control for SDAP layer establishment, configuration and releasing can be reused for both transmitter UE and receiver UE.

Proposal 2: Similar RRC control method as NR Uu can be reused for NR V2X SDAP sublayer in unicast case.

For groupcast and broadcast cases, pre-configuration or SIB-based configuration for QoS and SLRB can be used. TX SDAP sublayer management may be controlled by its RRC layer or considered as implementation and RX SDAP sublayer management is totally left to implementation, even RX SDAP sublayer is not needed because of no SDAP header handling.

In current NR Uu, the configuration for SDAP sublayer includes following information:

- PDU session ID;

- SDAP header present indication for UL/DL;

- Default DRB indication;

- Mapping information between QoS flow and DRB;

Among them, PDU session ID is equivalent to SDAP entity ID because there is one-to-one mapping between PDU session and SDAP entity. QoS flows belonging to different PDU session cannot belong to either same SDAP entity or same radio bearer. We think similar mapping relationship rules will be reused in NR V2X if SA2 agrees to reuse the similar QoS framework, e.g. PC5 link/destination ID instead of PDU session concept in NR Uu. Moreover Destination ID and QoS flow ID should be carried by each packet from high layer to transmitter UE AS layer. From the Destination ID of each packet, transmitter UE can decide which SDAP sublayer will be used or established. If there is no PDU session in NR SL, transmitter UE will maintain only one SDAP entity for each Destination ID.

Proposal 3: RAN2 to ask SA2 confirm whether PDU session concept in NR Uu can be replaced directly by Destination ID, which means transmitter UE maintains only one SDAP entity for each Destination ID.

And from the QoS flow ID of each packet, transmitter UE can get the QoS requirement of that packet and decide how to map to a SLRB. Without RRC configuration, mapping from QoS flow to SLRB is based on pre-configuration or SIB-based configuration. And default DRB is used to map those QoS flows that have no mapping configuration between RB.

Proposal 4: Without RRC configuration, SDAP entity will be established and managed for each destination ID by V2X transmitter UE based on destination ID and QFI carried in each packet from high layer.

For SDAP header, UL/DL headers have different roles in current Uu. UL SDAP header is mainly to differentiate QoS flows for corresponding UL N3 tunnel transmission and statistics or charging in core network. DL SDAP header is mainly for the purpose of reflective QoS/mapping. In our view, either reflective IP flow to QoS flow mapping or reflective QoS flow to SLRB mapping is unfeasible since the former is controlled by core network entity, e.g. PCF, and the latter is controlled by gNB. Furthermore reflective service or SLRB maintenance is difficult for V2X service. Hence reflective mapping function is not needed in NR V2X. That is to say, it is unnecessary to carry QoS flow ID from the transmitter to the receiver except that there is some statistic requirement based on per QoS flow level. RAN2 can send or reply LS to SA2 to ask whether there is any necessity of carrying QoS flow ID from the transmitter to the receiver in PC5 communication.

Proposal 5: From RAN2 perspective, SDAP header is not needed and an LS can be sent to SA2 to ask whether there is any necessity of carrying QoS flow ID in PC5 packets.

If there is no SDAP header in PC5 packets, the peer SDAP entity in receiver UE is not needed because of not performing any functions.

Conclusions

Based on the discussions given above, we have the following proposals：

Proposal 1: All three types of SL cast, i.e. unicast, groupcast and broadcast, need SDAP layer to perform PC5 QoS flow to SLRB mapping.

Proposal 2: Similar RRC control method as NR Uu can be reused for NR V2X SDAP sublayer in unicast case.

Proposal 3: RAN2 to ask SA2 confirm whether PDU session concept in NR Uu can be replaced directly by Destination ID, which means transmitter UE maintains only one SDAP entity for each Destination ID.

Proposal 4: Without RRC configuration, SDAP entity will be established and managed for each destination ID by V2X transmitter UE based on destination ID and QFI carried in each packet from high layer.

Proposal 5: From RAN2 perspective, SDAP header is not needed and an LS can be sent to SA2 to ask whether there is any necessity of carrying QoS flow ID in PC5 packets.

Reference

[1] RAN2 meeting, chairman minute.

[2] TS37.324 Service Data Adaptation Protocol (SDAP), Vf10.

[3] S2-1904823, LS response on unicast, groupcast and broadcast in NR sidelink, 201904.

Text proposal for TS38.300

- 6.5	SDAP Sublayer

The main services and functions of SDAP include:

-	Mapping between a QoS flow and a data radio bearer;

-	Marking QoS flow ID (QFI) in both DL and UL packets.

A single protocol entity of SDAP is configured for each individual PDU session.

For V2X sidelink communication, a single protocol entity of SDAP is configured for each destination ID.