3GPP TSG-RAN WG2 #107	Tdoc R2-1910268

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	11.10.4.4

Source:	Ericsson

Title:	Release and add of MCG in MR-DC

Document for:	Discussion, Decision

# Introduction

One of the objectives for the Rel-16 WI on DC and CA enhancements is:

- Efficient and low latency serving cell configuration/activation/setup: Minimizing signalling overhead and latency needed for initial cell setup, additional cell setup and additional cell activation for data transmission. [RAN2, RAN1, RAN4, RAN3]

  - This objective applies to MR-DC, NR-NR DC and CA

  - The objective should consider enhancements when starting from IDLE, INACTIVE mode and CONNECTED mode

This paper discusses enhancements to inter-MN HO in MR-DC

# Discussion

When a UE performs a full configuration, it releases all radio resources as well as any SCG configuration. The main reason to perform this procedure is when a target node does not understand and/or support certain features or configurations configured in the source.

In case the UE performs an inter-MN handover without SN change, the target MN requests the SN to add an SCG, and the SN responds with an SCG addition acknowledgement.

Figure 1: Inter-MN handover with/without MN initiated SN change procedure (TS 37.340)

Since the SN addition request can contain the source SCG configurations, the SN can compute a delta configuration and indicate in the SN addition request ack whether the configurations are full or delta.

An SN can indicate during SN addition whether the configurations are full or delta in the S-Node addition request acknowledge

In case the UE performs an inter-MN handover with or without SN change, if target MN is unable to understand the source MN radio resource configuration it would have to perform a full configuration in order to release the old configuration and provide the new. Even if the SN configurations should remain the same, the UE will perform a full configuration on the SN as well.

At inter-MN handover with/without SN change, if the MN has to perform full config, then the SN will also perform a full config

On the other hand, if the UE performs an SN change, the network can release and add the SCG to provide a full configuration without the need to perform a full configuration on the MN, which allows the MN to continue the connection.

SN change allows a release-and-add of the SCG to avoid a full configuration of the MN

If it was possible to perform a release-and-add of the MCG at inter-MN handover with/without SN change, the MN could perform a full config of the lower layers without affecting the SCG. Then SN would then be able to provide a delta configuration for the SCG based on the old configurations.

Introduce an MCG release-and-add flag in MR-DC to allow delta configurations for the SCG when full config of only the MCG is required

# Conclusion

In this contribution we made the following observations:

Observation 1	An SN can indicate during SN addition whether the configurations are full or delta in the S-Node addition request acknowledge

Observation 2	At inter-MN handover with/without SN change, if the MN has to perform full config, then the SN will also perform a full config

Observation 3	SN change allows a release-and-add of the SCG to avoid a full configuration of the MN

Based on the discussion we propose the following:

Proposal 1	Introduce an MCG release-and-add flag in MR-DC to allow delta configurations for the SCG when full config of only the MCG is required