3GPP TSG-RAN WG2#107	R2-1910275

Prague, Czech Republic, 26th – 30th August 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | DraftCR | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Measurement handling in MCGFailureInformation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-08-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#106, it was agreed to include in MCGFailureInformation available measurement results of MCG, SCG and non-serving cells. However, some general guidelines are required in order to add the procedural text and corresponding ASN.1. In this CR, we make the following assumptions: - only measurements configured by the MN should be included. In the procedural text, we have marked this “measurement configuration associated with MCG” - Measurement results should include also beam measurement results. - Neighbor cell measurements are reported for both MCG and SCG. Following these guidelines, in this CR we provide the necessary updates to the Running CR for 36.331 for CA&DC enhancements to include these measurements. In general, we have followed the same approach as in the measurement reporting section 5.6.13 and 5.6.13a |  |  |  |  |  |  |  |  |
| Summary of change: |  | *** NOTE: This CR is written relative to the Running CR for 36.331 for CA&DC enhancements *** In section 5.6.x.4 - Added procedural text for adding the measurement results for MCG, SCG and non-serving cells. In section 6.3.5 - Added the Information Element for MeasResultMCG-FailureMRDC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement results for MCG, SCG and non-serving cells cannot be included in MCGFailureInformation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.x.4, 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.6.x.4 Setting the contents of MeasResultMCG-FailureMRDC

The UE shall set the contents of the MeasResultMCG-FailureMRDC as follows:

1>	set the measResultServFreqList to include for each EUTRA MCG cell that is configured, if any, within measResultCell the quantities of the concerned SCell, if available according to performance requirements in TS 36.133 [16];

1>	for each EUTRA MCG serving frequency included in measResultServFreqList, include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	set the measResultNeighCells to include the best measured cells on non-serving EUTRA frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

2>	if the UE was configured to perform measurements for one or more non-serving EUTRA frequencies and measurement results are available, include the measResultListEUTRA;

2>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

1>	for each NR frequency the UE is configured to measure by measConfig for which measurement results are available:

2>	set the measResultFreqListNR to include the best measured cells, ordered such that the best cell is listed first using RSRP to order if RSRP measurement results are available for cells on this frequency, otherwise using RSRQ to order if RSRQ measurement results are available for cells on this frequency, otherwise using SINR to order, and based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available;

END OF CHANGES

START OF CHANGES

### 6.3.5	Measurement information elements

[…]

#### –	MeasResultMCG-FailureMRDC

The IE MeasResultMCG-FailureMRDC is used to provide measurement information regarding failures detected by the UE in (NG)EN-DC.

MeasResultMCG-FailureMRDC information element

-- ASN1START

MeasResultMCG-FailureMRDC ::=	SEQUENCE {

measResultServFreqList				MeasResultServFreqList-r10			OPTIONAL,

measResultNeighCells				MeasResultList2EUTRA-r9				OPTIONAL,

measResultFreqListNR				MeasResultFreqListFailNR-r15		OPTIONAL,

...

}

-- ASN1STOP

END OF CHANGES