3GPP TSG-RAN2 Meeting #107 	R2-1910548

# Prague, Czech Republic,  26 – 30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0155 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In a UE implementation, a UE supports NR and IMS voice over LTE should be allowed to not support IMS voice over NR. However, the current text does not allow the UE implementation. - If the UE supports IMS voice over NR, the UE is mandated to support IMS voice over LTE. - Remove TBDs for voiceOverSCG-BearerEUTRA-5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify in the field description of voiceOverNR so that the UE is allowed to support NR and IMS voice over LTE but does not support IMS voice over NR. - Clarify in the field description of voiceOverNR that if the UE supports IMS voice over NR, the UE is mandated to support IMS voice over LTE. - Remove TBDs for voiceOverSCG-BearerEUTRA-5GC. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: IMS voice over NR Inter-RAT handover from NR to LTE for IMS voice call Inter-operability: If the UE is implemented according to the CR but the network is not, there is no inter-operability issue since the changes for the UE align with the network’s expectation. If the network is implemented according to the CR but the UE is not, there may be unexpected behaviours for a case that the UE supports IMS voice over NR but does not support IMS voice over LTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - A UE supports NR and IMS voice over LTE is mandated to support IMS voice over NR. - The gNB does not know whether the UE supporting IMS voice over NR supports IMS voice over LTE. - TBDs remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.2.13	IMS Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| voiceOverEUTRA-5GC Indicates whether the UE supports IMS voice over E-UTRA via 5GC. It is mandated to the UE which is IMS voice capable in EUTRA/5GC; otherwise optional. | UE | No | No | No |
| voiceOverNR Indicates whether the UE supports IMS voice over NR. It is mandated to the UE which is IMS voice capable in NR otherwise optional. If this field is included, the UE capable of EUTRA/EPC shall support IMS voice over EUTRA/EPC. | UE | No | No | Yes |
| voiceOverSCG-BearerEUTRA-5GC Indicates whether the UE supports IMS voice over SCG bearer of NE-DC. | UE | No | No | N/A |