3GPP TSG-RAN WG2 #107	Tdoc R2-1910776

Prague, Czech Republic, 26 – 30 Aug 2019                       (Revision of R2-1907581)

Agenda Item:	11.2.1.2

Source:	Ericsson

Title:	Coexistence Between Configured and Dynamically Scheduled UL Grants

Document for:	Discussion, Decision

# Introduction

At RAN1#93, below agreements have been made concerning the configured grant transmission for NR-U.

The following modifications to the configured grant procedures are beneficial - Removing dependencies of HARQ process information to the timing - Introducing UCI on PUSCH to carry HARQ process ID, NDI, RVID - Introducing Downlink Feedback Information (DFI) including HARQ feedback for configured grant transmission - Increased flexibility on time domain resource allocation for the configured grant transmissions - Supporting retransmissions without explicit UL grant

In NR licensed, two different types of scheduling options are available, i.e. the dynamic/scheduled UL grants, and the configured grants (e.g. based on the SPS framework). As specified in NR licensed, both scheduling options can coexist and in general the UE shall prioritize dynamic UL grant.

However, in some cases due to the nature of configured grant scheme, race conditions between a dynamic UL grant and a configured grant may occur. According to RAN1 agreements highlighted above, in configured grant there is no association between HARQ ID and the timing, which implies that the gNB may provide a dynamic UL grant to schedule a certain HARQ process, while the UE has already started preparing a configured transmission for the same HARQ process. Or, similarly at the time of reception of dynamic UL grant, the UE has already performed a configured transmission for the same HARQ process.

However, in our understanding, not handling such issue might lead to spurious UE (re)transmissions which just increase interference in the unlicensed spectrum and UE battery consumption.

# Discussion

IN NR-U configured grant-based transmission, the decision of which HARQ ID to process in a certain slot is left to the UE. This implies that the network is not aware of which HARQ ID, the UE intends to transmit on a given slot when a configured grant is used. Therefore, it might happen that the gNB dynamically schedules a certain HARQ ID while the UE is preparing or just recently performed the (re)transmission of data with a configured grant and associated to the same HARQ ID.

In case the gNB provides a dynamic grant for transmitting a certain HARQ ID exactly in the same slot in which the UE intends to transmit this HARQ ID with a configured grant, the UE should prioritize the dynamic scheduling assignment as in NR licensed.

As in NR licensed, the UE prioritizes dynamically scheduled UL transmission when that occurs in the same slot as a configured UL grant, that is, no spec change.

Assuming that dynamically scheduled UL transmissions can be performed while a UE is configured with UL grant, we believe that it should be discussed how such two UL transmission schemes can coexistence. In particular, in our understanding, coexistence issues between a configured grant and a dynamically scheduled grant for the same HARQ process may arise in some cases, as also explained in [1]. Examples are illustrated in Figure 1 and summarized in the following:

- Scenario 1: The gNB dynamically schedules a certain HARQ ID while the UE has already started the preparation of a transmission with a configured grant for the same HARQ ID which should occur at slot n. In this case, the dynamic grant over PDCCH is received just a short time (e.g. 2 slots in Figure 1) before the planned transmission with the configured grant. At this point in time, the UE might not be able to interrupt the preparation of the planned transmission with the configured grant at slot n.

- Scenario 2: The gNB dynamically schedules a certain HARQ ID in the same slot or just a short time (e.g. 1 slot in Figure 1) after a performed transmission with a configured grant using the same HARQ ID. In this case, the UL grant might be sent by the gNB before the gNB has been able to process the received transmission with the configured grant.

In scenario 1, the UE could certainly perform both the transmission with configured grant at slot n and the scheduled UL transmission at slot n+2. For the scenario 2, the UE can certainly transmit both the data with the configured grant in slot n, and the scheduled UL data in slot n+5. However, the benefit of performing two transmissions so close in time for the same HARQ process is arguable. It can speed up and facilitate the correct decoding of the HARQ process, but it increases the overhead which is in general not desired, especially in the unlicensed spectrum.

Figure 1: Coexistence issues between configured grants and dynamically scheduled UL grants for the same HARQ process.

Given the above considerations, we observe the following:

- When a configured grant is configured, the HARQ process that the UE will transmit on a certain slot is not known by the gNB.

- It can happen that the gNB provides a DCI with UL grant for a HARQ process just a short time before or after the slot in which the UE intends to perform a transmission with a configured grant for the same HARQ process.

- Performing both a transmission with a configured grant and a scheduled UL transmission for the same HARQ process very close in time increases overhead which is not desirable, especially in the unlicensed spectrum.

Considering the above coexistence scenarios, the UE behavior might be ambiguous, because some UEs might be capable of performing both transmissions, while some other might need to discard one of the two grants. This would lead to an ambiguity of the UE behaviors. It is also worth noting that the gNB may not be able to process the consecutive transmissions using the same HARQ process close in time, so that the soft buffer may be overridden which may lead to packet loss.

One way to avoid such ambiguity is that the UE does not process a grant which is received for the HARQ process if transmitting using the grant would result in that the UE performs a transmission from the HARQ process which the UE recently (as defined by the time T) performed a transmission from. Another possible way which this may be realized is that the UE considers the grant invalid or in other way not act upon the grant. The time T can be configured by the gNB considering the time components including the gNB process time of a received PUSCH, the transmission time of a dynamic grant, and the delay between UL grant reception in DL (PDCCH) and UL data (PUSCH) transmission (i.e., configured via the delay parameter K2). The solution is illustrated in Figure 2.

- The time interval T is configured by the gNB considering below time components

  - the gNB process time of a received PUSCH

  - the transmission time of a dynamic grant

  - the delay between UL grant reception in DL (PDCCH) and UL data (PUSCH) transmission (i.e., configured via the delay parameter K2.

Figure 2: The UE skips processing UL dynamic grants during the time interval T after the recent PUSCH transmission.

We give below proposals accordingly.

For one HARQ process which is configured by the gNB for configured uplink grant, the UE skips processing UL dynamic grants which are valid for transmissions during the time interval T after a recent autonomous PUSCH transmission on that HARQ process.

# Conclusion

In section 2 we made the following observations:

Observation 1	When a configured grant is configured, the HARQ process that the UE will transmit on a certain slot is not known by the gNB.

Observation 2	It can happen that the gNB provides a DCI with UL grant for a HARQ process just a short time before or after the slot in which the UE intends to perform a transmission with a configured grant for the same HARQ process.

Observation 3	Performing both a transmission with a configured grant and a scheduled UL transmission for the same HARQ process very close in time increases overhead which is not desirable, especially in the unlicensed spectrum.

Observation 4	The time interval T is configured by the gNB considering below time components

a.	the gNB process time of a received PUSCH

b.	the transmission time of a dynamic grant

c.	the delay between UL grant reception in DL (PDCCH) and UL data (PUSCH) transmission (i.e., configured via the delay parameter K2.

Based on the discussion in section 2 we propose the following:

Proposal 1	As in NR licensed, the UE prioritizes dynamically scheduled UL transmission when that occurs in the same slot as a configured UL grant, that is, no spec change.

Proposal 2	For one HARQ process which is configured by the gNB for configured uplink grant, the UE skips processing UL dynamic grants which are valid for transmissions during the time interval T after a recent autonomous PUSCH transmission on that HARQ process.

# References

- R2-1814059, Configured UL grant enhancement for NR Unlicensed, Intel Corporation, 3GPP TSG-RAN WG2 103bis.