3GPP TSG-RAN2 Meeting # 107	R2-19xxxxx

# Prague, Czech Republic, 26th - 30th Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1248 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR for introduction of even further mobility enhancement in E-UTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2019-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of even further mobility enhancement in E-UTRAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of even further mobility enhancement in E-UTRAN. This CR captures the stage-2 description of feMob and it is based on RAN2 and RAN3 agreements made so far. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, features of non-split bearer (TBC) handover and conditional handover will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | [To be added if the changes are stable] |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the following terms and definitions apply.

Access Control: the process that checks whether a UE is allowed to access and to be granted services in a closed cell.

Aerial UE communication: functionality enabling Aerial UE function as defined in 23.17.

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Carrier frequency: center frequency of the cell.

Cell: combination of downlink and optionally uplink resources. The linking between the carrier frequency of the downlink resources and the carrier frequency of the uplink resources is indicated in the system information transmitted on the downlink resources.

Cell Group: in dual connectivity, a group of serving cells associated with either the MeNB or the SeNB.

Conditional Handover:   A handover procedure that is executed only when the execution condition(s) is met.

CHO configuration: Configuration of candidate cell(s) and execution condition(s) that are used for determining when/whether the CHO is executed.

CHO candidate cell: A candidate cell for CHO, for which UE has been configured with a CHO configuration.

Control plane CIoT EPS optimization: Enables support of efficient transport of user data (IP, non-IP or SMS) over control plane via the MME without triggering data radio bearer establishment, as defined in TS 24.301 [20]. In the context of this specification, a NB-IoT UE that only supports Control plane CIoT EPS optimization is a UE that does not support User plane CIoT EPS optimization and S1-U data transfer but may support other CIoT EPS optimizations.

CSG Cell: a cell broadcasting a CSG indicator set to true and a specific CSG identity.

CSG ID Validation: the process that checks whether the CSG ID received via handover messages is the same as the one broadcast by the target E-UTRAN.

CSG member cell: a cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN and for which the CSG whitelist of the UE includes an entry comprising cell's CSG ID and the respective PLMN identity.

DCN-ID: DCN identity identifies a specific dedicated core network (DCN).

Dual Connectivity: mode of operation of a UE in RRC_CONNECTED, configured with a Master Cell Group and a Secondary Cell Group.

En-gNB: as defined in TS 37.340 [76].

E-RAB: an E-RAB uniquely identifies the concatenation of an S1 Bearer and the corresponding Data Radio Bearer. When an E-RAB exists, there is a one-to-one mapping between this E-RAB and an EPS bearer of the Non Access Stratum as defined in [17].

Frequency layer: set of cells with the same carrier frequency.

FeMBMS: further enhanced multimedia broadcast multicast service.

Handover: procedure that changes the serving cell of a UE in RRC_CONNECTED.

Hybrid cell: a cell broadcasting a CSG indicator set to false and a specific CSG identity. This cell is accessible as a CSG cell by UEs which are members of the CSG and as a normal cell by all other UEs.

Local Home Network: as defined in TS 23.401 [17].

LTE bearer: in LTE-WLAN Aggregation, a bearer whose radio protocols are located in the eNB only to use eNB radio resources only.

LWA bearer: in LTE-WLAN Aggregation, a bearer whose radio protocols are located in both the eNB and the WLAN to use both eNB and WLAN resources.

LWAAP PDU: in LTE-WLAN Aggregation, a PDU with DRB ID generated by LWAAP entity for transmission over WLAN.

Make-Before-Break HO/SeNB change: maintaining source eNB/SeNB connection after reception of RRC message for handover or change of SeNB before the initial uplink transmission to the target eNB during handover or change of SeNB.

Master Cell Group: in dual connectivity, a group of serving cells associated with the MeNB, comprising of the PCell and optionally one or more SCells.

Master eNB: in dual connectivity, the eNB which terminates at least S1-MME.

MBMS-dedicated cell: cell dedicated to MBMS transmission.

MBMS/Unicast-mixed cell: cell supporting both unicast and MBMS transmissions.

FeMBMS/Unicast-mixed cell: cell supporting MBMS transmission and unicast transmission as SCell.

MCG bearer: in dual connectivity, a bearer whose radio protocols are only located in the MeNB to use MeNB resources only.

Membership Verification: the process that checks whether a UE is a member or non-member of a hybrid cell.

Multi-Connectivity: Mode of operation whereby a multiple Rx/Tx UE in the connected mode is configured to utilise radio resources amongst E-UTRA and/or NR provided by multiple distinct schedulers connected via non-ideal backhaul.

NB-IoT: NB-IoT allows access to network services via E-UTRA with a channel bandwidth limited to 200 kHz.

NB-IoT UE: a UE that uses NB-IoT.

Non-anchor carrier: in NB-IoT, a carrier where the UE does not assume that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

[Non-split bearer HO]: UE maintaining connection with both source eNB and target eNB after reception of RRC message for handover.

Editor’s note: FFS on the term to be used for non-split bearer solution. Candidate opotions include “Enhanced Make-Before-Break HO”, “simultaneous connectivity handover” and “Non-split beaer handover”.

NR: NR radio access.

PLMN ID Check: the process that checks whether a PLMN ID is the RPLMN identity or an EPLMN identity of the UE.

Power saving mode: mode configured and controlled by NAS that allows the UE to reduce its power consumption, as defined in TS 24.301 [20], TS 23.401 [17], TS 23.682 [57].

Primary PUCCH group: a group of serving cells including PCell whose PUCCH signalling is associated with the PUCCH on PCell.

Primary Timing Advance Group: Timing Advance Group containing the PCell. In this specification, Primary Timing Advance Group refers also to Timing Advance Group containing the PSCell unless explicitly stated otherwise.

ProSe-enabled Public Safety UE: a UE that the HPLMN has configured to be authorized for Public Safety use, and which is ProSe-enabled and supports ProSe procedures and capabilities specific to Public Safety. The UE may, but need not, have a USIM with one of the special access classes {12, 13, 14}.

ProSe Per-Packet Priority: a scalar value associated with a protocol data unit that defines the priority handling to be applied for transmission of that protocol data unit.

ProSe UE-to-Network Relay: a UE that provides functionality to support connectivity to the network for Remote UE(s).

ProSe UE-to-Network Relay Selection: Process of identifying a potential ProSe UE-to Network Relay, which can be used for connectivity services (e.g. to communicate with a PDN).

ProSe UE-to-Network Relay Reselection: process of changing previously selected ProSe UE-to-Network Relay and identifying potential a new ProSe UE-to-Network Relay, which can be be used for connectivity services (e.g. to communicate with PDN).

Public Safety ProSe Carrier: carrier frequency for public safety sidelink communication and public safety sidelink discovery.

PUCCH group: either primary PUCCH group or a secondary PUCCH group.

PUCCH SCell: a Secondary Cell configured with PUCCH.

<----------------------------------------------------Unmodified part omitted------------------------------------------------->

3.2	Abbreviations

<----------------------------------------------------Unmodified part omitted------------------------------------------------->

C/I	Carrier-to-Interference Power Ratio

CA	Carrier Aggregation

CAZAC	Constant Amplitude Zero Auto-Correlation

CBC	Cell Broadcast Center

CC	Component Carrier

CG	Cell Group

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CMAS	Commercial Mobile Alert Service

CMC	Connection Mobility Control

C-plane	Control Plane

C-RNTI	Cell RNTI

CoMP	Coordinated Multi Point

CP	Cyclic Prefix

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

CRE	Cell Range Extension

CRS	Cell-specific Reference Signal

CSA	Common Subframe Allocation

CSG	Closed Subscriber Group

CSI	Channel State Information

CSI-IM	CSI interference measurement

CSI-RS	CSI reference signal

DC	Dual Connectivity

<----------------------------------------------------Unmodified part omitted------------------------------------------------->

Start of next changes

10.1.2	Mobility Management in ECM-CONNECTED/CM-CONNECTED

The Intra-E-UTRAN-Access Mobility Support for UEs in ECM-CONNECTED/CM-CONNECTED handles all necessary steps for

-	Handover procedures, like processes that precede the final HO decision on the source network side (control and evaluation of UE and eNB measurements taking into account certain UE specific roaming and access restrictions), preparation of resources on the target network side, providing CHO configuration, commanding the UE to the new radio resources and finally releasing resources on the (old) source network side. It contains mechanisms to transfer context data between evolved nodes, and to update node relations on C-plane and U-plane.

-	DC specific procedures, like processes that precede the final decision for a certain configuration of a SeNB (control and evaluation of UE and network side measurements), preparation of respective resources on the network side of a SeNB, commanding the UE to the new radio resources configuration for a second connection and, if applicable, finally releasing resources of a SeNB. It contains mechanisms to transfer UE- and bearer-context data between involved nodes, and to update node relations on C-plane and U-plane.

In E-UTRAN RRC_CONNECTED state, network-controlled UE-assisted handovers and DC specific activities are performed and various DRX cycles are supported.

The UE makes measurements of attributes of the serving and neighbour cells to enable the process:

-	There is no need to indicate neighbouring cells to enable the UE to search and measure a cell i.e. E-UTRAN relies on the UE to detect the neighbouring cells;

-	For the search and measurement of inter-frequency neighbouring cells, at least the carrier frequencies need to be indicated;

-	The E-UTRAN signals reporting criteria for event-triggered and periodical reporting;

-	An NCL can be provided by the serving cell by RRC dedicated signalling to handle specific cases for intra- and inter-frequency neighbouring cells. This NCL contains cell specific measurement parameters (e.g. cell specific offset) for specific neighbouring cells;

-	Black lists can be provided to prevent the UE from measuring specific neighbouring cells.

For the UE measuring discovery signals (i.e. CRS and/or CSI-RS) of the serving and neighbour cells, the E-UTRAN indicates the measurement configuration to the UE, including the measurement timing configuration of the discovery signals.

Depending on whether the UE needs transmission/reception gaps to perform the relevant measurements, measurements are classified as gap assisted or non-gap assisted. A non-gap assisted measurement is a measurement on a cell that does not require transmission/reception gaps to allow the measurement to be performed. A gap assisted measurement is a measurement on a cell that does require transmission/reception gaps to allow the measurement to be performed. Gap patterns (as opposed to individual gaps) are configured and activated by RRC.

For non-split bearer handover, the following principles are applied:

-	Non-split bearerhandover can be executed only if the UE supports Dual Active Protocol Stack

-	A separate security key is used for each of the two active protocol stacks.

-	ROHC and remaining PDCP functions (e.g. ciphering, PDCP header addition and PDCP PDU creation) are executed separately at source eNB and target eNB. PDCP SN assignment (for DL) is done at source eNB. PDCP SDUs and the SN assigned to each SDU are then forwarded to target eNB.

Editor’s note: FFS how/whether we will specify the rules for UE when capability coordination is not utilized and UE capabilities are exceeded (we may leave this up to UE implementation).

In the text and figure(s) in the following clauses, intra-E-UTRA HO description is applicable for both intra-EPC and intra-5GC cases. In addition, the following differences are applicable for intra-5GC:

-	ng-eNB should be considered instead of eNB;

-	5GC should be considered instead of EPC, and NG interface should be considered instead of S1 interface;

-	Xn interface should be considered instead of X2 interface and the messages sent between ng-eNBs over Xn are defined in TS 38.423 [86];

-	AMF should be considered intead of MME, and UPF should be considered instead of Serving Gateway;

-	PDU session information should be considered instead of E-RAB QoS, and the QoS flow to DRB mapping rules applied to the UE should be forwarded to the target ng-eNB;

-	For the messages sent between MME and Serving Gateway, and between MME and eNB, use AMF/UPF/ng-eNB respectively;

-	The data forwarding defined in clause 9.2.3.2.3 in TS 38.300 [79] should be applied instead of clause 10.1.2.3;

-	The Dual Connectivity operation in clause 10.1.2.8 is not applicable to intra-5GC mobility. The corresponding Dual Connectivity operations for 5GC are described in TS 37.340 [76].

Editor’s note: FFS on the support of non-split bearer handover (TBC) and conditional handover in LTE/5GC.

10.1.2.1	Handover

The intra E-UTRAN HO of a UE in RRC_CONNECTED state is a UE-assisted network-controlled HO, with HO preparation signalling in E-UTRAN:

-	Part of the HO command comes from the target eNB and is transparently forwarded to the UE by the source eNB;

-	To prepare the HO, the source eNB passes all necessary information to the target eNB (e.g. E-RAB attributes and RRC context):

-	When CA is configured and to enable SCell selection in the target eNB, the source eNB can provide in decreasing order of radio quality a list of the best cells and optionally measurement result of the cells.

-	When DC is configured, the source MeNB provides the SCG configuration (in addition to the MCG configuration) to the target MeNB.

-	Both the source eNB and UE keep some context (e.g. C-RNTI) to enable the return of the UE in case of HO failure;

-	If RACH-less HO is not configured, the UE accesses the target cell via RACH following a contention-free procedure using a dedicated RACH preamble or following a contention-based procedure if dedicated RACH preambles are not available:

-	the UE uses the dedicated preamble until the handover procedure is finished (successfully or unsuccessfully);

-	If RACH-less HO is configured, the UE accesses the target cell via the uplink grant preallocated to the UE in the RRC message. If the UE does not receive the preallocated uplink grant in the RRC message from the source eNB, the UE monitors the PDCCH of the target cell;

-	If the access towards the target cell (using RACH or RACH-less procedure) is not successful within a certain time, the UE initiates radio link failure recovery using a suitable cell;

-	No ROHC context is transferred at handover;

-	No UDC context is transferred at handover;

-	ROHC context can be kept at handover within the same eNB.

10.1.2.1.1	C-plane handling

The preparation and execution phase of the HO procedure is performed without EPC involvement, i.e. preparation messages are directly exchanged between the eNBs. The release of the resources at the source side during the HO completion phase is triggered by the eNB. In case an RN is involved, its DeNB relays the appropriate S1 messages between the RN and the MME (S1-based handover) and X2 messages between the RN and target eNB (X2-based handover); the DeNB is explicitly aware of a UE attached to the RN due to the S1 proxy and X2 proxy functionality (see clause 4.7.6.6). The figure below depicts the basic handover scenario where neither MME nor Serving Gateway changes:

# Figure 10.1.2.1.1-1: Intra-MME/Serving Gateway HO

Below is a more detailed description of the intra-MME/Serving Gateway HO procedure:

0	The UE context within the source eNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

1	The source eNB configures the UE measurement procedures according to the roaming and access restriction information and e.g. the available multiple frequency band information. Measurements provided by the source eNB may assist the function controlling the UE's connection mobility.

2	A MEASUREMENT REPORT is triggered and sent to the eNB.

3	The source eNB makes decision based on MEASUREMENT REPORT and RRM information to hand off the UE.

4	The source eNB issues a HANDOVER REQUEST message to the target eNB passing necessary information to prepare the HO at the target side (UE X2 signalling context reference at source eNB, UE S1 EPC signalling context reference, target cell ID, KeNB*, RRC context including the C-RNTI of the UE in the source eNB, AS-configuration, E-RAB context and physical layer ID of the source cell + short MAC-I for possible RLF recovery). UE X2 / UE S1 signalling references enable the target eNB to address the source eNB and the EPC. The E-RAB context includes necessary RNL and TNL addressing information, and QoS profiles of the E-RABs.

5	Admission Control may be performed by the target eNB dependent on the received E-RAB QoS information to increase the likelihood of a successful HO, if the resources can be granted by target eNB. The target eNB configures the required resources according to the received E-RAB QoS information and reserves a C-RNTI and optionally a RACH preamble. The AS-configuration to be used in the target cell can either be specified independently (i.e. an "establishment") or as a delta compared to the AS-configuration used in the source cell (i.e. a "reconfiguration").

6	The target eNB prepares HO with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source eNB. The HANDOVER REQUEST ACKNOWLEDGE message includes a transparent container to be sent to the UE as an RRC message to perform the handover. The container includes a new C-RNTI, target eNB security algorithm identifiers for the selected security algorithms, may include a dedicated RACH preamble, and possibly some other parameters i.e. access parameters, SIBs, etc. If RACH-less HO is configured, the container includes timing adjustment indication and optionally a preallocated uplink grant. The HANDOVER REQUEST ACKNOWLEDGE message may also include RNL/TNL information for the forwarding tunnels, if necessary.

NOTE:	As soon as the source eNB receives the HANDOVER REQUEST ACKNOWLEDGE, or as soon as the transmission of the handover command is initiated in the downlink, data forwarding may be initiated.

Steps 7 to 16 provide means to avoid data loss during HO and are further detailed in 10.1.2.1.2 and 10.1.2.3.

7	The target eNB generates the RRC message to perform the handover, i.e. RRCConnectionReconfiguration message including the mobilityControlInformation, to be sent by the source eNB towards the UE. The source eNB performs the necessary integrity protection and ciphering of the message.

The UE receives the RRCConnectionReconfiguration message with necessary parameters (i.e. new C-RNTI, target eNB security algorithm identifiers, and optionally dedicated RACH preamble, target eNB SIBs, etc.) and is commanded by the source eNB to perform the HO. If RACH-less HO is configured, the RRCConnectionReconfiguration includes timing adjustment indication and optionally preallocated uplink grant for accessing the target eNB. If preallocated uplink grant is not included, the UE should monitor PDCCH of the target eNB to receive an uplink grant. The UE does not need to delay the handover execution for delivering the HARQ/ARQ responses to source eNB.

If Make-Before-Break HO is configured, the connection to the source cell is maintained after the reception of RRCConnectionReconfiguration message with mobilityControlInformation before the UE executes initial uplink transmission to the target cell.

NOTE:	If Make-Before-Break HO is configured, the source eNB decides when to stop transmitting to the UE.

NOTE:	The UE can be configured with Make-Before-Break HO and RACH-less HO simultaneously.

Editor’s note: For non-split bearer handover (TBC), this section may need to be updated and FFS on the updates.

8	The source eNB sends the SN STATUS TRANSFER message to the target eNB to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status of E-RABs for which PDCP status preservation applies (i.e. for RLC AM). The uplink PDCP SN receiver status includes at least the PDCP SN of the first missing UL SDU and may include a bit map of the receive status of the out of sequence UL SDUs that the UE needs to retransmit in the target cell, if there are any such SDUs. The downlink PDCP SN transmitter status indicates the next PDCP SN that the target eNB shall assign to new SDUs, not having a PDCP SN yet. The source eNB may omit sending this message if none of the E-RABs of the UE shall be treated with PDCP status preservation.

9	If RACH-less HO is not configured, after receiving the RRCConnectionReconfiguration message including the mobilityControlInformation , UE performs synchronisation to target eNB and accesses the target cell via RACH, following a contention-free procedure if a dedicated RACH preamble was indicated in the mobilityControlInformation, or following a contention-based procedure if no dedicated preamble was indicated. UE derives target eNB specific keys and configures the selected security algorithms to be used in the target cell.

If RACH-less HO is configured, UE performs synchronisation to target eNB. UE derives target eNB specific keys and configures the selected security algorithms to be used in the target cell.

10	If RACH-less HO is not configured, the target eNB responds with UL allocation and timing advance.

10a If RACH-less HO is configured and the UE did not get the periodic pre-allocated uplink grant in the RRCConnectionReconfiguration message including the mobilityControlInfo, the UE receives uplink grant via the PDCCH of the target cell. The UE uses the first available uplink grant after synchronization to the target cell.

11	When the RACH-less HO is not configured and the UE has successfully accessed the target cell, the UE sends the RRCConnectionReconfigurationComplete message (C-RNTI) to confirm the handover, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the target eNB, which indicates that the handover procedure is completed for the UE. The target eNB verifies the C-RNTI sent in the RRCConnectionReconfigurationComplete message. The target eNB can now begin sending data to the UE.

When the RACH-less HO is configured, after the UE has received uplink grant, the UE sends the RRCConnectionReconfigurationComplete message (C-RNTI) to confirm the handover, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the target eNB. The target eNB verifies the C-RNTI sent in the RRCConnectionReconfigurationComplete message. The target eNB can now begin sending data to the UE. The handover procedure is completed for the UE when the UE receives the UE contention resolution identity MAC control element from the target eNB.

12	The target eNB sends a PATH SWITCH REQUEST message to MME to inform that the UE has changed cell.

13	The MME sends a MODIFY BEARER REQUEST message to the Serving Gateway.

14	The Serving Gateway switches the downlink data path to the target side. The Serving gateway sends one or more "end marker" packets on the old path to the source eNB and then can release any U-plane/TNL resources towards the source eNB.

15	The Serving Gateway sends a MODIFY BEARER RESPONSE message to MME.

16	The MME confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

17	By sending the UE CONTEXT RELEASE message, the target eNB informs success of HO to source eNB and triggers the release of resources by the source eNB. The target eNB sends this message after the PATH SWITCH REQUEST ACKNOWLEDGE message is received from the MME.

18	Upon reception of the UE CONTEXT RELEASE message, the source eNB can release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

When an X2 handover is used involving HeNBs and when the source HeNB is connected to a HeNB GW, a UE CONTEXT RELEASE REQUEST message including an explicit GW Context Release Indication is sent by the source HeNB, in order to indicate that the HeNB GW may release of all the resources related to the UE context.

Start of next changes

10.1.2.1a	Conditional Handover

Conditional Handover (CHO) is characterized by a configured execution condition that determines when/whether the corresponding HO command is executed. Upon receiving the CHO configuration, UE starts evaluating the condition(s) and executes the HO command once the condition is met. The following principles apply to CHO:

-	The CHO configuration contains the configuration of CHO candidate cell(s) and execution condition(s).

-	An execution condition is defined as measurement event like A3 and A5.

Editor’s note: FFS the detail of conditions might be modified or omitted according to progress of stage 3.

-	Reception of HO command (without conditional component), as described in clause 10.1.2.1 overrides any configured CHO configuration.

Editor’s note: FFS how CHO execution condition is handled.

-	The network can release a configured CHO configuration using RRC message.

Editor’s note: FFS whether CHO commands can be released in other ways.

10.1.2.1a.1	C-plane handling

# Figure 10.1.2.1a-1: Conditional Handover

1.	The source eNB configures the UE with measurement configuration, which may be used by UE to trigger Measurement Reports for potential CHO candidate cell(s).

2.	A MEASUREMENT REPORT is triggered and sent to the source eNB.

3.	The source eNB makes decision on the usage of CHO to handoff the UE based on MEASUREMENT REPORT information.

4.	The source eNB sends CHO request to the eNB(s) of candidate cell(s).

5.	Admission control on CHO may be performed by the eNB(s) of candidate cell(s).

6.  The eNB(s) of candidate cell(s) sends CHO response to the source eNB.

7. 	The source eNB sends a RRC message with CHO configuration(s) to the UE. The source eNB decides on the condition for the execution of CHO and adds the condition to the RRC message sent to UE. Multiple CHO candidate cell configurations can be sent in either one or multiple RRC messages. Deconfiguration of CHO candidate cells is performed by RRC signalling.

Editor’s note: FFS on signalling details. FFS how CHO execution is handled.

Editor’s note: FFS whether a RRC acknowledgement message to the source eNB is needed for CHO configuration.

8. 	UE maintains connection with source eNB after receiving CHO configuration, and starts evaluating the CHO execution condition(s) for the CHO candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source eNB without non-split bearer handover configured, applies corresponding configuration for that target cell and synchronises to the target eNB.

Editor’s note: FFS on the parallel support of non-split bearer handover (TBC) and conditional handover in LTE/5GC.

9-10. 	The UE accesses to the target eNB and replies with a RRC message.

11. Steps 12-18 as in Figure 10.1.2.1.1-1.

Editor’s note: FFS how to perform data forwarding.

Editor’s note: FFS whether the procedure can be combined with ‘simultaneous connectivity handover.

10.1.2.1a.2	U-plane handling

Editor’s note: FFS based on RAN3 decisions

End of changes

Annex: RAN2 agreements

Note: This sub-clause lists all RAN2 agreements on feMob (including FFSs). This sub-clause is only for information and it will be removed if the CR is to be agreed.

RAN2#106

The CR (R2-1908271) is endorsed as running stage-2 CR. RAN2 agreed to have email discussion every meeting to progress on the running CR(s). An email discussion [106#84] was allocated to capture updates based on agreements into the stage-2 CR.

Two email discussions [106 #41] and [106 #42] were allocated on CHO execution details and CHO configuration for both LTE and NR. One email discussion [106#40] was allocated on beam specific aspects of CHO only for NR.

For Reduction in user data interruption during handover:

Agreements

- Simultaneous UL PUSCH transmission does not need to be supported for the HO interruption solution.

2	UL PUSCH switches from source to target after reception of the first UL grant from the target eNB

Agreements

1	We will not specify single active protocol stack solution (option 0/1/2).

2	We will specify dual active with specified capability coordination that does not have to be utilized by the network. FFS how/whether we will specify the rules for UE when capability coordination is not utilized and UE capabilities are exceeded (we may leave this up to UE implementation).

For Handover robustness improvements:

Agreements

1	Conditional handover (CHO) is introduced in LTE to solve robustness/reliability issue.

2	The source cell decides on the condition for the execution of CHO.

3	The source cell adds the condition for the execution of CHO to the RRC message sent to UE.

4	Multiple CHO candidate cells can be sent in either one or multiple RRC messages. FFS on signalling details. FFS how CHO execution is handled.

5	CHO execution does not trigger measurement report.

6	A3/A5-like CHO execution condition shall be specified.

Agreements for LTE conditional HO (From NR mob discussions)

1	Separate CHO execution condition(s) can be configured for each individual candidate cells.

2	Define a CHO execution condition by the measurement identity which identifies a measurement configuration. (FFS to be addressed in stage 3 which parts of the measurement configuration are used for the CHO triggering).

3	As a baseline CHO can be triggered based on a condition consisting of a single event, singe quantity.

3.1	The single trigger quantity can be configured to be RSRP, RSRQ or RS-SINR.

FFS Whether multiple triggering conditions are required.

4	Deconfiguration of CHO candidates is performed by RRC signalling (we will not introduce timer based mechanism for the UE to deconfiguration of the CHO candidates).

5	Baseline that configuration of all CHO candidates are released after successful (any) handover completion (sending complete message to the target cell).

FFS if it might be possible to keep CHO candidates after the HO.

6	UE shall not stop T310 and shall not start T304 when it receives configuration of a CHO candidate

7	The timer T310 is stopped and timer T304-like is started when the UE begins execution of a conditional handover for a target cell. (Stage 3 detail whether we reuse T304 or define a new timer)

Working assumption:

8	At RLF the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed.

9	At legacy handover failure (T304 expiry) or failure to access a CHO candidate cell (T304-like expiry), the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed.

RAN2#105bis

For Reduction in user data interruption during handover:

=>	Any solution that is specified will be modelled as a single PDCP entity on UE side.

For Handover robustness improvements:

Agreements

1: 	The CHO command contains at least the configuration information of target cell(s) and triggering conditions.

=> FFS who decides the triggering conditions (source, target or source+target)

=> FFS on transparent containers.

=> FFS on the Stage-3 details

Agreements

1  Existing Ax measurement events can be used for executing CHO. FFS which Ax events can be used.

2  Conventional handover overrides any configured conditional handover command

3  The network can inform the UE to release CHO configurations (e.g. candidate cells) by RRC signaling.

=> FFS how “CHO cmd” is formulated in Stage-3 signalling

=> FFS whether UE continues to receive source cell while executing CHO cmd.

=> FFS what UE does if it receives HO cmd while executing CHO cmd.

=> FFS what UE does if NW removes CHO cmd while executing the same CHO cmd.

=> FFS whether UE stores CHO commands in failure cases

=> FFS whether CHO candidates can be released via other means.

RAN2#105

For Reduction in user data interruption during handover:

Agreements

1	Specify the ”non-split bearer” solution candidate for the Rel-16 E-UTRA enhancements minimizing the interruption time during mobility.

2	Decide during the work item phase whether a single active protocol stack or two active protocol stacks are used in enhanced Rel-16 E-UTRAN mobility solution.

3	Agree the following common aspects for “non-split bearer” solution candidate:

a.	PDCP SN assignment (for DL) is done at source eNB. PDCP SDUs and the SN assigned to each SDU are then forwarded to target eNB. Details of how SN information is transferred is FFS.

b.	RoHC and remaining PDCP functions (e.g. ciphering, PDCP PDU creation) are executed separately at each network node

c.	The UE procedure when UE detaches from the source cell is explicitly defined in the specifications (e.g. via procedural text and/or via dedicated message/indication.).

d.	In case of two active protocol stacks, a separate security key is used for each of the protocol stacks.

4	RAN2 is asked to work further on the details of the following open issues:

a.	When detaching from the source shall occur and whether it has to be separately considered from the UE’s and NW’s side

b.	Whether data forwarding is done “late” or “early”. Consider potential combination with CHO and how SN Status transfer is done and how HFN is handled.

c.	LS to RAN3 on data forwarding enhancements to enable reduced interruption time during HO

5	The detailed assumptions of simultaneous transmission/reception for the solutions depend on the feedback from RAN1 and RAN4 (i.e. response to R2-1815706). RAN2 shall continue working based on the received LS replies.

=>FFS how security asepcts are handled and whether changes to LTE baseline are needed.

=>FFS whether there is single active protocol stack or two simultaneously active protocol stacks

=>FFS how to detach from the source cell (seen from the NW’s side and UE’s side)

=>FFS How to do data forwarding (early/late, including handling the SN, security, CHO impact)

For Handover robustness improvements:

=>	FFS how to include the CHO conditions in UE configuration

Agreements

1: The baseline operation for E-UTRAN Conditional HO procedure assumes HO command type of message contains HO triggering condition(s) and dedicated RRC configuration(s). UE accesses the prepared target when the relevant condition is met.

3: The baseline operation for E-UTRAN Conditional HO assumes the source eNB remains responsible for RRC until UE successfully sends RRC Connection Reconfiguration Complete message to target eNB.

4: RAN2 assumes late packet forwarding (i.e. not done immediately when the CHO target cells become prepared) could be more suitable for E-UTRAN CHO when there are multiple candidate target cells. In case of single prepared candidate target cell, early packet forwarding could be considered as an option. Detailed decisions require RAN3 study.

5: RAN2 will inform the Conditional HO assumptions (including the baseline operation) to RAN3 via LS at RAN#105bis, requesting RAN3 to kindly work on the CHO scheme aspects matching their expertise (e.g. data forwarding).

RAN2#104

For Reduction in user data interruption during handover:

Agreements

1	We aim to consider some form of multiple connectivity during study phase depending on RAN4 reply on applicability to different scenarios.

For Handover robustness improvements:

Agreements

1	RAN2 will consider a conditional handover: This is defined as UE having network configuration for initiating access to a target cell based on configured condition(s).

2	Usage of conditional handover is decided by network. UE evaluates when the condition is valid.

=>	FFS on the exact details of the procedures

Agreements

1	Support configuration of one or more candidate cells for conditional handover.

=>	FFS how many candidate cells (UE and network impacts should be clarified).

Others:

Agreements

1	We will prioritize solutions for LTE/EPC in this WID. Can discuss LTE/5GC support based on Stage-3 details.

2	Do not consider solutions for handover between LTE/EPC and LTE/5GC.

RAN2#103bis

Organizational:

Agreements

1	RAN2 minutes will capture the agreements (as normal) – we will not have a TR-like document.

=> FFS on 5GC applicability in this WID. Contributions are invited to next meeting.

Agreements

1	Solution proposals should consider at least the following metrics:

-	Mobility robustness

- 	Interruption time

2	Other aspects should also be considered, e.g.

-	Applicable deployment scenarios

-	Signalling overhead

-	Specification effort

-	UE/network complexity

For Reduction in user data interruption during handover:

=>	Use the protocol stack comparison in this contribution as baseline for further discussions between the split bearer and non-split bearer solutions.

=>	We should discuss the security key aspects more when we discuss the details of the solutions.

=>	Consider how to do reordering in non-split case

=>	FFS whether single or dual RRC (and e.g. whether we have 1 or 2 S1-C connections) is considered (S1-C would affect also RAN3)

=>	FFS how duplication is considered (depending on solution details)

For Handover robustness improvements:

Agreements

1 	Evaluate new solutions compared to LTE Re-15 mechanisms.