3GPP TSG-WG2 Meeting #107	R2-1911591

# Prague, Czech Republic, 26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | CRnum | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of additional enhancements for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2019/09/30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of additional enhancements for NB-IoT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of additional enhancements for NB-IoT - SON: - Reporting of Cell Global Identity and strongest measured cell(s) (ANR) - SON: Reporting of Random access performance - SON: Reporting of Radio link failure (RLF) - Improved multi-carrier operation: - DL channel quality reporting in MSG3 - DL channel quality reporting in connected mode - Mobility enhancements: Assistance information for idle mode inter-RAT cell selection for NB-IoT to and from LTE, LTE-MTC and GERAN - Connection to 5GC Change ‘CIOT EPS optimization’ to ‘CIoT EPS Optimisation’ for alignment with other specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The additional enhancements for NB-IoT are not captured in TS 36.300 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 4.1, 4.3.2, 4.10, 6.0, 6.1, 6.1.2.0, 6.1.2.2, 6.3.1, 7.1, 7.2, 7.3, 7.3a.1, 7.3a.2, 7.3a.2, 7.3b, 7.4, 10.0, 10.1.4, 10.1.5.0, 10.1.6, 10.2.x (new), 11.0, 11.x, 14.1, 14.3.2, 18, 19.2.2.2.1, 19.2.2.25, 19.2.2.30, 19.2.2.31, 22.3.2a, 22.3.4x (new), 22.4.2.x (new), 22.4.3, 22.4.5, 24.1, 24.2.1, 24.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-1910160, RAN2#106, Initial version |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 25.913: "Requirements for Evolved UTRA (E-UTRA) and Evolved UTRAN (E-UTRAN)".

[3]	3GPP TS 36.201: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; General description".

[4]	3GPP TS 36.211:"Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[5]	3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[6]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[7]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; Measurements".

[8]	IETF RFC 4960 (09/2007): "Stream Control Transmission Protocol".

[9]	3GPP TS 36.302: "Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer".

[10]	Void

[11]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[12]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[13]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[14]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification".

[15]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[16]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[17]	3GPP TS 23.401: "Technical Specification Group Services and System Aspects; GPRS enhancements for E-UTRAN access".

[18]	3GPP TR 24.801: "3GPP System Architecture Evolution (SAE); CT WG1 aspects".

[19]	3GPP TS 23.402: "3GPP System Architecture Evolution: Architecture Enhancements for non-3GPP accesses".

[20]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[21]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); "Requirements for support of radio resource management".

[22]	3GPP TS 33.401: "3GPP System Architecture Evolution: Security Architecture".

[23]	3GPP TS 23.272: "Circuit Switched Fallback in Evolved Packet System; Stage 2".

[24]	Void.

[25]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[26]	3GPP TS 23.003: "Numbering, addressing and identification".

[27]	3GPP TR 25.922: "Radio Resource Management Strategies".

[28]	3GPP TS 23.216: "Single Radio voice Call continuity (SRVCC); Stage 2".

[29]	3GPP TS 32.421: "Subscriber and equipment trace: Trace concepts and requirements".

[30]	3GPP TS 32.422: "Subscriber and equipment trace; Trace control and configuration management".

[31]	3GPP TS 32.423: "Subscriber and equipment trace: Trace data definition and management".

[32]	Void.

[33]	3GPP TS 22.220: "Service Requirements for Home NodeBs and Home eNodeBs".

[34]	3GPP TS 22.268: "Public Warning System (PWS) Requirements".

[35]	IETF RFC 3168 (09/2001): "The Addition of Explicit Congestion Notification (ECN) to IP".

[36]	3GPP TS 25.446: "MBMS synchronisation protocol (SYNC)".

[37]	3GPP TS 22.168: "Earthquake and Tsunami Warning System (ETWS) requirements; Stage 1".

[38]	Void.

[39]	Void.

[40]	3GPP TS 29.274: "Tunnelling Protocol for Control Plane (GTPv2-C); Stage 3".

[41]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[42]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[43]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[44]	3GPP TS 36.443: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); M2 Application Protocol (M2AP)".

[45]	3GPP TS 36.444: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); M3 Application Protocol (M3AP)".

[46]	3GPP TS 36.420: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 general aspects and principles".

[47]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)"

[48]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description"

[49]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs"

[50]	3GPP TR 36.816: "Evolved Universal Terrestrial Radio Access (E-UTRA); Study on signalling and procedure for interference avoidance for in-device coexistence".

[51]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specifications of User Equipment (UE) positioning in E-UTRAN".

[52]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[53]	3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

[54]	3GPP TS 23.251: "Technical Specification Group Services and System Aspects; Network Sharing; Architecture and functional description".

[55]	3GPP TS 23.139: "3GPP system – fixed broadband access network interworking".

[56]	3GPP TS 23.007: "Technical Specification Group Core Network and Terminals; Restoration procedures".

[57]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[58]	3GPP TS 24.312: "Access Network Discovery and Selection Function (ANDSF) Management Object (MO)".

[59]	3GPP TR 36.842: "Study on Small Cell enhancements for E-UTRA and E-UTRAN; Higher layer aspects"

[60]	3GPP TR 36.932: "Scenarios and Requirements for Small Cell Enhancements for E-UTRA and E-UTRAN".

[61]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[62]	3GPP TS 23.303: "Technical Specification Group Services and System Aspects; Proximity-based services (ProSe)"

[63]	3GPP TS 36.314: "Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 - Measurements".

[64]	3GPP TR 36.889: "Study on Licensed-Assisted Access to Unlicensed Spectrum".

[65]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.".

[66]	3GPP TS 36.360: "LTE-WLAN Aggregation Adaptation Protocol (LWAAP) specification".

[67]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks".

[68]	3GPP TS 36.361: "LTE/WLAN Radio Level Integration Using IPsec Tunnel (LWIP) encapsulation; Protocol specification".

[69]	3GPP TS 36.463: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless LAN (WLAN); Xw application protocol (XwAP)".

[70]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[71]	3GPP TS 22.185: "Service requirements for V2X services; Stage 1".

[72]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[73]	IETF RFC 7567 "IETF Recommendations Regarding Active Queue Management".

[74]	3GPP TS 26.114: "Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[75]	3GPP TS 24.386: "User Equipment (UE) to V2X control function; protocol aspects; Stage 3".

[76]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity".

[77]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[78]	3GPP TS 36.355: " Evolved Universal Terrestrial Radio Access (E-UTRA);LTE Positioning Protocol (LPP)".

[79]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description, Stage 2".

[80]	3GPP TS 37.324: "NR; Service Data Protocol (SDAP) specification".

[81]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[82]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[83]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[84]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[85]	3GPP TS 25.412: "UTRAN Iu interface signalling transport".

[86]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[87]	Void

[88]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[89]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[90]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access".

[xx]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3"

Next change

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply.

Access Control: the process that checks whether a UE is allowed to access and to be granted services in a closed cell.

Aerial UE communication: functionality enabling Aerial UE function as defined in 23.17.

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Carrier frequency: center frequency of the cell.

Cell: combination of downlink and optionally uplink resources. The linking between the carrier frequency of the downlink resources and the carrier frequency of the uplink resources is indicated in the system information transmitted on the downlink resources.

Cell Group: in dual connectivity, a group of serving cells associated with either the MeNB or the SeNB.

Control plane CIoT 5GS Optimisation: Enables exchange of efficient transport of user data (IP, Ethernet and Unstructured) or SMS messages over control plane via the AMF without triggering user-plane resource establishment, as defined in TS 24.501 [xx]. In the context of this specification, a NB-IoT UE that only supports Control plane CIoT 5GS Optimisation is a UE that does not support User plane CIoT 5GS Optimisation and NG-U data transfer but may support other CIoT 5GS Optimisations.

Control plane CIoT EPS Optimisation: Enables support of efficient transport of user data (IP, non-IP or SMS) over control plane via the MME without triggering data radio bearer establishment, as defined in TS 24.301 [20]. In the context of this specification, a NB-IoT UE that only supports Control plane CIoT EPS Optimisation is a UE that does not support User plane CIoT EPS Optimisation and S1-U data transfer but may support other CIoT EPS Optimisations.

CSG Cell: a cell broadcasting a CSG indicator set to true and a specific CSG identity.

CSG ID Validation: the process that checks whether the CSG ID received via handover messages is the same as the one broadcast by the target E-UTRAN.

CSG member cell: a cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN and for which the CSG whitelist of the UE includes an entry comprising cell's CSG ID and the respective PLMN identity.

DCN-ID: DCN identity identifies a specific dedicated core network (DCN).

Dual Connectivity: mode of operation of a UE in RRC_CONNECTED, configured with a Master Cell Group and a Secondary Cell Group.

en-gNB: as defined in TS 37.340 [76].

E-RAB: an E-RAB uniquely identifies the concatenation of an S1 Bearer and the corresponding Data Radio Bearer. When an E-RAB exists, there is a one-to-one mapping between this E-RAB and an EPS bearer of the Non Access Stratum as defined in [17].

Frequency layer: set of cells with the same carrier frequency.

FeMBMS: further enhanced multimedia broadcast multicast service.

FeMBMS/Unicast-mixed cell: cell supporting MBMS transmission and unicast transmission as SCell.

Handover: procedure that changes the serving cell of a UE in RRC_CONNECTED.

Hybrid cell: a cell broadcasting a CSG indicator set to false and a specific CSG identity. This cell is accessible as a CSG cell by UEs which are members of the CSG and as a normal cell by all other UEs.

Local Home Network: as defined in TS 23.401 [17].

LTE bearer: in LTE-WLAN Aggregation, a bearer whose radio protocols are located in the eNB only to use eNB radio resources only.

LWA bearer: in LTE-WLAN Aggregation, a bearer whose radio protocols are located in both the eNB and the WLAN to use both eNB and WLAN resources.

LWAAP PDU: in LTE-WLAN Aggregation, a PDU with DRB ID generated by LWAAP entity for transmission over WLAN.

Make-Before-Break HO/SeNB change: maintaining source eNB/SeNB connection after reception of RRC message for handover or change of SeNB before the initial uplink transmission to the target eNB during handover or change of SeNB.

Master Cell Group: in dual connectivity, a group of serving cells associated with the MeNB, comprising of the PCell and optionally one or more SCells.

Master eNB: in dual connectivity, the eNB which terminates at least S1-MME.

MBMS-dedicated cell: cell dedicated to MBMS transmission.

MBMS/Unicast-mixed cell: cell supporting both unicast and MBMS transmissions.

MCG bearer: in dual connectivity, a bearer whose radio protocols are only located in the MeNB to use MeNB resources only.

Membership Verification: the process that checks whether a UE is a member or non-member of a hybrid cell.

Multi-Connectivity: Mode of operation whereby a multiple Rx/Tx UE in the connected mode is configured to utilise radio resources amongst E-UTRA and/or NR provided by multiple distinct schedulers connected via non-ideal backhaul.

NB-IoT: NB-IoT allows access to network services via E-UTRA with a channel bandwidth limited to 200 kHz.

NB-IoT UE: a UE that uses NB-IoT.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

Non-anchor carrier: in NB-IoT, a carrier where the UE does not assume that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

NR: NR radio access

PLMN ID Check: the process that checks whether a PLMN ID is the RPLMN identity or an EPLMN identity of the UE.

Power saving mode: mode configured and controlled by NAS that allows the UE to reduce its power consumption, as defined in TS 24.301 [20], TS 23.401 [17], TS 23.682 [57].

Primary PUCCH group: a group of serving cells including PCell whose PUCCH signalling is associated with the PUCCH on PCell.

Primary Timing Advance Group: Timing Advance Group containing the PCell. In this specification, Primary Timing Advance Group refers also to Timing Advance Group containing the PSCell unless explicitly stated otherwise.

ProSe-enabled Public Safety UE: a UE that the HPLMN has configured to be authorized for Public Safety use, and which is ProSe-enabled and supports ProSe procedures and capabilities specific to Public Safety. The UE may, but need not, have a USIM with one of the special access classes {12, 13, 14}.

ProSe Per-Packet Priority: a scalar value associated with a protocol data unit that defines the priority handling to be applied for transmission of that protocol data unit.

ProSe UE-to-Network Relay: a UE that provides functionality to support connectivity to the network for Remote UE(s).

ProSe UE-to-Network Relay Selection: Process of identifying a potential ProSe UE-to Network Relay, which can be used for connectivity services (e.g. to communicate with a PDN).

ProSe UE-to-Network Relay Reselection: process of changing previously selected ProSe UE-to-Network Relay and identifying potential a new ProSe UE-to-Network Relay, which can be be used for connectivity services (e.g. to communicate with PDN).

Public Safety ProSe Carrier: carrier frequency for public safety sidelink communication and public safety sidelink discovery.

PUCCH group: either primary PUCCH group or a secondary PUCCH group.

PUCCH SCell: a Secondary Cell configured with PUCCH.

RACH-less HO/SeNB change: skipping random access procedure during handover or change of SeNB.

Receive Only Mode: See TS 23.246 [48].

Remote UE: a ProSe-enabled Public Safety UE, that communicates with a PDN via a ProSe UE-to-Network Relay.

SCG bearer: in dual connectivity, a bearer whose radio protocols are only located in the SeNB to use SeNB resources.

Secondary Cell Group: in dual connectivity, a group of serving cells associated with the SeNB, comprising of PSCell and optionally one or more SCells.

Secondary eNB: in dual connectivity, the eNB that is providing additional radio resources for the UE but is not the Master eNB.

Secondary PUCCH group: a group of SCells whose PUCCH signalling is associated with the PUCCH on the PUCCH SCell.

Secondary Timing Advance Group: Timing Advance Group containing neither the PCell nor PSCell.

Short Processing Time: For 1 ms TTI length, the operation with short processing time in UL data transmission and DL data reception.

Short TTI: TTI length based on a slot or a subslot.

Sidelink: UE to UE interface for sidelink communication, V2X sidelink communication and sidelink discovery. The Sidelink corresponds to the PC5 interface as defined in TS 23.303 [62].

Sidelink Control period: period over which resources are allocated in a cell for sidelink control information and sidelink data transmissions. The Sidelink Control period corresponds to the PSCCH period as defined in TS 36.213 [6].

Sidelink communication: AS functionality enabling ProSe Direct Communication as defined in TS 23.303 [62], between two or more nearby UEs, using E-UTRA technology but not traversing any network node. In this version, the terminology "sidelink communication" without "V2X" prefix only concerns PS unless specifically stated otherwise.

Sidelink discovery: AS functionality enabling ProSe Direct Discovery as defined in TS 23.303 [62], using E-UTRA technology but not traversing any network node.

Split bearer: in dual connectivity, a bearer whose radio protocols are located in both the MeNB and the SeNB to use both MeNB and SeNB resources.

Split LWA bearer: in LTE-WLAN Aggregation, a bearer whose radio protocols are located in both the eNB and the WLAN to use both eNB and WLAN radio resources.

Switched LWA bearer: in LTE-WLAN Aggregation, a bearer whose radio protocols are located in both the eNB and the WLAN but uses WLAN radio resources only.

Timing Advance Group: a group of serving cells that is configured by RRC and that, for the cells with an UL configured, use the same timing reference cell and the same Timing Advance value.

User plane CIoT 5GS Optimisation: Enables support for change from 5GMM-IDLE mode to 5GMM-CONNECTED mode without the need for using the Service Request procedure, as defined in TS 24.501 [xx].

User plane CIoT EPS Optimisation: Enables support for change from EMM-IDLE mode to EMM-CONNECTED mode without the need for using the Service Request procedure, as defined in TS 24.301 [20].

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [72], between nearby UEs, using E-UTRA technology but not traversing any network node.

WLAN Termination: the logical node that terminates the Xw interface on the WLAN side.

Next change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

1xCSFB	Circuit Switched Fallback to 1xRTT

5GC	5G Core Network

ABS	Almost Blank Subframe

AC	Access Category

ACK	Acknowledgement

ACLR	Adjacent Channel Leakage Ratio

AM	Acknowledged Mode

AMBR	Aggregate Maximum Bit Rate

ANDSF	Access Network Discovery and Selection Function

ANR	Automatic Neighbour Relation

ARP	Allocation and Retention Priority

ARQ	Automatic Repeat Request

AS	Access Stratum

AUL	Autonomous Uplink

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BL	Bandwidth reduced Low complexity

BR-BCCH	Bandwidth Reduced Broadcast Control Channel

BSR	Buffer Status Report

C/I	Carrier-to-Interference Power Ratio

CA	Carrier Aggregation

CAZAC	Constant Amplitude Zero Auto-Correlation

CBC	Cell Broadcast Center

CC	Component Carrier

CG	Cell Group

CIF	Carrier Indicator Field

CIoT	Cellular Internet of Things

CMAS	Commercial Mobile Alert Service

CMC	Connection Mobility Control

C-plane	Control Plane

C-RNTI	Cell RNTI

CoMP	Coordinated Multi Point

CP	Cyclic Prefix

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

CRE	Cell Range Extension

CRS	Cell-specific Reference Signal

CSA	Common Subframe Allocation

CSG	Closed Subscriber Group

CSI	Channel State Information

CSI-IM	CSI interference measurement

CSI-RS	CSI reference signal

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCN	Dedicated Core Network

DeNB	Donor eNB

DFTS	DFT Spread OFDM

DL	Downlink

DMTC	Discovery Signal Measurement Timing Configuration

DRB	Data Radio Bearer

DRS	Discovery Reference Signal

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

DTX	Discontinuous Transmission

DwPTS	Downlink Pilot Time Slot

E-CID	Enhanced Cell-ID (positioning method)

E-RAB	E-UTRAN Radio Access Bearer

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EAB	Extended Access Barring

ECGI	E-UTRAN Cell Global Identifier

ECM	EPS Connection Management

EDT	Early Data Transmission

eHRPD	enhanced High Rate Packet Data

eIMTA	Enhanced Interference Management and Traffic Adaptation

EMM	EPS Mobility Management

eNB	E-UTRAN NodeB

EPC	Evolved Packet Core

EPDCCH	Enhanced Physical Downlink Control Channel

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

G-RNTI	Group RNTI

GBR	Guaranteed Bit Rate

GERAN	GSM EDGE Radio Access Network

GNSS	Global Navigation Satellite System

GP	Guard Period

GRE	Generic Routing Encapsulation

GSM	Global System for Mobile communication

GUMMEI	Globally Unique MME Identifier

GUTI	Globally Unique Temporary Identifier

GWCN	GateWay Core Network

H-SFN	Hyper System Frame Number

HARQ	Hybrid ARQ

(H)eNB	eNB or HeNB

HO	Handover

HPLMN	Home Public Land Mobile Network

HRPD	High Rate Packet Data

HSDPA	High Speed Downlink Packet Access

ICIC	Inter-Cell Interference Coordination

IDC	In-Device Coexistence

IP	Internet Protocol

ISM	Industrial, Scientific and Medical

KPAS	Korean Public Alert System

L-GW	Local Gateway

LAA	Licensed-Assisted Access

LB	Load Balancing

LBT	Listen Before Talk

LCG	Logical Channel Group

LCR	Low Chip Rate

LCS	LoCation Service

LHN	Local Home Network

LHN ID	Local Home Network ID

LIPA	Local IP Access

LMU	Location Measurement Unit

LPPa	LTE Positioning Protocol Annex

LTE	Long Term Evolution

LWA	LTE-WLAN Aggregation

LWAAP	LTE-WLAN Aggregation Adaptation Protocol

LWIP	LTE WLAN Radio Level Integration with IPsec Tunnel

LWIP-SeGW	LWIP Security Gateway

MAC	Medium Access Control

MBMS	Multimedia Broadcast Multicast Service

MBR	Maximum Bit Rate

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MCCH	Multicast Control Channel

MCE	Multi-cell/multicast Coordination Entity

MCG	Master Cell Group

MCH	Multicast Channel

MCS	Modulation and Coding Scheme

MDT	Minimization of Drive Tests

MeNB	Master eNB

MGW	Media Gateway

MIB	Master Information Block

MIMO	Multiple Input Multiple Output

MME	Mobility Management Entity

MMTEL	Multimedia telephony

MO-EDT	Mobile Originated Early Data Transmission

MPDCCH	MTC Physical Downlink Control Channel

MSA	MCH Subframe Allocation

MSI	MCH Scheduling Information

MSP	MCH Scheduling Period

MTC	Machine-Type Communications

MTCH	Multicast Traffic Channel

MTSI	Multimedia Telephony Service for IMS

N2	Reference point between the NG-RAN and the AMF

NACK	Negative Acknowledgement

NAS	Non-Access Stratum

NB-IoT	Narrow Band Internet of Things

NCC	Next Hop Chaining Counter

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NG-RAN	NG Radio Access Network

NH	Next Hop key

NNSF	NAS Node Selection Function

NPBCH	Narrowband Physical Broadcast channel

NPDCCH	Narrowband Physical Downlink Control channel

NPDSCH	Narrowband Physical Downlink Shared channel

NPRACH	Narrowband Physical Random Access channel

NPUSCH	Narrowband Physical Uplink Shared channel

NPRS	Narrowband Positioning Reference Signal

NPSS	Narrowband Primary Synchronization Signal

NR	NR Radio Access

NRT	Neighbour Relation Table

NSSS	Narrowband Secondary Synchronization Signal

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OPI	Offload Preference Indicator

OTDOA	Observed Time Difference Of Arrival (positioning method)

P-GW	PDN Gateway

P-RNTI	Paging RNTI

PA	Power Amplifier

PAPR	Peak-to-Average Power Ratio

PBCH	Physical Broadcast CHannel

PBR	Prioritised Bit Rate

PCC	Primary Component Carrier

PCCH	Paging Control Channel

PCell	Primary Cell

PCFICH	Physical Control Format Indicator CHannel

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control CHannel

PDCP	Packet Data Convergence Protocol

PDN	Packet Data Network

PDSCH	Physical Downlink Shared CHannel

PDU	Protocol Data Unit

PHICH	Physical Hybrid ARQ Indicator CHannel

PHY	Physical layer

PLMN	Public Land Mobile Network

PMCH	Physical Multicast CHannel

PMK	Pairwise Master Key

PPPP	ProSe Per-Packet Priority

PPPR	ProSe Per-Packet Reliability

PRACH	Physical Random Access CHannel

PRB	Physical Resource Block

ProSe	Proximity based Services

PSBCH	Physical Sidelink Broadcast CHannel

PSC	Packet Scheduling

PSCCH	Physical Sidelink Control CHannel

PSCell	Primary SCell

PSDCH	Physical Sidelink Discovery CHannel

PSK	Pre-Shared Key

PSM	Power Saving Mode

PSSCH	Physical Sidelink Shared CHannel

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control CHannel

PUSCH	Physical Uplink Shared CHannel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QCI	QoS Class Identifier

QoE	Quality of Experience

QoS	Quality of Service

R-PDCCH	Relay Physical Downlink Control CHannel

RA-RNTI	Random Access RNTI

RAC	Radio Admission Control

RACH	Random Access Channel

RANAC	RAN-based Notification Area code

RAT	Radio Access Technology

RB	Radio Bearer

RBC	Radio Bearer Control

RCLWI	RAN Controlled LTE-WLAN Interworking

RF	Radio Frequency

RIBS	Radio-interface based synchronization

RIM	RAN Information Management

RLC	Radio Link Control

RMTC	RSSI Measurement Timing Configuration

RN	Relay Node

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNC	Radio Network Controller

RNL	Radio Network Layer

RNTI	Radio Network Temporary Identifier

ROHC	Robust Header Compression

ROM	Receive Only Mode

RRC	Radio Resource Control

RRM	Radio Resource Management

RU	Resource Unit

S-GW	Serving Gateway

S-RSRP	Sidelink Reference Signal Received Power

S1-MME	S1 for the control plane

SAE	System Architecture Evolution

SAP	Service Access Point

SBCCH	Sidelink Broadcast Control Channel

SC-FDMA	Single Carrier – Frequency Division Multiple Access

SC-MCCH	Single Cell Multicast Control Channel

SC-MTCH	Single Cell Multicast Transport Channel

SC-N-RNTI	Single Cell Notification RNTI

SC-PTM	Single Cell Point To Multiploint

SC-RNTI	Single Cell RNTI

SCC	Secondary Component Carrier

SCell	Secondary Cell

SCG	Secondary Cell Group

SCH	Synchronization Channel

SCTP	Stream Control Transmission Protocol

SD-RSRP	Sidelink Discovery Reference Signal Received Power

SDAP	Service Data Adaptation Protocol

SDF	Service Data Flow

SDMA	Spatial Division Multiple Access

SDU	Service Data Unit

SeGW	Security Gateway

SeNB	Secondary eNB

SFN	System Frame Number

SI	System Information

SI-RNTI	System Information RNTI

S1-U	S1 for the user plane

SIB	System Information Block

SIPTO	Selected IP Traffic Offload

SIPTO@LN	Selected IP Traffic Offload at the Local Network

SL-BCH	Sidelink Broadcast Channel

SL-DCH	Sidelink Discovery Channel

SL-RNTI	Sidelink RNTI

SL-SCH	Sidelink Shared Channel

SPDCCH	Short PDCCH

SPID	Subscriber Profile ID for RAT/Frequency Priority

SPT	Short Processing Time

SPUCCH	Short PUCCH

SR	Scheduling Request

SRB	Signalling Radio Bearer

sTAG	Secondary Timing Advance Group

STCH	Sidelink Traffic Channel

SU	Scheduling Unit

TA	Tracking Area

TAG	Timing Advance Group

TB	Transport Block

TCP	Transmission Control Protocol

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TEID	Tunnel Endpoint Identifier

TFT	Traffic Flow Template

TM	Transparent Mode

TMGI	Temporary Mobile Group Identity

TNL	Transport Network Layer

TTI	Transmission Time Interval

U-plane	User plane

UAC	Unified Access Control

UDC	Uplink Data Compression

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UMTS	Universal Mobile Telecommunication System

UpPTS	Uplink Pilot Time Slot

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

V2I	Vehicle-to-Infrastructure

V2N	Vehicle-to-Network

V2P	Vehicle-to-Pedestrian

V2V	Vehicle-to-Vehicle

V2X	Vehicle-to-Everything

VRB	Virtual Resource Block

WLAN	Wireless Local Area Network

WT	WLAN Termination

WUS	Wake Up Signal

X2-C	X2-Control plane

X2 GW	X2 GateWay

X2-U	X2-User plane

Xw-C	Xw-Control plane

Xw-U	Xw-User plane

Next change

## 4.1	Functional Split

The eNB hosts the following functions:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and sidelink (scheduling);

-	IP header compression, uplink data decompression and encryption of user data stream;

-	Selection of an MME at UE attachment when no routing to an MME can be determined from the information provided by the UE;

-	Routing of User Plane data towards Serving Gateway;

-	Scheduling and transmission of paging messages (originated from the MME);

-	Scheduling and transmission of broadcast information (originated from the MME or O&M);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Scheduling and transmission of PWS (which includes ETWS and CMAS) messages (originated from the MME);

-	CSG handling;

-	Transport level packet marking in the uplink;

-	S-GW relocation without UE mobility, as defined in TS 23.401 [17];

-	SIPTO@LN handling;

-	Maintaining security and radio configuration for User Plane CIoT EPS Optimisations, as defined in TS 24.301 [20];

-	Optionally registering with the X2 GW (if used).

The DeNB hosts the following functions in addition to the eNB functions:

-	S1/X2 proxy functionality for supporting RNs;

-	S11 termination and S-GW/P-GW functionality for supporting RNs.

The MME hosts the following functions (see TS 23.401 [17]):

-	NAS signalling;

-	NAS signalling security;

-	AS Security control;

-	Selection of CIoT EPS Optimisations (e.g., Control Plane CIoT EPS Optimisation, as defined in TS 24.301 [20]);

-	Inter CN node signalling for mobility between 3GPP access networks;

-	Idle mode UE Reachability (including control and execution of paging retransmission);

-	Tracking Area list management (for UE in idle and active mode);

-	PDN GW and Serving GW selection;

-	MME selection for handovers with MME change;

-	SGSN selection for handovers to 2G or 3G 3GPP access networks;

-	Roaming;

-	Authentication;

-	Bearer management functions including dedicated bearer establishment;

-	Support for PWS (which includes ETWS and CMAS) message transmission;

-	Optionally performing paging optimisation;

-	S-GW relocation without UE mobility, as defined in TS 23.401 [17].

NOTE 1:	The MME should not filter the PAGING message based on the CSG IDs towards macro eNBs.

The Serving Gateway (S-GW) hosts the following functions (see TS 23.401 [17]):

-	The local Mobility Anchor point for inter-eNB handover;

-	Mobility anchoring for inter-3GPP mobility;

-	E-UTRAN idle mode downlink packet buffering and initiation of network triggered service request procedure;

-	Lawful Interception;

-	Packet routeing and forwarding;

-	Transport level packet marking in the uplink and the downlink;

-	Accounting on user and QCI granularity for inter-operator charging;

-	UL and DL charging per UE, PDN, and QCI.

The PDN Gateway (P-GW) hosts the following functions (see TS 23.401 [17]):

-	Per-user based packet filtering (by e.g. deep packet inspection);

-	Lawful Interception;

-	UE IP address allocation;

-	Transport level packet marking in the uplink and the downlink;

-	UL and DL service level charging, gating and rate enforcement;

-	DL rate enforcement based on APN-AMBR;

This is summarized on the figure below where yellow boxes depict the logical nodes, white boxes depict the functional entities of the control plane and blue boxes depict the radio protocol layers.

NOTE 2:	There is no logical E-UTRAN node other than the eNB needed for RRM purposes.

NOTE 3:	MBMS related functions in E-UTRAN are described separately in clause 15.

# Figure 4.1-1: Functional Split between E-UTRAN and EPC

Next change

### 4.3.2	Control plane

The figure below shows the protocol stack for the control-plane, where:

-	PDCP sublayer (terminated in eNB on the network side) performs the functions listed for the control plane in clause 6, e.g. ciphering and integrity protection;

-	RLC and MAC sublayers (terminated in eNB on the network side) perform the same functions as for the user plane;

-	RRC (terminated in eNB on the network side) performs the functions listed in clause 7, e.g.:

-	Broadcast;

-	Paging;

-	RRC connection management;

-	RB control;

-	Mobility functions;

-	UE measurement reporting and control, except for NB-IoT.

-	NAS control protocol (terminated in MME on the network side) performs among other things:

-	EPS bearer management;

-	Authentication;

-	ECM-IDLE mobility handling;

-	Paging origination in ECM-IDLE;

-	Security control.

NOTE 1:	The NAS control protocol is not covered by the scope of this TS and is only mentioned for information.

# Figure 4.3.2-1: Control-plane protocol stack

NOTE 2:	For a NB-IoT UE that only supports Control Plane CIoT EPS Optimisation, as defined in TS 24.301 [20], PDCP is bypassed. For a NB-IoT UE that supports Control Plane CIoT EPS Optimisation and S1-U data transfer or User Plane CIoT EPS Optimisation, as defined in TS 24.301 [20], PDCP is also bypassed (i.e. not used) until AS security is activated.

Next change

## 4.10	NB-IoT

NB-IoT provides access to network services using physical layer optimized for very low power consumption (e.g. full carrier bandwidth is 180 kHz, subcarrier spacing can be 3.75 kHz or 15 kHz).

As indicated in the relevant clauses in this specification, a number of E-UTRA protocol functions supported by all Rel-8 UEs are not used for NB-IoT and need not be supported by eNBs and UEs only using NB-IoT.

In this version of the specification, a number of functions including inter-RAT mobility, handover, measurement reports, public warning functions, GBR, CSG, support of HeNBs, relaying, carrier aggregation, dual connectivity, NAICS, real-time services, interference avoidance for in-device coexistence, RAN assisted WLAN interworking, sidelink communication/discovery, V2X sidelink communication, MDT, emergency call, CS fallback, ACB, EAB, ACDC, SSAC, aerial UE Communication, EN-DC and RRC_INACTIVE state are not supported for NB-IoT. This is not further stated in the corresponding procedures.

Next change

## 6.0	Overview

Layer 2 is split into the following sublayers: Medium Access Control (MAC), Radio Link Control (RLC) and Packet Data Convergence Protocol (PDCP).

This clause gives a high level description of the Layer 2 sub-layers in terms of services and functions. The three figures below depict the PDCP/RLC/MAC architecture for downlink, uplink and Sidelink, where:

-	Service Access Points (SAP) for peer-to-peer communication are marked with circles at the interface between sublayers. The SAP between the physical layer and the MAC sublayer provides the transport channels. The SAPs between the MAC sublayer and the RLC sublayer provide the logical channels.

-	The multiplexing of several logical channels (i.e. radio bearers) on the same transport channel (i.e. transport block) is performed by the MAC sublayer;

-	In both uplink and downlink, when neither CA nor DC are configured, only one transport block is generated per TTI in the absence of spatial multiplexing;

-	In Sidelink, only one transport block is generated per TTI for each carrier.

# Figure 6-1: Layer 2 Structure for DL

# Figure 6-2: Layer 2 Structure for UL

NOTE 1:	The eNB may not be able to guarantee that a L2 buffer overflow will never occur. If such overflow occurs, UE may discard packets in the L2 buffer.

NOTE 2:	For a NB-IoT UE that only supports Control Plane CIoT EPS Optimisations, as defined in TS 24.301 [20], PDCP is bypassed. For a NB-IoT UE that supports Control Plane CIoT EPS Optimisation and S1-U data transfer or User Plane CIoT EPS Optimisation, as defined in TS 24.301 [20], PDCP is also bypassed (i.e. not used) until AS security is activated.

# Figure 6-3: Layer 2 Structure for Sidelink

Next change

#### 6.1.2.0	General

Different kinds of data transfer services as offered by MAC. Each logical channel type is defined by what type of information is transferred.

A general classification of logical channels is into two groups:

-	Control Channels (for the transfer of control plane information);

-	Traffic Channels (for the transfer of user plane information).

There is one MAC entity per CG. MAC generally consists of several function blocks (transmission scheduling functions, per UE functions, MBMS functions, MAC control functions, transport block generation…). Transparent Mode is only applied to BCCH, BR-BCCH, PCCH and SBCCH.

NOTE:	For a NB-IoT UE that only uses Control Plane CIoT EPS/5GS Optimisations, as defined in TS 24.301 [20] and TS 24.501 [xx], there is only one dedicated logical channel per UE.

Next change

#### 6.1.2.2	Traffic Channels

Traffic channels are used for the transfer of user plane information only. The traffic channels offered by MAC are:

-	Dedicated Traffic Channel (DTCH)

A Dedicated Traffic Channel (DTCH) is a point-to-point channel, dedicated to one UE, for the transfer of user information. A DTCH can exist in both uplink and downlink. DTCH is not supported for a NB-IoT UE that only uses Control Plane CIoT EPS/5GS Optimisation, as defined in TS 24.301 [20] and TS 24.501 [xx].

-	Multicast Traffic Channel (MTCH)

A point-to-multipoint downlink channel for transmitting traffic data from the network to the UE. This channel is only used by UEs that receive MBMS.

-	Single-Cell Multicast Traffic Channel (SC-MTCH)

A point-to-multipoint downlink channel for transmitting traffic data from the network to the UE using SC-PTM transmission. This channel is only used by UEs that receive MBMS using SC-PTM.

-	Sidelink Traffic Channel (STCH)

A Sidelink Traffic Channel (STCH) is a point-to-multipoint channel, for transfer of user information from one UE to other UE(s). This channel is used only by sidelink communication capable UEs and V2X sidelink communication capable UEs. Point-to-point communication between two sidelink communication capable UEs is also realized with an STCH.

Next change

### 6.3.1	Services and Functions

Except for NB-IoT, the main services and functions of the PDCP sublayer for the user plane include:

-	Header compression and decompression: ROHC only;

-	Compression and decompression of uplink PDCP SDUs: DEFLATE based UDC only;

-	Transfer of user data;

-	In-sequence delivery of upper layer PDUs at PDCP re-establishment procedure for RLC AM;

-	For split bearers in DC (only support for RLC AM) and LWA bearers (only support for RLC AM and RLC UM): PDCP PDU routing for transmission and PDCP PDU reordering for reception;

-	Duplicate detection of lower layer SDUs at PDCP re-establishment procedure for RLC AM;

-	Retransmission of PDCP SDUs at handover and, for split bearers in DC and LWA, of PDCP PDUs at PDCP data-recovery procedure, for RLC AM;

-	Ciphering and deciphering;

-	Timer-based SDU discard in uplink;

-	Duplication of PDCP PDUs;

-	For PDCP duplication, reordering and duplicate detection at the receiver.

For NB-IoT UE when AS security is activated, the main services and functions of the PDCP sublayer for the user plane include:

-	Header compression and decompression: ROHC only;

-	Transfer of user data;

-	In-sequence delivery of upper layer PDUs at PDCP re-establishment procedure for RLC AM;

-	Duplicate detection of lower layer SDUs at PDCP re-establishment procedure for RLC AM;

-	Ciphering and deciphering;

-	Timer-based SDU discard in uplink.

NOTE 1:	When compared to UTRAN, the lossless DL RLC PDU size change is not required.

The main services and functions of the PDCP for the control plane include:

-	Ciphering and Integrity Protection;

-	Transfer of control plane data.

Except for NB-IoT, the main services and functions of the PDCP sublayer for the control plane also include:

-	Duplication of PDCP PDUs;

-	For PDCP duplication, reordering and duplicate detection at the receiver.

NOTE 2:	For a NB-IoT UE that only supports Control Plane CIoT EPS Optimisation, as defined in TS 24.301 [20], PDCP is bypassed. For a NB-IoT UE that supports Control Plane CIoT EPS Optimisation and S1-U data transfer or User Plane CIoT EPS Optimisation, as defined in TS 24.301 [20], PDCP is not used until AS security is activated.

Next change

## 7.1	Services and Functions

The main services and functions of the RRC sublayer include:

-	Broadcast of System Information related to the non-access stratum (NAS);

-	Broadcast of System Information related to the access stratum (AS);

-	Paging;

-	Establishment, maintenance and release of an RRC connection between the UE and E-UTRAN including:

-	Allocation of temporary identifiers between UE and E-UTRAN;

-	Configuration of signalling radio bearer(s) for RRC connection:

-	Low priority SRB and high priority SRB;

-	For NB-IoT, a UE dedicated SRB is supported before AS security is activated and only one UE dedicated SRB is supported after AS security is activated;

-	For a NB-IoT UE that supports S1-U data transfer or User Plane CIoT EPS Optimisation, as defined in TS 24.301 [20]; or

-	For a NB-IoT UE that supports NG-U data transfer or User Plane CIoT 5GS Optimisation, as defined in TS 24.501 [xx]:

-	One DRB is supported by default and up to two DRBs are supported optionally;

-	For a UE that supports User Plane CIoT EPS Optimisation, as specified in TS 24.301 [20]; or

-	For a UE that supports User Plane CIoT 5GS Optimisation, as specified in TS 24.501 [82]:

-	Suspension/resuming of the RRC connection;

-	Security functions including key management;

-	Establishment, configuration, maintenance and release of point to point Radio Bearers;

-	Mobility functions including:

-	UE measurement reporting and control of the reporting for inter-cell and inter-RAT mobility;

-	Handover;

-	UE cell selection and reselection and control of cell selection and reselection;

-	Context transfer at handover.

-	Notification and counting for MBMS services;

-	Establishment, configuration, maintenance and release of Radio Bearers for MBMS services;

-	QoS management functions;

-	UE measurement reporting and control of the reporting;

-	NAS direct message transfer to/from NAS from/to UE.

Next change

## 7.2	RRC protocol states & state transitions

RRC uses the following states:

-	RRC_IDLE:

-	PLMN selection;

-	DRX configured by NAS;

-	Broadcast of system information;

-	Paging;

-	Cell re-selection mobility;

-	The UE shall have been allocated an id which uniquely identifies the UE in a tracking area;

-	No RRC context stored in the eNB (except for a UE that supports User Plane CIoT EPS/5GS Optimisations, as specified in TS 24.301 [20] and TS 24.501 [xx], where a context may be stored for the resume procedure);

-	Sidelink communication transmission and reception;

-	Sidelink discovery announcement and monitoring;

-	V2X sidelink communication transmission and reception;

-	EDT.

-	RRC_CONNECTED:

-	UE has an E-UTRAN-RRC connection;

-	UE has context in E-UTRAN;

-	E-UTRAN knows the cell which the UE belongs to;

-	Network can transmit and/or receive data to/from UE;

-	Network controlled mobility (handover and inter-RAT cell change order to GERAN with NACC);

-	Neighbour cell measurements;

-	Sidelink communication transmission and reception;

-	Sidelink discovery announcement and monitoring;

-	V2X sidelink communication transmission and reception;

-	At PDCP/RLC/MAC level:

-	UE can transmit and/or receive data to/from network;

-	UE monitors control signalling channel for shared data channel to see if any transmission over the shared data channel has been allocated to the UE;

-	UE also reports channel quality information and feedback information to eNB;

-	DRX period can be configured according to UE activity level for UE power saving and efficient resource utilization. This is under control of the eNB.

E-UTRA connected to 5GC additionally supports RRC_INACTIVE state, which has the same characteristics as RRC_INACTIVE of NR connected to 5GC, as specified in TS 38.300 [79].

Next change

## 7.3	Transport of NAS messages

The AS provides reliable in-sequence delivery of NAS messages in a cell. During handover, message loss or duplication of NAS messages can occur.

In E-UTRAN, NAS messages are either concatenated with RRC messages or carried in RRC without concatenation. Upon arrival of concurrent NAS messages for the same UE requiring both concatenation with RRC for the high priority queue and also without concatenation for the lower priority queue, the messages are first queued as necessary to maintain in-sequence delivery.

In downlink, when an EPS bearer (EPC) or PDU Session (5GC) establishment or release procedure is triggered, or for EDT in case of Control Plane CIoT EPS Optimisation (EPC) or Control Plane CIoT 5GS Optimisation (5GC), the NAS message should normally be concatenated with the associated RRC message. When the EPS bearer (EPC) or PDU Session (5GC) is modified and when the modification also depends on a modification of the radio bearer, the NAS message and associated RRC message should normally be concatenated. Concatenation of DL NAS with RRC message is not allowed otherwise. In uplink, concatenation of NAS messages with RRC message is used only for transferring the initial NAS message during connection setup and for EDT in case of Control Plane CIoT EPS Optimisation (EPC) or Control Plane CIoT 5GS Optimisation (5GC). Initial Direct Transfer is not used in E-UTRAN and no NAS message is concatenated with RRC connection request.

Multiple NAS messages can be sent in a single downlink RRC message during EPS bearer (EPC) or PDU Session (5GC) establishment or modification. In this case, the order of the NAS messages in the RRC message shall be kept the same as that in the corresponding S1-AP (EPC) or NG-AP (5GC) message in order to ensure the in-sequence delivery of NAS messages.

NOTE:	NAS messages are integrity protected and ciphered by PDCP, in addition to the integrity protection and ciphering performed by NAS.

Next change

## 7.4	System Information

System information is divided into the MasterInformationBlock (MIB) and a number of SystemInformationBlocks (SIBs):

-	MasterInformationBlock defines the most essential physical layer information of the cell required to receive further system information;

-	SystemInformationBlockPos contains positioning assistance data;

-	SystemInformationBlockType1 and SystemInformationBlockType1-BR (for a BL UE or UE in enhanced coverage) contain information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2 contains common and shared channel information;

-	SystemInformationBlockType3 contains cell re-selection information, mainly related to the serving cell;

-	SystemInformationBlockType4 contains information about the serving frequency and intra-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType5 contains information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType6 contains information about UTRA frequencies and UTRA neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType7 contains information about GERAN frequencies relevant for cell re-selection (including cell re-selection parameters for each frequency);

-	SystemInformationBlockType8 contains information about CDMA2000 frequencies and CDMA2000 neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType9 contains a home eNB name (HNB name);

-	SystemInformationBlockType10 contains an ETWS primary notification;

-	SystemInformationBlockType11 contains an ETWS secondary notification;

-	SystemInformationBlockType12 contains a CMAS warning notification;

-	SystemInformationBlockType13 contains MBMS-related information;

-	SystemInformationBlockType14 contains information about Extended Access Barring for access control;

-	SystemInformationBlockType15 contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16 contains information related to GPS time and Coordinated Universal Time (UTC );

-	SystemInformationBlockType17 contains information relevant for traffic steering between E-UTRAN and WLAN;

-	SystemInformationBlockType18 contains information related to sidelink communication;

-	SystemInformationBlockType19 contains information related to sidelink discovery;

-	SystemInformationBlockType20 contains information related to SC-PTM;

-	SystemInformationBlockType21 contains information related to V2X sidelink communication;

-	SystemInformationBlockType24 contains information about NR frequencies and NR neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency);

-	SystemInformationBlockType25 contains information about UAC parameters;

-	SystemInformationBlockType26 contains additional information related to V2X sidelink communication;

-	SystemInformationBlockTypeXX contains assistance information for inter-RAT cell selection to NB-IoT.

System information for NB-IoT is divided into the MasterInformationBlock-NB (MIB-NB) and a number of SystemInformationBlocks-NB (SIBs-NB):

-	MasterInformationBlock-NB defines the most essential information of the cell required to receive further system information;

-	SystemInformationBlockType1-NB contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2-NB contains common radio resource configuration information;

-	SystemInformationBlockType3-NB contains cell re-selection information for intra-frequency, inter-frequency;

-	SystemInformationBlockType4-NB contains neighboring cell related information relevant for intra-frequency cell re-selection;

-	SystemInformationBlockType5-NB contains neighboring cell related information relevant for inter-frequency cell re-selection;

-	SystemInformationBlockType14-NB contains information about access barring;

-	SystemInformationBlockType15-NB contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16-NB contains information related to GPS time and Coordinated Universal Time (UTC);

-	SystemInformationBlockType20-NB contains information related to SC-PTM;

-	SystemInformationBlockType22-NB contains common radio resource configuration information for paging and random access procedure on non-anchor carriers;

-	SystemInformationBlockType23-NB contains common additional radio resource configuration information for random access procedure on anchor and non-anchor carriers;

-	SystemInformationBlockTypeXX-NB contains assistance information for inter-RAT cell selection to E-UTRAN and/or GERAN.

On MBMS-dedicated cell, only system information relevant for receiving MBMS service is broadcasted. MasterInformationBlock-MBMS (MIB-MBMS) and SystemInformationBlockType1-MBMS (SIB1-MBMS) are used instead of MIB and SIB1 respectively:

-	MasterInformationBlock-MBMS defines the most essential physical layer information of the cell required to receive further system information on MBMS-dedicated cell;

-	SystemInformationBlockType1-MBMS contains information relevant for receiving MBMS service and defines the scheduling of other system information blocks on MBMS-dedicated cell;

The MIB is mapped on the BCCH and carried on BCH while all other SI messages are mapped on the BCCH and BR-BCCH, and carried on DL-SCH. Except for BL UEs, UEs in enhanced coverage and NB-IoT UEs, all other SI messages than the MIB which are dynamically carried on DL-SCH, can be identified through the SI-RNTI (System Information RNTI). Both the MIB and SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs and UEs in enhanced coverage) use a fixed schedule with a periodicity of 40 and 80 ms respectively. The scheduling of other SI messages is flexible and indicated by SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs and UEs in enhanced coverage, and SystemInformationBlockType1-NB for NB-IoT). For NB-IoT, the MIB-NB is mapped on the BCCH and carried on BCH while all other SI messages are mapped on the BCCH and carried on DL-SCH. Both the MIB-NB and SystemInformationBlockType1-NB use a fixed schedule with a periodicity of 640 and 2560 ms respectively. The MIB-NB contains all information required to acquire SIB1-NB and SIB1-NB contains all information required to acquire other SI messages.

On MBMS-dedicated cell, the MIB-MBMS and SIB1-MBMS use a fixed schedule with a periodicity of 160 ms. Additionally, SIB1-MBMS may be scheduled in additional non-MBSFN subframes indicated in MIB-MBMS.

For NB-IoT, in TDD mode, the MIB-TDD-NB is transmitted on the same NB-IoT carrier as NPSS/NSSS, SystemInformationBlockType1-NB can be transmitted on NB-IoT carrier other than the MIB-NB, and the SI messages can be transmitted on a NB-IoT carrier other than the MIB-NB. At most two NB-IoT carriers are used to transmit the MIB-NB, SystemInformationBlockType1-NB and the SI messages.

Except for NB-IoT, the eNB may schedule DL-SCH transmissions concerning logical channels other than BCCH or BR-BCCH in the same subframe as used for BCCH or BR-BCCH. The minimum UE capability restricts the BCCH or BR-BCCH mapped to DL-SCH e.g. regarding the maximum rate.

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. For NB-IoT UEs, BL UEs, and UEs in CE, the UE is not required to detect SIB changes when in RRC_CONNECTED, and the network may release the NB-IoT UE, BL UE or UE in CE to RRC_IDLE if it wants the NB-IoT UE, BL UE or UE in CE to acquire changed SIB(s).

Except for NB-IoT, system information may also be provided to the UE by means of dedicated signalling e.g. upon handover.

Next change

## 7.3a	CIoT signalling reduction optimisations

### 7.3a.1	General

Which solution of CIoT signalling reduction optimisations to be used is configured over NAS signalling between the UE and the MME or the AMF.

For NB-IoT, PDCP is not used while AS security is not activated.

### 7.3a.2	Control Plane CIoT EPS/5GS Optimisations

The RRC connection established for Control Plane CIoT EPS optimisation, as defined in TS 24.301 [20], and Control Plane CIoT 5GS Optimisation, as defined in TS 24.501 [xx], are characterized as below:

-	A UL NAS signalling message or UL NAS message carrying data can be transmitted in a UL RRC container message (see Figure 7.3a.2-1). A DL NAS signaling or DL NAS data can be transmitted in a DL RRC container message;

-	for NB-IoT:

-	RRC connection reconfiguration is not supported;

-	Data radio bearer (DRB) is not used;

-	AS security is not used;

-	A non-anchor carrier can be configured for all unicast transmissions during RRC connection establishment or re-establishment.

-	There is no differentiation between the different data types (i.e. IP, non-IP or SMS) in the AS.

# Figure 7.3a.2-1: The RRC connection established for Control Plane CIoT EPS/5GS Optimisations

### 7.3a.3	User Plane CIoT EPS/5GS Optimisations

The RRC connection established for User Plane CIoT EPS Optimisation, as defined in TS 24.301 [20], and User Plane CIoT 5GS Optimisation, as defined in TS 24.501 [xx], are characterized as below:

-	A RRC connection suspend procedure is used at RRC connection release, the (ng-)eNB may request the UE to retain the UE AS context including UE capability in RRC_IDLE;

-	A RRC connection resume procedure is used at transition from RRC_IDLE to RRC_CONNECTED where previously stored information in the UE as well as in the (ng-)eNB is utilised to resume the RRC connection. In the message to resume, the UE provides a Resume ID to be used by the (ng-)eNB to access the stored information required to resume the RRC connection;

-	At suspend-resume, security is continued. Re-keying is not supported in RRC connection resume procedure. The short MAC-I is reused as the authentication token at RRC connection reestablishment procedure and RRC connection resume procedure by the UE. The (ng-)eNB provides the NCC in the RRCConnectionResume message as well. And also the UE resets the COUNT;

-	Multiplexing of CCCH and DTCH in the transition from RRC_IDLE to RRC CONNECTED is not supported;

-	For NB-IoT, a non-anchor carrier can be configured for all unicast transmissions when an RRC connection is re-established, resumed or reconfigured additionally when an RRC connection is established.

The RRC connection suspend and resume procedures are illustrated in Figures 7.3a.3-1/7.3a.3-1a and 7.3a.3-2/7.3a.3-2a, respectively. Note that the description here is only intended as an overview and all parameters are therefore not listed in the message flows.

# Figure 7.3a.3-1: RRC Connection Suspend procedure in EPS

# Figure 7.3a.3-1a: RRC Connection Suspend procedure in 5GS

1.	Due to some triggers, e.g. the expiry of a UE inactivity timer, the (ng-)eNB decides to suspend the RRC connection.

2.	In EPS, the eNB initiates the S1-AP UE Context Suspend procedure to inform the MME that the RRC connection is being suspended. In 5GS, the ng-eNB initiates the NG-AP UE Context Suspend procedure to inform the AMF that the RRC connection is being suspended.

3.	In EPS, the MME requests the S-GW to release all S1-U bearers for the UE. In 5GS, the AMF requests the SMF to suspend the PDU session and the SMF requests the UPF to release the tunnel information for the UE.

4.	MME/AMF Acks step 2.

5.	The eNB suspends the RRC connection by sending an RRCConnectionRelease message with the releaseCause set to rrc-Suspend. The message includes the Resume ID which is stored by the UE. Optionally, for EDT, the message also includes the NextHopChainingCount which is stored by the UE.

6.	The UE stores the AS context, suspends all SRBs and DRBs, and enters RRC_IDLE.

Figure 7.3a.3-2: RRC Connection Resume procedure in EPS

# Figure 7.3a.3-2a: RRC Connection Resume procedure in 5GS

1.	At some later point in time (e.g. when the UE is being paged or when new data arrives in the uplink buffer) the UE resumes the connection by sending an RRCConnectionResumeRequest to the (ng-)eNB. The UE includes its Resume ID, the establishment cause, and authentication token. The authentication token is calculated in the same way as the short MAC-I used in RRC connection re-establishment and allows the (ng-)eNB to verify the UE identity.

Editor’s note: FFS if ResumeID or I-RNTI is used for 5GS.

2.	Provided that the Resume ID exists and the authentication token is successfully validated, the (ng-)eNB responds with an RRCConnectionResume. The message includes the Next Hop Chaining Count (NCC) value which is required in order to re-establish the AS security.

3.	The UE resumes all SRBs and DRBs and re-establishes the AS security. The UE is now in RRC_CONNECTED.

Editor’s note: FFS which 5GS or EPS security principles are used for the User Plane CIoT 5GS Optimisations.

4.	The UE responds with an RRCConnectionResumeComplete confirming that the RRC connection was resumed successfully, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the (ng-)eNB.

5.	For EPS, the eNB initiates the S1-AP Context Resume procedure to notify the MME about the UE state change. For 5GS, the ng-eNB initiates the NG-AP UE Context Resume procedure to notify the AMF about the UE state change.

6.	For EPS, the MME requests the S-GW to activate the S1-U bearers for the UE. For 5GS, the AMF requests the SMF to resume the PDU session and the SMF requests the UPF to establish the tunnel information for the UE.

7.	MME/AMF Acks step 5.

An RRC connection can also be resumed in an (ng-)eNB (the new (ng-)eNB) different from the one where the connection was suspended (the old (ng-)eNB). Inter (ng-)eNB connection resumption is handled using context fetching, whereby the new (ng-)eNB retrieves the UE context from the old (ng-)eNB over the X2/Xn interface. The new (ng-)eNB provides the Resume ID which is used by the old (ng-)eNB to identify the UE context. This is illustrated in Figure 7.3a.3-3/7.3a.3-3a.

# Figure 7.3a.3-3: RRC Connection Resume procedure in different eNB in EPS

Figure 7.3a.3-3a: RRC Connection Resume procedure in different ng-eNB in 5GS

1.	Same as step 1 in the intra (ng-)eNB connection resumption.

2.	The new (ng-)eNB locates the old (ng-)eNB using the Resume ID and retrieves the UE context by means of the X2-AP (for EPS) or Xn-AP (for 5GS) Retrieve UE Context procedure.

3.	The old (ng-)eNB responds with the UE context associated with the Resume ID.

4.	Same as step 2 in the intra (ng-)eNB connection resumption.

5.	Same as step 3 in the intra (ng-)eNB connection resumption.

6.	Same as step 4 in the intra (ng-)eNB connection resumption.

7.	For EPS, the new eNB initiates the S1-AP Path Switch procedure to establish a S1 UE associated signalling connection to the serving MME and to request the MME to resume the UE context. For 5GS, the new ng-eNB initiates the NG-AP Path Switch procedure to establish a NG UE associated signalling connection to the serving AMF and to request the AMF to resume the UE context.

8.	For EPS, the MME requests the S-GW to activate the S1-U bearers for the UE and updates the downlink path. For 5GS, the AMF requests the SMF to resume the PDU session and the SMF requests the UPF to create the tunnel information for the UE and update the downlink path.

9.	MME/AMF Acks step 7.

10.	For EPS, after the S1-AP Path Switch procedure the new eNB triggers release of the UE context at the old eNB by means of the X2-AP UE Context Release procedure. For 5GS, after the NG-AP Path Switch procedure the new ng-eNB triggers release of the UE context at the old ng-eNB by means of the Xn-AP UE Context Release procedure.

For a NB-IoT UE that supports Control Plane CIoT EPS Optimisation and S1-U data transfer or User Plane CIoT EPS Optimisation, as defined in TS 24.301 [20], and for a NB-IoT UE that supports Control Plane CIoT 5GS Optimisation and NG-U data transfer or User Plane CIoT 5GS Optimisation, as defined in TS 24.501 [xx], PDCP is not used until AS security is activated.

Next change

## 7.3b	MO-EDT

### 7.3b.1	General

MO-EDT allows one uplink data transmission optionally followed by one downlink data transmission during the random access procedure.

MO-EDT is triggered when the upper layers have requested the establishment or resumption of the RRC Connection for Mobile Originated data (i.e., not signalling or SMS) and the uplink data size is less than or equal to a TB size indicated in the system information. MO-EDT is not used for data over the control plane when using the User Plane CIoT EPS/5GS Optimisations.

MO-EDT is only applicable to BL UEs, UEs in Enhanced Coverage and NB-IoT UEs.

### 7.3b.2	MO-EDT for Control Plane CIoT EPS/5GS Optimisations

MO-EDT for Control Plane CIoT EPS Optimisation, as defined in TS 24.301 [20], and Control Plane CIoT 5GS Optimisation, as defined in TS 24.501 [xx], are characterized as below:

-	Uplink user data are transmitted in a NAS message concatenated in UL RRCEarlyDataRequest message on CCCH;

-	Downlink user data are optionally transmitted in a NAS message concatenated in DL RRCEarlyDataComplete message on CCCH;

-	There is no transition to RRC CONNECTED.

The MO-EDT procedure for Control Plane CIoT EPS Optimisations and Control Plane CIoT 5GS Optimisations are illustrated in Figure 7.3b-1 and Figure 7.3b-1a respectively.

# Figure 7.3b-1: MO-EDT for Control Plane CIoT EPS Optimisations

# Figure 7.3b-1a: MO-EDT for Control Plane CIoT 5GS Optimisations

0.	Upon connection establishment request for Mobile Originated data from the upper layers, the UE initiates the early data transmission procedure and selects a random access preamble configured for EDT.

1.	UE sends RRCEarlyDataRequest message concatenating the user data on CCCH.

2.	For EPS, the eNB initiates the S1-AP Initial UE message procedure to forward the NAS message and establish the S1 connection. For 5GS, the ng-eNB initiates the NG-AP Initial UE message procedure to forward the NAS message. The (ng-)eNB may indicate in this procedure that this connection is triggered for EDT.

3.	For EPS, the MME requests the S-GW to re-activate the EPS bearers for the UE. For 5GS, the AMF determines the PDU session contained in the NAS message.

4.	For EPS, the MME sends the uplink data to the S-GW. For 5GS, the AMF sends the PDU Session ID and the uplink data to the SMF and the SMF forwards the uplink data to the UPF.

5.	For EPS, if downlink data are available, the S-GW sends the downlink data to the MME. For 5GS, if downlink data are available, the UPF forwards the downlink data to SMF and the SFM forwards the downlink data to AMF.

6.	If downlink data are received from the S-GW or SMF, the MME or AMF forwards the data to the eNB or ng-eNB via DL NAS Transport procedure and may also indicate whether further data are expected. Otherwise, the MME or AMF may trigger Connection Establishment Indication procedure and also indicate whether further data are expected.

7.	If no further data are expected, the (ng-)eNB can send the RRCEarlyDataComplete message on CCCH to keep the UE in RRC_IDLE. If downlink data were received in step 6, they are concatenated in RRCEarlyDataComplete message.

8.	For PS, the S1 connection is released and the EPS bearers are deactivated. For 5GS, the AN release procedure is started.

NOTE 1:	If the MME/AMF or the (ng-)eNB decides to move the UE in RRC_CONNECTED mode, RRCConnectionSetup message is sent in step 7 to fall back to the legacy RRC Connection establishment procedure; the (ng-)eNB will discard the zero-length NAS PDU received in RRCConnectionSetupComplete message.

NOTE 2:	If neither RRCEarlyDataComplete nor, in case of fallback, RRCConnectionSetup is received in response to RRCEarlyDataRequest, the UE considers the UL data transmission not successful.

### 7.3b.3	MO-EDT for User Plane CIoT EPS Optimisations

Editor’s note: To update this section for 5GS once the description is available in TS 23.502.

MO-EDT for User Plane CIoT EPS Optimisations, as defined in TS 24.301 [20], is characterized as below:

-	The UE has been provided with a NextHopChainingCount in the RRCConnectionRelease message with suspend indication;

-	Uplink user data are transmitted on DTCH multiplexed with UL RRCConnectionResumeRequest message on CCCH;

-	Downlink user data are optionally transmitted on DTCH multiplexed with DL RRCConnectionRelease message on DCCH;

-	The short resume MAC-I is reused as the authentication token for RRCConnectionResumeRequest message and is calculated using the integrity key from the previous connection;

-	The user data in uplink and downlink are ciphered. The keys are derived using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous RRC connection;

-	The RRCConnectionRelease message is integrity protected and ciphered using the newly derived keys;

-	There is no transition to RRC CONNECTED.

The MO-EDT procedure for User Plane CIoT EPS Optimisations is illustrated in Figure 7.3b-2.

# Figure 7.3b-2: MO-EDT for User Plane CIoT EPS Optimisations

0.	Upon connection resumption request for Mobile Originated data from the upper layers, the UE initiates the early data transmission procedure and selects a random access preamble configured for EDT.

1.	The UE sends an RRCConnectionResumeRequest to the eNB, including its Resume ID, the establishment cause, and an authentication token. The UE resumes all SRBs and DRBs, derives new security keys using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous connection and re-establishes the AS security. The user data are ciphered and transmitted on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH.

2.	The eNB initiates the S1-AP Context Resume procedure to resume the S1 connection and re-activate the S1-U bearers.

3.	The MME requests the S-GW to re-activate the S1-U bearers for the UE.

4.	The MME confirms the UE context resumption to the eNB.

5.	The uplink data are delivered to the S-GW.

6.	If downlink data are available, the S-GW sends the downlink data to the eNB.

7.	If no further data are expected from the S-GW, the eNB can initiate the suspension of the S1 connection and the deactivation of the S1-U bearers.

8.	The eNB sends the RRCConnectionRelease message to keep the UE in RRC_IDLE. The message includes the releaseCause set to rrc-Suspend, the resumeID, the NextHopChainingCount and drb-ContinueROHC which are stored by the UE. If downlink data were received in step 6, they are sent ciphered on DTCH multiplexed with the RRCConnectionRelease message on DCCH.

NOTE 1:	If the MME or eNB decides the UE to move in RRC_CONNECTED mode, RRCConnectionResume message is sent in step 7 to fall back to the RRC Connection resume procedure. In that case, the RRCConnectionResume message is integrity protected and ciphered with the keys derived in step 1 and the UE ignores the NextHopChainingCount included in the RRCConnectionResume message. Downlink data can be transmitted on DTCH multiplexed with the RRCConnectionResume message. In addition, an RRCConnectionSetup can also be sent in step 7 to fall back to the RRC Connection establishment procedure.

NOTE 2:	If neither RRCConnectionRelease nor, in case of fallback, RRCConnectionResume is received in response to RRCConnectionResumeRequest for MO-EDT, the UE considers the UL data transmission not successful.

For MO-EDT for User Plane CIoT EPS Optimisations, an RRC connection can also be resumed in an eNB (the new eNB) different from the one where the connection was suspended (the old eNB). Inter eNB connection resumption is handled using context fetching, whereby the new eNB retrieves the UE context from the old eNB over the X2 interface. The new eNB provides the Resume ID which is used by the old eNB to identify the UE context. This is illustrated in Figure 7.3b-3.

Figure: 7.3b-3: MO-EDT for User Plane CIoT EPS Optimisations in different eNB

1.	Same as step 1 in the intra eNB connection resumption.

2.	The new eNB locates the old eNB using the Resume ID and retrieves the UE context by means of the X2-AP Retrieve UE Context procedure.

3.	The old eNB responds with the UE context associated with the Resume ID.

4.	The new eNB initiates the S1-AP Path Switch procedure to establish a S1 UE associated signalling connection to the serving MME and to request the MME to resume the UE context.

5.	The MME requests the S-GW to activate the S1-U bearers for the UE and updates the downlink path.

6.	MME Acks step 5.

7.	After the S1-AP Path Switch procedure the new eNB triggers release of the UE context at the old eNB by means of the X2-AP UE Context Release procedure.

8.	Same as step 5 in the intra eNB connection resumption.

9.	Same as step 6 in the intra eNB connection resumption.

10.	Same as step 7 in the intra eNB connection resumption.

11.	Same as step 8 in the intra eNB connection resumption.

Next change

# 10	Mobility

## 10.0	General

Load balancing is achieved in E-UTRAN with handover, redirection mechanisms upon RRC release, DC and through the usage of inter-frequency and inter-RAT absolute priorities and inter-frequency Qoffset parameters.

Measurements to be performed by a UE for mobility are classified in at least four measurement types:

-	Intra-frequency E-UTRAN measurements;

-	Inter-frequency E-UTRAN measurements;

-	Inter-RAT measurements for UTRAN and GERAN;

-	Inter-RAT measurements of CDMA2000 HRPD or 1xRTT frequencies.

For each measurement type one or several measurement objects can be defined (a measurement object defines e.g. the carrier frequency to be monitored).

For each measurement object one or several reporting configurations can be defined (a reporting configuration defines the reporting criteria). Three reporting criteria are used: event triggered reporting, periodic reporting and event triggered periodic reporting.

The association between a measurement object and a reporting configuration is created by a measurement identity (a measurement identity links together one measurement object and one reporting configuration of same RAT). By using several measurement identities (one for each measurement object, reporting configuration pair) it is possible:

-	To associate several reporting configurations to one measurement object and;

-	To associate one reporting configuration to several measurement objects.

The measurements identity is as well used when reporting results of the measurements.

Measurement quantities are considered separately for each RAT.

Measurement commands are used by E-UTRAN to order the UE to start measurements, modify measurements or stop measurements.

For NB-IoT:

-	Handover, measurement reports and inter-RAT mobility are not supported;

-	10.1.1 Mobility Management in ECM-IDLE, 10.1.4 Paging and C-plane establishment, 10.1.5 Random Access Procedure, 10.1.6 Radio Link Failure, 10.1.7 Radio Access Network Sharing and all their clauses are applicable;

-	10.2.x Idle mode Inter-RAT Cell Selection to/from NB-IoT is supported;

-	All other subclauses of clause 10 are not applicable.

Next change

### 10.1.4	Paging and C-plane establishment

Paging groups (where multiple UEs can be addressed) are used on PDCCH:

-	Precise UE identity is found on PCH;

-	DRX configurable via BCCH and NAS, for NB-IoT DRX configurable via BCCH only;

-	Only one subframe allocated per paging interval per UE;

-	The network may divide UEs to different paging occasions in time;

-	There is no grouping within paging occasion;

-	One paging RNTI for PCH.

When extended DRX (eDRX) is used in idle mode, the following are applicable:

-	The DRX cycle is extended up to and beyond 10.24s in idle mode, with a maximum value of 2621.44 seconds (43.69 minutes); For NB-IoT, the maximum value of the DRX cycle is 10485.76 seconds (2.91 hours);

-	The hyper SFN (H-SFN) is broadcast by the cell and increments by one when the SFN wraps around;

-	Paging Hyperframe (PH) refers to the H-SFN in which the UE starts monitoring paging DRX during a Paging Time Window (PTW) used in ECM-IDLE. The PH is determined based on a formula that is known by the MME/AMF, UE and eNB as a function of eDRX cycle and UE identity;

-	During the PTW, the UE monitors paging for the duration of the PTW (as configured by NAS) or until a paging message is including the UE's NAS identity received for the UE, whichever is earlier. The possible starting offsets for the PTW are uniformly distributed within the PH and defined in TS 36.304 [11];

-	MME/AMF uses the formulas defined in TS 36.304 [11] to determine the PH as well as the beginning of the PTW and sends the S1 paging request just before the occurrence of the start of PTW or during PTW to avoid storing paging messages in the eNB;

-	ETWS, CMAS, PWS requirement may not be met when a UE is in eDRX. For EAB, if the UE supports SIB14, when in extended DRX, it acquires SIB14 before establishing the RRC connection;

-	When the eDRX cycle is longer than the system information modification period, the UE verifies that stored system information remains valid before establishing an RRC connection. Paging message can be used for system information change notification, when including systemInfoModification-eDRX, for a UE configured with eDRX cycle longer than the system information modification period.

NB-IoT UEs, BL UEs or UEs in enhanced coverage can use WUS, when configured in the cell, to reduce the power consumption related to paging monitoring.

When WUS is used in idle mode, the following are applicable:

-	The WUS is used to indicate that the UE shall monitor MPDCCH or NPDCCH to receive paging in that cell;

-	For a UE not configured with extended DRX, the WUS is associated to one paging occasion (N = 1);

-	For a UE configured with extended DRX, the WUS can be associated to one or multiple paging occasion(s) (N ≥ 1) in a PTW;

-	If UE detects the WUS, the UE shall monitor the following N paging occasions unless it has received a paging message;

-	The paging operation in the MME is not aware of the use of the WUS in the eNB.

The timing between WUS and the paging occasion (PO) is illustrated in Figure 10.1.4-1. The UE can expect WUS repetitions during "Configured maximum WUS duration" but the actual WUS transmission can be shorter, e.g. for UE in good coverage. The UE does not monitor WUS during the non-zero "Gap".

# Figure 10.1.4-1: Illustration of WUS timing

For NB-IoT, UE in RRC_IDLE receives paging on the anchor carrier or on a non anchor carrier based on system information.

Next change

#### 10.1.5.0	General

The random access procedure is characterized by:

-	Common procedure for FDD and TDD;

-	One procedure irrespective of cell size and the number of serving cells when CA is configured;

The random access procedure is performed for the following events related to the PCell:

-	Initial access from RRC_IDLE;

-	RRC Connection Re-establishment procedure, as defined in TS 24.301 [20];

-	Handover, except for NB-IoT or when RACH-less HO is configured;

-	DL data arrival during RRC_CONNECTED requiring random access procedure:

-	E.g. when UL synchronisation status is "non-synchronised".

-	UL data arrival during RRC_CONNECTED requiring random access procedure:

-	E.g. when UL synchronisation status is "non-synchronised" or there are no PUCCH resources for SR available.

-	For positioning purpose during RRC_CONNECTED requiring random access procedure:

-	E.g. when timing advance is needed for UE positioning.

The random access procedure is also performed on a SCell to establish time alignment for the corresponding sTAG.

For E-UTRA connected to 5GC, the random access procedure is also performed for the transition from RRC_INACTIVE.

In DC, the random access procedure is also performed on at least PSCell upon SCG addition/modification, if instructed, or upon DL/UL data arrival during RRC_CONNECTED requiring random access procedure. The UE initiated random access procedure is performed only on PSCell for SCG.

Furthermore, the random access procedure takes two distinct forms:

-	Contention based (applicable to all six events, but the sixth event for positioning is applicable for NB-IoT only);

-	Non-contention based (applicable to only handover, DL data arrival, positioning and obtaining timing advance alignment for a sTAG).

Normal DL/UL transmission can take place after the random access procedure.

An RN supports both contention-based and non-contention-based random access. When an RN performs the random access procedure, it suspends any current RN subframe configuration, meaning it temporarily disregards the RN subframe configuration. The RN subframe configuration is resumed at successful random access procedure completion.

For NB-IoT, the random access procedure is performed on the anchor carrier or on a non anchor carrier based on system information.

#### 10.1.5.1	Contention based random access procedure

The contention based random access procedure is outlined on Figure 10.1.5.1-1 below:

# Figure 10.1.5.1-1: Contention based Random Access Procedure

The four steps of the contention based random access procedures are:

1)	Random Access Preamble on RACH in uplink:

-	There are two possible groups defined and one is optional. If both groups are configured the size of message 3 and the pathloss are used to determine which group a preamble is selected from. The group to which a preamble belongs provides an indication of the size of the message 3 and the radio conditions at the UE. The preamble group information along with the necessary thresholds are broadcast on system information.

2)	Random Access Response generated by MAC on DL-SCH:

-	Semi-synchronous (within a flexible window of which the size is one or more TTI) with message 1;

-	No HARQ;

-	Addressed to RA-RNTI on PDCCH;

-	Conveys at least RA-preamble identifier, Timing Alignment information for the pTAG, initial UL grant and assignment of Temporary C-RNTI (which may or may not be made permanent upon Contention Resolution);

-	Intended for a variable number of UEs in one DL-SCH message.

3)	First scheduled UL transmission on UL-SCH:

-	Uses HARQ;

-	Size of the transport blocks depends on the UL grant conveyed in step 2.

-	For initial access:

-	Conveys the RRC Connection Request generated by the RRC layer and transmitted via CCCH;

-	Conveys at least NAS UE identifier but no NAS message;

-	RLC TM: no segmentation.

-	For RRC Connection Re-establishment procedure:

-	Conveys the RRC Connection Re-establishment Request generated by the RRC layer and transmitted via CCCH;

-	RLC TM: no segmentation;

-	Does not contain any NAS message.

-	After handover, in the target cell:

-	Conveys the ciphered and integrity protected RRC Handover Confirm generated by the RRC layer and transmitted via DCCH;

-	Conveys the C-RNTI of the UE (which was allocated via the Handover Command);

-	Includes an uplink Buffer Status Report when possible.

-	For other events:

-	Conveys at least the C-RNTI of the UE;

-	In the procedure to resume the RRC connection or in the EDT procedure for User Plane CIoT EPS/5GS Optimisations:

-	Conveys the RRC Connection Resume Request generated by the RRC layer and transmitted via CCCH;

-	Conveys a Resume ID to resume the RRC connection;

-	For the EDT procedure for User Plane CIoT EPS/5GS Optimisations:

-	Conveys ciphered user data transmitted via DTCH;

-	RLC UM/AM: no segmentation;

-	Does not contain any NAS message.

-	For NB-IoT:

-	In the procedure to setup the RRC connection:

-	An indication of the amount of data for subsequent transmission(s) on SRB or DRB can be indicated.

-	For EDT for Control Plane CIoT EPS/5GS Optimisations:

-	Conveys the RRC Early Data Request generated by the RRC layer and transmitted via CCCH;

-	Conveys NAS UE identifier and user data concatenated in a NAS message;

-	RLC TM: no segmentation.

4)	Contention Resolution on DL:

-	Early contention resolution shall be used i.e. eNB does not wait for NAS reply before resolving contention;

-	For NB-IoT, for initial access, RRC connection resume procedure and RRC Connection Re-establishment procedure, eNB may transmit MAC PDU containing the UE contention resolution identity MAC control element without RRC response message;

NOTE:	In Release 13, NB-IoT UEs do not support the MAC PDU containing the UE contention resolution identity MAC control element without RRC response message for initial access, RRC connection resume procedure and RRC Connection Re-establishment procedure.

-	Not synchronised with message 3;

-	HARQ is supported;

-	Addressed to:

-	The Temporary C-RNTI on PDCCH for initial access and after radio link failure;

-	The C-RNTI on PDCCH for UE in RRC_CONNECTED.

-	HARQ feedback is transmitted only by the UE which detects its own UE identity, as provided in message 3, echoed in the Contention Resolution message;

-	For initial access, RRC Connection Re-establishment procedure and EDT for Control Plane CIoT EPS/5GS Optimisations, no segmentation is used (RLC-TM).

The Temporary C-RNTI is promoted to C-RNTI for a UE which detects RA success and does not already have a C-RNTI; it is dropped by others. A UE which detects RA success and already has a C-RNTI, resumes using its C-RNTI.

When CA is configured, the first three steps of the contention based random access procedures occur on the PCell while contention resolution (step 4) can be cross-scheduled by the PCell.

When DC is configured, the first three steps of the contention based random access procedures occur on the PCell in MCG and PSCell in SCG. When CA is configured in SCG, the first three steps of the contention based random access procedures occur on the PSCell while contention resolution (step 4) can be cross-scheduled by the PSCell.

Next change

### 10.1.6	Radio Link Failure

Two phases govern the behaviour associated to radio link failure as shown on Figure 10.1.6-1:

-	First phase:

-	started upon radio problem detection;

-	leads to radio link failure detection;

-	no UE-based mobility;

-	based on timer or other (e.g. counting) criteria (T1).

-	Second Phase:

-	started upon radio link failure detection or handover failure;

-	leads to RRC_IDLE;

-	UE-based mobility;

-	Timer based (T2).

# Figure 10.1.6-1: Radio Link Failure

Table 10.1.6-1 below describes how mobility is handled with respect to radio link failure:

# Table 10.1.6-1: Mobility and Radio Link Failure

| Cases | First Phase | Second Phase | T2 expired |
| --- | --- | --- | --- |
| UE returns to the same cell | Continue as if no radio problems occurred | Activity is resumed by means of explicit signalling between UE and eNB | Go via RRC_IDLE |
| UE selects a different cell from the same eNB | N/A | Activity is resumed by means of explicit signalling between UE and eNB | Go via RRC_IDLE |
| UE selects a cell of a prepared eNB (NOTE) | N/A | Activity is resumed by means of explicit signalling between UE and eNB | Go via RRC_IDLE |
| UE selects a cell of a different eNB that is not prepared (NOTE) | N/A | Go via RRC_IDLE | Go via RRC_IDLE |
| NOTE: a prepared eNB is an eNB which has admitted the UE during an earlier executed HO preparation phase, or obtains the UE context during the Second Phase. |  |  |  |

For a NB-IoT UE that only uses Control Plane CIoT EPS Optimisations, as defined in TS 24.301 [20] and does not support RRC Connection re-establishment for the control plane as defined in TS 36.331 [16], at the end of the first phase, the UE enters RRC_IDLE (there is no second phase).In the Second Phase, in order to resume activity and avoid going via RRC_IDLE when the UE returns to the same cell or when the UE selects a different cell from the same eNB, or when the UE selects a cell from a different eNB, the following procedure applies:

-	The UE stays in RRC_CONNECTED;

-	The UE accesses the cell through the random access procedure;

-	Except for a NB-IoT UE using only Control Plane CIoT EPS Optimisations, the UE identifier used in the random access procedure for contention resolution (i.e. C-RNTI of the UE in the cell where the RLF occurred + physical layer identity of that cell + short MAC-I based on the keys of that cell) is used by the selected eNB to authenticate the UE and check whether it has a context stored for that UE:

-	If the eNB finds a context that matches the identity of the UE, or obtains this context from the previously serving eNB, it indicates to the UE that its connection can be resumed;

-	If the context is not found, RRC connection is released and UE initiates procedure to establish new RRC connection. In this case UE is required to go via RRC_IDLE.

-	For a NB-IoT UE using only Control Plane CIoT EPS Optimisations, the UE identifier used in the random access procedure for contention resolution (i.e. S-TMSI of the UE at the time where the RLF occurred + UL NAS MAC + UL NAS COUNT) is used by the selected eNB to request the MME to authenticate the UE's re-establishment request and provide the UE context:

-	If the authentication of the UE is successful and a context is provided, it indicates to the UE that its connection can be resumed;

-	If no context is provided, the RRC connection is released and UE initiates procedure to establish new RRC connection. In this case UE is required to go via RRC_IDLE.

Editor’s Note: It is FFS whether RRC Connection re-establishment for the control plane is supported for NB-IoT connected to 5GC.

The radio link failure procedure applies also for RNs, with the exception that the RN is limited to select a cell from its DeNB cell list. Upon detecting radio link failure, the RN discards any current RN subframe configuration (for communication with its DeNB), enabling the RN to perform normal contention-based RACH as part of the re-establishment. Upon successful re-establishment, an RN subframe configuration can be configured again using the RN reconfiguration procedure.

For DC, PCell supports above phases. In addition, the first phase of the radio link failure procedure is supported for PSCell. However, upon detecting RLF on the PSCell, the re-establishment procedure is not triggered at the end of the first phase. Instead, UE shall inform the radio link failure of PSCell to the MeNB.

NOTE:	If the recovery attempt in the second phase fails, the details of the RN behaviour in RRC_IDLE to recover an RRC connection are up to the RN implementation.

Next change

### 10.2.x	Idle mode Inter-RAT Cell Selection to/from NB-IoT

NB-IoT may provide assistance information for inter-RAT cell selection to E-UTRAN/GERAN and E-UTRAN may provide assistance information for inter-RAT cell selection to NB-IoT. A UE may use the assistance information provided by the network for cell selection to/from NB-IoT.

Next change

## 11.0	General

In order to utilise the SCH resources efficiently, a scheduling function is used in MAC. In this clause, an overview of the scheduler is given in terms of scheduler operation, signalling of scheduler decisions, and measurements to support scheduler operation.

For NB-IoT, the Basic Scheduler Operation in 11.1 and the uplink buffer status reports part in 11.3 are applicable, the UE-AMBR part in 11.4 is applicable only for UE which is enabled to use S1-U data transfer or User Plane CIoT EPS Optimisation (EPC) or for UE which is enabled to use NG-U data transfer or User Plane CIoT 5GS Optimisation (5GC), and all other subclauses of clause 11 are not applicable.

Next change

## 11.x	DL channel quality reporting in NB-IoT

The DL channel quality report in RRC_IDLE is defined by the following characteristics:

-	The reporting is configured by eNB via system information;

-	The report is related to the DL carrier used for the initial random access procedure;

-	The report is carried in the RRC message during the random access procedure.

The DL channel quality report in RRC_CONNECTED is defined by the following characteristics:

-	The reporting is triggered by the eNB via a MAC Control Element;

-	The report is related to the configured DL carrier used in unicast transmission;

-	The report is carried in a MAC Control Element.

Next change

## 14.1	Overview and Principles

The following principles apply to E-UTRAN security:

-	The keys used for NAS and AS protection shall be dependent on the algorithm with which they are used.

-	The eNB keys are cryptographically separated from the EPC keys used for NAS protection (making it impossible to use the eNB key to figure out an EPC key).

-	For SCG bearers in DC, the SeNB keys are cryptographically separated from the eNB keys.

-	The AS (RRC and UP) and NAS keys are derived in the EPC/UE from key material that was generated by a NAS (EPC/UE) level AKA procedure (KASME) and identified with a key identifier (KSIASME).

-	For SCG bearers in DC, the AS (UP) keys are derived in the SeNB/UE from key material that was generated in the MeNB/UE.

-	The eNB key (KeNB) is sent from the EPC to the eNB when the UE is entering ECM-CONNECTED state (i.e. during RRC connection or S1 context setup).

-	For SCG bearers in DC, the SeNB key (S-KeNB) is sent from the MeNB to the SeNB when adding an SCG.

-	For LWA bearers, the WT Counter, if included in LWA Configuration, is used when computing the S-KWT (as specified in TS 33.401 [22], clause G and TS 36.331 [16], clause 5.6.14.2). If WT Counter is not signalled to the UE, the UE uses authentication methods specified in TS 33.402 [70], clause 6, as described in 22A.1.8.

-	For LWIP, the LWIP Counter in the LWIP Configuration is used when computing the LWIP-PSK (as specified in TS 33.401 [13], clause A.13, and TS 36.331 [16], subcause 5.6.17.2).

-	Separate AS and NAS level security mode command procedures are used. AS level security mode command procedure configures AS security (RRC and user plane) and NAS level security mode command procedure configures NAS security. Both integrity protection and ciphering for RRC are activated within the same AS SMC procedure. User plane ciphering is activated at the same time as RRC ciphering.

-	Keys stored inside eNBs shall never leave a secure environment within the eNB (except when done in accordance with this or other 3GPP specifications), and user plane data ciphering/deciphering shall take place inside the secure environment where the related keys are stored.

-	Key material for the eNB keys is sent between the eNBs during ECM-CONNECTED intra-E-UTRAN mobility and from the MeNB to the SeNB in DC for SCG bearer during SCG addition and SCG change.

-	A sequence number (COUNT) is used as input to the ciphering and integrity protection. A given sequence number must only be used once for a given eNB key (except for identical re-transmission) on the same radio bearer in the same direction. The same sequence number can be used for both ciphering and integrity protection.

-	A hyper frame number (HFN) (i.e. an overflow counter mechanism) is used in the eNB and UE in order to limit the actual number of sequence number bits that is needed to be sent over the radio. The HFN needs to be synchronized between the UE and eNB.

-	No integrity protection initialisation number (FRESH).

-	Since SIM access is not granted in E-UTRAN TS 33.401 [22] except for making IMS Emergency calls, idle mode UE not equipped with USIM shall not attempt to reselect to E-UTRAN unless it is originating an IMS Emergency call. The RNC may try to prevent handover to E-UTRAN for example by identifying a SIM based UE from the security keys provided by the CN.

A simplified key derivation is depicted on Figure 14.1-1 below, where:

-	KNASint is a key, which shall only be used for the protection of NAS traffic with a particular integrity algorithm This key is derived by UE and MME from KASME , as well as an identifier for the integrity algorithm.

-	KNASenc is a key, which shall only be used for the protection of NAS traffic with a particular encryption algorithm. This key is derived by UE and MME from KASME, as well as an identifier for the encryption algorithm.

-	KeNB is a key derived by UE and MME from KASME. KeNB may also be derived by the target eNB from NH at handover. KeNB shall be used for the derivation of KRRCint, KRRCenc and KUPenc, and for the derivation of KeNB* upon handover.

-	KeNB* is a key derived by UE and source eNB from either KeNB or from a fresh NH. KeNB* shall be used by UE and target eNB as a new KeNB for RRC and UP traffic.

-	KUPenc is a key, which shall only be used for the protection of UP traffic with a particular encryption algorithm. This key is derived by UE and eNB from KeNB, as well as an identifier for the encryption algorithm.

-	KRRCint is a key, which shall only be used for the protection of RRC traffic with a particular integrity algorithm. KRRCint is derived by UE and eNB from KeNB, as well as an identifier for the integrity algorithm.

-	KRRCenc is a key, which shall only be used for the protection of RRC traffic with a particular encryption algorithm. KRRCenc is derived by UE and eNB from KeNB as well as an identifier for the encryption algorithm.

-	Next Hop (NH) is used by UE and eNB in the derivation of KeNB* for the provision of "forward security", as specified in TS 33.401 [22]. NH is derived by UE and MME from KASME and KeNB when the security context is established, or from KASME and previous NH, otherwise.

-	Next Hop Chaining Count (NCC) is a counter related to NH (i.e. the amount of Key chaining that has been performed) which allow the UE to be synchronised with the eNB and to determine whether the next KeNB* needs to be based on the current KeNB or a fresh NH.

# Figure 14.1-1: Key Derivation

Key derivation for SCG bearers in DC is depicted on Figure 14.1-2 below, where:

-	SCG Counter is a counter used as freshness input into S-KeNB derivations (see TS 33.401 [22], Annex E.2.4).

# Figure 14.1-2: DC Key Derivation

The MME invokes the AKA procedures by requesting authentication vectors to the HE (Home environment) if no unused EPS authentication vectors have been stored. The HE sends an authentication response back to the MME that contains a fresh authentication vector, including a base-key named KASME. Thus, as a result of an AKA run, the EPC and the UE share KASME. From KASME, the NAS keys, (and indirectly) KeNB keys and NH are derived. The KASME is never transported to an entity outside of the EPC, but KeNB and NH are transported to the eNB from the EPC when the UE transitions to ECM-CONNECTED. From the KeNB, the eNB and UE can derive the UP and RRC keys.

RRC and UP keys are refreshed at handover. KeNB* is derived by UE and source eNB from target PCI, target frequency and KeNB (this is referred to as a horizontal key derivation and is indicated to UE with an NCC that does not increase) or from target PCI, target frequency and NH (this is referred to as a vertical key derivation and is indicated to UE with an NCC increase). KeNB* is then used as new KeNB for RRC and UP traffic at the target. When the UE goes into ECM-IDLE all keys are deleted from the eNB except for the UE which was enabled to use User Plane CIoT EPS Optimisation.

For SCG Bearers in DC, UP keys are updated at SCG change by indicating in RRC signalling to the UE the value of the SCG Counter to be used in key derivation. When KeNB is refreshed, SCG Counter shall be reset and S-KeNB shall be newly derived from the KeNB.

COUNT reusing avoidance for the same radio bearer identity in RRC_CONNECTED mode without KeNB change is left to eNB implementation e.g. by using intra-cell handover, smart management of radio bearer identities or triggering a transition to RRC_IDLE.

SCG bearers in DC share a common pool of radio bearer identities (DRB IDs) together with the MCG bearers and when no new DRB ID can be allocated for an SCG bearer without guaranteeing COUNT reuse avoidance, the MeNB shall derive a new S-KeNB. SeNB indicates to MeNB when uplink or downlink PDCP COUNTs are about to wrap around and MeNB shall update the S-KeNB. To update the S-KeNB, the MeNB increases the SCG Counter and uses it to derive a new S-KeNB from the currently active KeNB in the MeNB. The MeNB sends the newly derived S-KeNB to the SeNB. The newly derived S-KeNB is then used by the SeNB in computing a new encryption key KUPenc which is used with all DRBs in the SeNB for this UE. Furthermore, when the SCG Counter approaches its maximum value, the MeNB refreshes the currently active KeNB, before any further S-KeNB is derived.

In case of HFN de-synchronisation in RRC_CONNECTED mode between the UE and eNB, the UE is pushed to IDLE.

Next change

### 14.3.2	RRC_CONNECTED to RRC_IDLE

Except for the UE which was enabled to use User Plane CIoT EPS Optimisation, on RRC_CONNECTED to RRC_IDLE transitions, eNBs shall delete the keys they store such that state for idle mode UEs only has to be maintained in MME. It is also assumed that eNB does no longer store state information about the corresponding UE and deletes the current keys from its memory. In particular, on connected to idle transitions:

-	The eNB and UE deletes NH, KeNB , KRRCenc , KRRCint and KUPenc and related NCC.

-	MME and UE keeps KASME, KNASint and KNASenc stored.

On RRC_CONNECTED to RRC_IDLE transitions for the UE which was enabled to use User Plane CIoT EPS Optimisation, the eNBs, the UE and MME shall maintain the keys they store.

Next change

# 18	UE capabilities

RRC signalling carries AS capabilities and NAS signalling carries NAS capabilities. The UE capability information is stored in the MME. In the uplink, except of NB-IoT no capability information is sent early in e.g. RRCConnectionRequest message. For NB-IoT, early indications for multi-tone support (IOT bit) and multi-carrier support (IOT bit) are sent in RRCConnectionRequest-NB message. In the downlink, enquiry procedure of the UE capability is supported.

# Figure 18-1: Initial UE Capability Handling

The MME stores the UE Radio Capability uploaded in the UE CAPABILITY INFO INDICATION message.

The possible RAT-Types are: EUTRAN, UTRAN, GERAN-PS, GERAN-CS, CDMA2000-1XRTT. The GERAN capability is divided into separate parts. MS Classmark 2 and Classmark 3 are used for CS domain (in both AS and NAS) and MS Radio Access Capability is used for PS domain. The main part of CDMA2000 capabilities is not handled by the eNB or the MME, but is exchanged via tunnelling (see 10.3.2). The small part of CDMA2000 capabilities (for CDMA2000-1XRTT) is needed for the eNB to be able to build messages for the target CDMA2000 RNC (see 10.3.2).

The eNB may acquire the UE capabilities after a Handover completion. The UE capabilities are then uploaded to the MME.

Usually during handover preparation, the source RAN node transfers both the UE source RAT capabilities and the target RAT capabilities to the target RAN node, in order to minimize interruptions and to follow the principles in clause 10.2.2. The source RAN is not mandated to acquire other RAT capabilities (i.e. other than the source and target RAT capabilities) in order to start a handover preparation. This is described in clause 19.2.2.5.6. However, there are exceptions to this principle:

-	For handover from GERAN to EUTRAN, due to limitations in GERAN radio interface signalling, source RAT (GERAN) never provides the EUTRA capabilities to the target RAN node.

-	At handover from UTRAN to EUTRAN, it is optional to forward the UTRAN capabilities to the target RAN.

The UTRAN capabilities, i.e. the INTER RAT HANDOVER INFO, include START-CS, START-PS and "predefined configurations", which are "dynamic" IEs. In order to avoid the START values desynchronisation and the key replaying issue, the eNB always enquiry the UE UTRAN capabilities at transition from RRC_IDLE to RRC_CONNECTED and before Handover to UTRAN. The eNB does not upload the UE UTRAN capabilities to the MME.

Due to limitations in radio interface signalling, transfer of EUTRA capabilities is not supported in GERAN.

For a NB-IoT UE that supports S1-U data transfer or User Plane CIoT EPS Optimisation, as defined in TS 24.301 [20], the procedure in Figure 18-1 is applicable except that RAT-Types and handover are not supported.

If a request to retrieve the UE Radio Capability is included in the DOWNLINK NAS TRANSPORT message, the eNB may request the UE Radio Capability from the UE and provide it to the MME in the UE CAPABILITY INFO INDICATION message. The detailed procedure is defined in TS 36.413 [25].

For a UE that supports Control Plane CIoT EPS Optimisation, as defined in TS 24.301 [20], the MME may initiate Connection Establishment Indication procedure to provide UE Radio Capability to the eNB after receiving INITIAL UE MESSAGE message. If the UE Radio Capability is not included in the procedure, this may trigger the eNB to request the UE Radio Capability from the UE and to provide it to the MME in the UE CAPABILITY INFO INDICATION message. The detailed procedure is defined in TS 36.413 [25].

In NB-IoT, for a UE that supports Control Plane CIoT EPS Optimisations, as defined in TS 24.301 [20], the eNB, based on configuration, may retrieve the UE Radio capability from the MME upon reception of RRC Connection Request as defined in TS 23.401 [17], clauses 5.3.4B.2 and 5.3.4B.3.

Next change

##### 19.2.2.2.1	S1 UE Context Release (EPC triggered)

# Figure 19.2.2.2.1-1: S1 UE Context Release procedure (EPC triggered)

-	The EPC initiates the UE Context Release procedure by sending the S1 UE Context Release Command towards the E-UTRAN. The eNodeB releases all related signalling and user data transport resources.

-	The eNB confirms the S1 UE Context Release activity with the S1 UE Context Release Complete message. The behaviour of the eNodeB in case of Control Plane CIoT EPS Optimisation is specified in TS 23.401 [11].

-	In the course of this procedure the EPC releases all related resources as well, except context resources in the EPC for mobility management and the default EPS Bearer/E-RAB configuration.

Next change

#### 19.2.2.25	Connection Establishment Indication procedure

# Figure 19.2.2.25-1: Connection Establishment Indication procedure

The Connection Establishment Indication procedure is used in case of CIoT EPS Optimisation and it enables the MME to provide information to the eNB to complete the establishment of the UE-associated logical S1-connection and/or to trigger the eNB to obtain and report UE Radio Capability.

The MME may trigger Connection Establishment Indication procedure either

-	after receiving INITIAL UE MESSAGE message, if the MME has no NAS PDU to send in DL in case of Control Plane CIoT EPS Optimisation, or

-	after receiving an eNB CP RELOCATION INDICATION message in case of Control Plane CIoT EPS Optimisation, or

-	when deciding to trigger the eNB to obtain and report UE Radio Capability.

The UE Radio Capability may be provided from the MME to the eNB in this procedure. If the UE radio capability is not included, this may trigger the eNB to request the UE Radio Capability from the UE and to provide it to the MME in the UE CAPABILITY INFO INDICATION message.

NAS-level security information may be provided from the MME to the eNB in this procedure in case the UE-associated logical S1-signalling connection is established for a NB-IOT UE using Control Plane CIoT EPS Optimisations after the MME receives an eNB CP RELOCATION INDICATION message from the eNB, as described in TS 33.401 [22].

This procedure is initiated by the MME.

Next change

#### 19.2.2.30	eNB CP Relocation Indication

The eNB CP Relocation Indication procedure is only applicable for NB-IOT UEs using Control Plane CIoT EPS Optimisations. The purpose of the eNB CP Relocation Indication procedure is to request the MME to authenticate the UE's re-establishment request as described in TS 33.401 [22], and initiate the establishment of the UE-associated logical S1-connection after the UE has initiated a RRC Re-establishment procedure in an eNB.

When the eNB receives the RRCConnectionReestablishmentRequest message, it triggers the eNB CP Relocation Indication procedure including NAS-level security information received from the UE. If the MME authenticates the request, it initiates the Connection Establishment Indication procedure including NAS-level security information to be sent to the UE in the RRCConnectionReestablishment message. In case the MME cannot authenticate the UE's request, the CONNECTION ESTABLISHMENT INDICATION message does not contain security information, and the eNB shall fail the RRC Re-establishment. In case of authentication failure, the eNB and the MME should locally release the allocated S1 resources, if any.

# Figure 19.2.2.30-1: eNB CP Relocation Indication procedure (highlighted in blue)

# Interactions with the MME CP Relocation and UE Context Release procedures:

In case of successful UE authentication, the MME initiates the UE Context Release procedure to release the UE's old S1-connection. The MME may initiate the MME CP Relocation procedure before the release procedure in order to trigger the return of non-delivered NAS PDUs to the MME.

Next change

#### 19.2.2.31	MME CP Relocation Indication

The MME CP Relocation Indication procedure is only applicable for UEs using Control Plane CIoT EPS Optimisations. The purpose of the MME CP Relocation Indication procedure is to inform the previously serving eNB that the UE's connection is to be relocated to a new eNB.

Upon reception of the MME CP RELOCATION INDICATION message, the old eNB shall terminate the delivery of downlink NAS PDUs towards the UE, and initiate NAS Non Delivery Indication procedure(s) to report the non-delivery of any NAS PDUs previously received from the MME.

# Figure 19.2.2.31-1: MME CP Relocation Indication procedure (highlighted in blue).

Next change

# 22	Support for self-configuration and self-optimisation

## 22.3	Self-configuration

### 22.3.2a	Automatic Neighbour Relation Function

Editor’s Note: Expectation is that this section becomes applicable to NB-IoT without change. Need to be confirmed by RAN3.

The purpose of the Automatic Neighbour Relation (ANR) function is to relieve the operator from the burden of manually managing Neighbour Cell Relations (NCRs). Figure 22.3.2a-1 shows ANR and its environment:

Figure 22.3.2a-1: Interaction between eNB and O&M due to ANR

The ANR function resides in the eNB and manages the conceptual Neighbour Cell Relation Table (NCRT). Located within ANR, the Neighbour Detection Function finds new neighbours and adds them to the NCRT. ANR also contains the Neighbour Removal Function which removes outdated NCRs. The Neighbour Detection Function and the Neighbour Removal Function are implementation specific.

A Neighbour Cell Relation (NCR) in the context of ANR is defined as follows:

An existing Neighbour Relation from a source cell to a target cell means that eNB controlling the source cell:

a)	Knows the ECGI/CGI and PCI of the target cell.

b)	Has an entry in the Neighbour Cell Relation Table for the source cell identifying the target cell.

c)	Has the attributes in this Neighbour Cell Relation Table entry defined, either by O&M or set to default values.

For each cell that the eNB has, the eNB keeps a NCRT, see Figure 22.3.2a-1. For each NCR, the NCRT contains the Target Cell Identifier (TCI), which identifies the target cell. For E-UTRAN, the TCI corresponds to the E-UTAN Cell Global Identifier (ECGI) and Physical Cell Identifier (PCI) of the target cell. Furthermore, each NCR has three attributes, the NoRemove, the NoHO and the NoX2 attribute. These attributes have the following definitions:

-	No Remove: If checked, the eNB shall not remove the Neighbour Cell Relation from the NRT.

-	No HO: If checked, the Neighbour Cell Relation shall not be used by the eNB for handover reasons.

-	No X2: If checked, the Neighbour Relation shall not use an X2 interface in order to initiate procedures towards the eNB parenting the target cell.

Neighbour Cell Relations are cell-to-cell relations, while an X2 link is set up between two eNBs. Neighbour Cell Relations are unidirectional, while an X2 link is bidirectional.

NOTE:	The neighbour information exchange, which occurs during the X2 Setup procedure or in the eNB Configuration Update procedure, may be used for ANR purpose.

The ANR function also allows O&M to manage the NCRT. O&M can add and delete NCRs. It can also change the attributes of the NCRT. The O&M system is informed about changes in the NCRT.

Next change

### 22.3.4x	Automatic Neighbour Relation Function in NB-IoT

The ANR (Automatic Neighbour Relation) function relies on cells broadcasting their identity on global level, E-UTRAN Cell Global Identifier (ECGI).

# Figure 22.3.4x-1: Automatic Neighbour Relation Function in case of NB-IoT

The purpose of SON/ANR reporting in NB-IoT is network optimisation. The measurements are performed when the UE is in RRC_IDLE and reported next time the UE enters RRC_CONNECTED. ANR measurement reporting is not supported when the UE uses the Control Plane CIoT EPS Optimisation.

The function works as follows:

The eNB serving cell A has an ANR function. During connected mode, the eNB can configure the UE to perform measurements on a frequency and read the CGI of the strongest cell if the quality is above a given RSRP threshold. The eNB may use different policies for instructing the UE to do measurements.

1	When releasing the RRC connection, the eNB configures the UE to perform ANR measurements on one or more frequencies. The RRC connection is released and the UE enters RRC_IDLE.

When the UE is in RRC_IDLE and remains camped on the cell from which the ANR measurement configuration was received, the UE performs the ANR measurements requested by the eNB:

2a	For each of the configured frequency, the UE performs measurements, identifies the strongest cell and stores the cell measurement results for later reporting.

2b	For each of the configured frequency, if the NRSRP of the strongest cell is above the configured threshold, the UE reads the ECGI, the TAC and all available PLMN ID(s) of the related neighbour cell and stores the information for later reporting.

NOTE:		The measurement rules for cell re-selection and the relaxed monitoring measurement rules as specified in TS 36.304 [11] do not apply while performing an ANR measurement.

When the UE establishes or resumes the RRC connection:

3	The UE reports the availability of an ANR report.

When the eNB receives the indication of the ANR report availability, the following sequence may be used whilst UE is in RRC_CONNECTED mode:

4a	The eNB requests the UE to provide the report.

4b	The UE reports the stored cells and associated information.

When the UE returns to RRC_IDLE, if the UE has indicated the availability of the ANR report, the UE discards the old ANR configuration and ANR report.

Next change

#### 22.4.2.x	Connection failure due to Radio Link Failure in NB-IoT

In NB-IoT, the function of Mobility Robustness Optimization is to detect connection failures due to radio link failure.

Solution for failure scenarios consists of one or more of following functions:

-	Detection of the failure after RRC re-establishment attempt;

-	Detection of the failure after RRC connection setup;

-	Retrieval of information needed for problem analysis.

Triggering of each of these functions is optional and depends on situation and implementation.

Detection of the failure after RRC re-establishment attempt:

UE provides the RLF Report to the eNB after successful RRC connection re-establishment.

Detection of the failure after RRC connection setup:

In case the RRC connection re-establishment fails or the UE does not perform any RRC connection re-establishment, the UE makes the RLF Report available to the eNB after reconnecting from idle mode. Availability of the RLF Report at the RRC connection setup procedure is the indication that a RLF failure occured and that the RLF Report from this occurence could be obtained by the network.

Retrieval of information needed for problem analysis

The information needed for detailed problem analysis may be retrieved from both, the UE and the network sides. The information that is collected at the UE is provided to the network with the RLF Report.

The RLF Report from the UE includes the following information:

-	The E-CGI of the last cell that served the UE.

-	The radio measurements of the last cell that served the UE.

-	Time elapsed from the connection failure till RLF Report signalling.

Editor’s Note: Handling of the connection failure in the eNB is up to RAN3.

Next change

### 22.4.3	Support for RACH Optimisation

The setting of RACH parameters that can be optimized are:

-	RACH configuration (resource unit allocation);

-	RACH preamble split (among dedicated, group A, group B, RSRP level, NRSRP level (for NB-IoT), EDT);

-	RACH backoff parameter value;

-	RACH transmission power control parameters.

RACH optimization is supported by UE reported information and by PRACH parameters or NPRACH parameters (for NB-IoT) exchange between eNBs.

UEs which receive polling signalling shall report the below information:

-	Number of RACH preambles sent until the successful RACH completion;

-	Contention resolution failure;

-	For BL UE or UE in enhanced coverage or NB-IoT UE, the RSRP (NRSRP for NB-IoT) level in which the UE started the random access procedure;

-	For BL UE or UE in enhanced coverage or NB-IoT UE, an EDT fallback indication.

Next change

### 22.4.5	Radio Link Failure report

The RLF Report from the UE can be used for both coverage optimization and mobility robustness optimization.

The UE stores the latest RLF or, except for NB-IoT, handover failure related information, and indicates RLF report availability at each subsequent LTE RRC connection (re-)establishment and, except for NB-IoT, handover to an LTE cell until the RLF report is fetched by the network or for 48 hours after the RLF or handover failure is detected.

The UE keeps the information during state transitions and RAT changes, and indicates RLF report availability again after it returns to the LTE RAT.

The UE only indicates RLF report availability and only provides the RLF report to the network if the current RPLMN is a PLMN that was present in the UE's EPLMN List or was the RPLMN at the time the RLF or handover failure was detected.

Editor’s Note: Need to be discussed that all above also applies to NB-IoT.

Next change

# 24	Support for 5GC

## 24.1	General

The E-UTRA connected to 5GC is supported as part of NG-RAN. The E-UTRA can be connected to both EPC and 5GC.

The overall architecture of E-UTRA connected to 5GC as part of NG-RAN is described in TS 38.300 [79], where the term " ng-eNB" is used for E-UTRA connected to 5GC. However, in this specification the term "eNB" is used for both cases unless there is a specific need to disambiguate between eNB and ng-eNB.

E-UTRA connected to 5GC supports the following functions:

-	5G NAS message transport (see clause 7.3);

-	5G security framework (see TS 38.300 [79]), except that data integrity protection is not supported;

-	Access Control (see TS 38.300 [79]);

-	Flow-based QoS (see TS 38.300 [79]);

-	Network slicing (see TS 38.300 [79]);

-	SDAP (see TS 37.324 [80]), except for NB-IoT;

-	NR PDCP (see TS 38.323 [81]);

-	Support of UEs in RRC_INACTIVE state, except for NB-IoT;

-	CIoT signalling reduction optimisations for NB-IoT UEs (see clause 7.3a);

-	MO-EDT for NB-IoT UEs (see clause 7.3b).

Editor’s note: It is FFS whether NR PDCP is supported for NB-IoT connected to 5GC

Editor’s note: It is FFS whether the Automatic Neighbour Relation Function in NB-IoT is supported for NB-IoT connected to 5GC.

## 24.2	Radio Protocol Architecture

### 24.2.1	User Plane

The figure below shows the protocol stack for the user plane, where SDAP, NR PDCP, RLC and MAC sublayers perform the functions listed in clause 6.5 of TS 38.300 [79], clause 6.4 of TS 38.300 [79], clause 6.3, and clause 6.2 respectively.

# Figure 24.2.1-1: User Plane Protocol Stack

NOTE 1:	For a NB-IoT UE, SDAP is bypassed.

NOTE:	For a NB-IoT UE that only supports Control Plane CIoT 5GS Optimisation, as defined in TS 25.401 [xx], PDCP is bypassed. For a NB-IoT UE that supports Control Plane CIoT 5GS Optimisation and N3 data transfer or User Plane CIoT 5GS Optimisation, as defined in TS 25.401 [xx], PDCP is also bypassed (i.e. not used) until AS security is activated.

### 24.2.2	Control Plane

The figure below shows the protocol stack for the control plane, where:

-	NR PDCP sublayer (terminated in ng-eNB on the network side) performs the functions listed for the control plane in clause 6.4 of TS 38.300 [79]. The UE uses only NR PDCP for both SRBs and DRBs when connected to 5GC;

-	RLC and MAC sublayers (terminated in ng-eNB on the network side) perform the functions listed in clause 6.2 and 6.1;

-	RRC (terminated in ng-eNB on the network side) performs the functions listed in clause 7;

-	NAS control protocol (terminated in AMF on the network side) performs the functions listed in TS 23.501 [82], for instance: authentication, mobility management, security control.

# Figure 24.2.2-1: Control Plane Protocol Stack

## 24.3	Layer 2

The layer 2 of E-UTRA connected to 5GC is split into the following sublayers: Medium Access Control (MAC), Radio Link Control (RLC), NR Packet Data Convergence Protocol (PDCP) and Service Data Adaptation Protocol (SDAP).

-	The physical layer offers to the MAC sublayer transport channels, see clause 5;

-	The MAC sublayer offers to the RLC sublayer logical channels, see clause 6.1;

-	The RLC sublayer offers to the NR PDCP sublayer RLC channels, see clause 6.2;

-	The NR PDCP sublayer offers to the SDAP sublayer data radio bearers, and offers to the RRC sublayer signalling radio bearers, see TS 38.323 [81];

-	The SDAP sublayer offers to 5GC QoS flows, see TS 37.324 [80].

## 24.4	CN Selection

For a cell that provides E-UTRA connectivity to both 5GC and EPC within a PLMN, the UE upper layer performs CN selection between EPC and 5GC. The UE AS layer indicates available CN type(s) to upper layers for CN type selection and in addition for NB-IoT, the supported CIoT features. The UE NAS layer indicates selected CN type (if available) with selected PLMN during PLMN selection procedure, as defined in TS 36.304 [11].

## 24.5	Mobility

Intra-EUTRA inter-system Handover (i.e., handover between E-UTRA connected to 5GC and E-UTRA connected to EPC) is described in clause 10.2.2c and in TS 23.502 [83].

The inter-RAT intra-5GC Handover (i.e., handover between E-UTRA connected to 5GC and NR connected to 5GC) is described in clause 9.3.1.2 of TS 38.300 [79].

Inter-RAT handover to/from GERAN/UTRAN/CDMA2000 and cell change order to GERAN with NACC are not supported, and CS fallback described in clause 10.2.5 is not applied except for the functionality of release with redirection to GERAN/UTRAN.

The following mobility procedures are supported:

-	RRC Connection Release with Redirection to GERAN/UTRAN/CDMA2000/EUTRAN;

-	Cell Change Order to GERAN without NACC.

When the UE is connected to E-UTRA/5GC, inter system fallback towards E-UTRAN is performed when 5GC does not support emergency services, voice services, for load balancing etc. Depending on factors such as CN interface availability, network configuration and radio conditions, the fallback procedure results in either RRC CONNECTED state mobility (handover procedure) or RRC IDLE state mobility (redirection), see TS 23.501 [82] and TS 36.331 [16].

Except for NB-IoT, in the N2 signalling procedure, the AMF based on support for emergency services, voice service, any other services or for load balancing etc, may indicate the target CN type as EPC or 5GC to the ng-eNB node. When the target CN type is received by ng-eNB, the target CN type is also conveyed to the UE in RRC Connection Release message.

The mobility in RRC_INACTIVE is described in clause 10.1.9.

For E-UTRA connected to 5GC, in RRC_IDLE the UE monitors the PCCH for CN-initiated paging information, in RRC_INACTIVE the UE monitors the PCCH for RAN-initiated and CN-initiated paging information. The RAN-initiated and CN-initiated paging occasions overlap and the same paging mechanism is used for both. Except for NB-IoT, the extended DRX (eDRX) is not used for E-UTRA connected to 5GC.

## 24.6	Slicing

E-UTRA connected to 5GC supports network slicing functionality. The details of network slicing are specified in TS 23.501 [82] and clause 16.3 of TS 38.300 [79].

## 24.7	Access Control

E-UTRA connected to 5GC supports unified access control (UAC) functionality. The details of UAC are defined in TS 38.300 [79].

For E-UTRA connected to both EPC and 5GC, E-UTRA broadcasts the access control information associated with EPC and 5GC separately and the UE AS uses the access control information associated with the core network type selected by NAS.