3GPP TSG RAN WG2#107bis	R2-1912026

Chongqing, China, 14 – 18 October, 2019

3GPP TSG RAN WG1 #98	R1-1909900

Prague, CZ, August 26th – 30th, 2019

Title:	Reply LS on supported BW for initial BWP

Response to:	R1-1908010/R2-1908301

Release:	Release 15

Work Item:	NR_NewRAT-Core

Source:	TSG RAN WG1

To:	TSG RAN WG2, TSG RAN WG4

Cc:	-

Contact Person:

#### Name:	Karri Ranta-aho

E-mail Address:	Karri.Ranta-aho@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 would like to thank RAN2 on the LS on supported BW for initial BWP in R1-1908010/R2-1908301.

The LS had three questions addressed both to RAN1 and RAN4, and RAN1 answer is provided below

Which BWP-bandwidths is a UE is expected to support: Only the BWP-bandwidths matching exactly the supported channel bandwidths or also values less than the exact channel bandwidth (possibly including any value - in number of PRBs - lower than the supported channel bandwidths)?

RAN1 view:

  - As per the RAN1 UE feature 6-1, all UEs are required to support “at least BWPs which is the same as the set of specified channel BW” without any capability indication.

  - Specifically, for initial DL BWP, all possible sizes of CORESET 0 (24, 48, 96 PRBs) need to be supported by all UEs.

  - RAN1 specifications have the flexibility to support any BWP size from 1 PRB to 275 PRBs, although currently Rel-15 does not support BWP sizes smaller than the RBG size or the PRG size.

Can the network make any assumptions regarding supported initial BWP bandwidths (when UE capabilities are not yet known)?

RAN1 view:

  - RAN1’s understanding is that all UEs must be able to support all possible CORESET 0 sizes and in addition all RAN4-specified UE channel BWs and all the corresponding transmission bandwidths (e.g. for 20 MHz channel bandwidth and 30 kHz SCS the maximum transmission BW defined by RAN4 is 51 PRBs) for a given band deemed mandatory by RAN4, subject to capability signalling channelBWs-DL and channelBWs-UL (not known to the network before capability information exchange). RAN4 may provide information on the status of these existing capability bits.

  - Whether the network can assume other bandwidths of initial DL/UL BWP for all UEs based on the mandatory UE channel bandwidths defined in RAN4 should be left to RAN4.

Does the RAN2 agreement that UEs shall support an initial UL BWP bandwidth equal to CORESET#0 have any impact to RAN1/4 specifications?

RAN1 view:

  - No.

2. Actions to RAN WG2 and RAN WG4

RAN1 respectfully asks RAN2 and RAN4 to take the information provided into account.

3. Dates of Next TSG-RAN WG1 Meetings:

3GPP RAN1#98bis		14 – 20 October 2019	Chongqing, P.R. China

3GPP RAN1#99		18 – 22 November 2019	Reno, NV, USA