3GPP TSG RAN WG2 #107bis	 R2-1912638

Chongqing, China, 14 – 18 October, 2019

Agenda Item:	12.5 Other LTE Rel-16 WIs

Source: 	Ericsson

Title:  	Discussion on incoming LS for QoE measurement collection in LTE

Document for:	Discussion and Decision

# Introduction

RAN2 has received an LS from SA5 related to QoE measurement collection, see [3]. SA5 has performed a large study of QoE measurement collection and the result is described in and asks other working groups to implement each part respectively. Here the impacts on RAN2 related to the LS are discussed.

# Discussion

## QoE Measurement Collection

Quality of Experience Measurement Collection enables configuration of application layer measurements in the UE and transmission of QoE measurement result files by means of RRC signalling. Application layer measurement configuration received from OAM or CN is encapsulated in a transparent container, which is forwarded to UE in a downlink RRC message. Application layer measurements received from UE's higher layer are encapsulated in a transparent container and sent to network in an uplink RRC message. The result container at forwarded to a TCE, Trace Collector Entity. In Figure 1, the RRC signalling flow for QoE measurements in LTE are shown.

Figure 1: QoE measurement collection in LTE

## LS impacts

In [6] and [7] the QoE feature is described. The following two signalling diagrams in [7] contain the activation and deactivation of QoE measurements and the handling at handover.

Figure 4.3.1-1: QMC activation and reporting in LTE

Figure 4.3.2.2-1: Handling of QMC activation in case of handover in LTE

Also, further details are described and the issues with RAN2 impacts are analysed below.

### Several QoE measurements

Due to an error in ASN.1, it is currently not possible to activate different types of QoE measurements at the same time. In RAN2#105 RAN2 the CR in [2] was discussed but postponed as it contained an NBC change. Companies also thought a correction was not needed in release-15, but could wait until release-16. In SA groups the different types of QoE measurements are independent measurements and it is understood that the different types can be activated at the same time. This is captured in [6], 5.1.1:

REQ-EUSPC-CON-8: The operator shall have a capability to order several QoE measurement collections from each UE simultaneously.

RAN2 is therefore asked to correct the ASN.1 error so that several QoE measurements can be activated at the same time.

Proposal 1: Correct the ASN.1 error so that several QoE measurements can be activated at the same time.

### WithinArea

RAN2 previously received an LS from SA5 asking RAN2 to add an inside area indication to be sent to the UE at each handover. The area in which the UE should perform the QoE measurements, e.g. a list of cells, is never sent to the UE in order to avoid RRC signalling overhead, see also LS [2]. That means that it is up to the network to ensure that the measurements are started and stopped at the right time. One thing that complicates this is that SA4 has a requirement that once a measurement has been started, it should continue until the end of the session even if the UE exists the area in the middle of the session. The reason is that if measurements are stopped in the middle of the session the report cannot be used. It is explicitly stated in 26.247 and 26.114 that a started measurement shall continue until end of the session:

26.247, clause 10.1:

The QoE configuration shall only be checked by the client when each session starts, and thus all logging and reporting criterias for an ongoing session shall be unaffected by any QoE configuration changes received during that session. This also includes evaluation of any filtering criterias, such as geographical filtering, which shall only be done when the session starts. Thus changes to the QoE configuration will only affect sessions started after these configuration changes have been received.

26.114, clause 16.3:

The QoE configuration shall only be checked by the client when each session starts, and thus all logging and reporting criterias for an ongoing session shall be unaffected by any QoE configuration changes received during that session. This also includes evaluation of any filtering criterias, such as geographical filtering, which shall only be done when the session starts. Thus changes to the QoE configuration will only affect sessions started after these configuration changes have been received.

These requirements mean that if the new cell is inside the area, the UE should continue the measurements in the new cell. If the new cell is not inside the area, the UE should finalize the measurements for the ongoing session, but not start any new measurements.

eNB knows when the UE comes to an area border, but it doesn’t know when the measurements start and stop as the measurements are performed in the application layer in the UE. If the eNB would trigger release of the measurements at the area border, the measurements would be interrupted in the middle of the session.

If the eNB waits with releasing the measurements, the eNB doesn’t know when the measurements end and cannot release them at the right time. eNB may wait until the UE enters RRC_IDLE, but then it may happen that the UE starts new measurements outside the area while it is still in RRC_CONNECTED, which is not the desired behaviour.

The inside area indication has previously been discussed in RAN2 and some companies have said that it can be solved by eNB implementation. However, it has never been explained at what point eNB should stop the measurements in order to achieve the behaviour as specified in 26.114. RAN2 is therefore kindly asked to clarify when the eNB should stop the measurements in order to achieve the behaviour as specified in 26.114.

Proposal 2: Discuss and clarify when eNB should stop the QoE measurements in order not to interrupt any ongoing measurements and to avoid the UE from starting new measurements outside the area.

If RAN2 can explain how the wanted behaviour can be achieved, a reply LS can be sent to SA5 with the explanation. If no explanation or alternative solution can be given, the inside area should be added according to the request from SA5 and according to what has been specified in 26.114 and 28.405.

Proposal 3: Add the withinArea in the signalling at handover according to the incoming LS or give an explanation on how the problem can be solved otherwise.

### Streaming indication

In [7] SA5 has also concluded the need for a streaming indication. The purpose of the streaming indication is to inform the OAM server that a streaming session has started and that a QoE report is to be expected. This is to ensure that the content of the configuration files cannot be incorrect without it being discovered. A question that arises is why there only is an indication for streaming services and not for other services, e.g. MTSI service, and whether SA5 would like the indication to be applicable for any type of service (and maybe have a more generic name) or whether they would like one indication for each type of service. It is therefore proposed that RAN2 discusses the streaming indication and possibly asks SA5 to clarify the use of it.

Proposal 4: Discuss the streaming indication and possibly ask SA5 to clarify whether it is applicable for all service types.

### Temporary stop of measurements

In [7] SA5 the use of a temporary stop of measurements is described:

### 4.3.4	Temporary stop and restart of QoE information reporting during RAN overload in LTE

In case of overload in RAN, the eNB may temporarily stop the reportingfrom the UE by send the RRCConnectionReconfiguration message [9] to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to temporarily stop application layer measurement reporting in otherConfig [9]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the temporary stop request. The application stops the reporting and stops recording further information when the data in the reporting container is used. Then the recorded data is kept until it is reported or when the UE request session is ended.

When the overload situation in RAN is ended the eNB restart the reporting from the UE by send the RRCConnectionReconfiguration message [9] to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to restart application layer measurement reporting in otherConfig [9]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the restart request. The application restarts the reporting and recording if it was stopped.

The benefit of using temporary stop instead of releasing the QoE measurements is that the UE can continue doing the QoE measurements, but not send any reports if the network is overloaded. When the overload situation is ended the reports can be sent again. Also, the QoE configuration file does not have to be sent again after the overload situation, which decreases the amount of signalling.

Proposal 5: Add the temporary stop and restart of QoE measurements.

### UE capability

The QoE feature in release 15 is already frozen, so it is proposed to add a UE capability for the support of the release 16 additions.

Proposal 6: Add a UE capability for the support the release 16 additions for QoE measurements.

### CR

A CR with the additions related to the LS is provided in [8] and a reply LS is provided in [9]. RAN2 is kindly asked to discuss and agree on a CR and an LS response.

# Summary

RAN2 is kindly asked to discuss the following proposal:

Proposal 1: Correct the ASN.1 error so that several QoE measurements can be activated at the same time.

Proposal 2: Discuss and clarify when eNB should stop the QoE measurements in order not to interrupt any ongoing measurements and to avoid the UE from starting new measurements outside the area.

Proposal 3: Add the withinArea in the signalling at handover according to the incoming LS or give an explanation on how the problem can be solved otherwise.

Proposal 4: Discuss the streaming indication and possibly ask SA5 to clarify whether it is applicable for all service types.

Proposal 5: Add the temporary stop and restart of QoE measurements.

Proposal 6: Add a UE capability for the support the release 16 additions for QoE measurements.

# References

- RP-181752, Revised SID:Study on RAN-centric data collection and utilization for LTE and NR, September 2018

- R2-1804086, Reply LS on Attributes for QoE measurement collection

- R2-1900096, Reply LS on Attributes for QoE measurement collection (S5-187524; contact: Ericsson)

- R2-1906230, Attributes for QoE measurement collection in LTE

- S5-195659, LS on QoE Measurement Collection

- TS 28.404, Quality of Experience (QoE) measurement collection, Concepts, use cases and requirements

- TS 28.405, Quality of Experience (QoE) measurement collection, Control and configuration

- R2-1912639, Corrections of QoE measurement collection

- R2-1912640, Draft reply LS on Management of QoE measurement collection