3GPP TSG-RAN WG2 Meeting #107bis	R2-1913221

Chongqing,  China, 14 – 18 October 2019

Agenda item:	6.13.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Preamble groups for 2-step RACH

WID/SID:	NR_2step_RACH-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

After the previous RAN1#98, the following conclusion was achieved over email approval:

Agreement: - For RRC_INACTIVE/IDLE state, at least support up to two msgA PUSCH configurations for Rel.16 o Using different preamble groups for the indications of different configurations in case of two configurations o Support of more than two configurations is not precluded, and if supported FFS the following mechanisms for the indications of different configurations § Alt.1: Using different preamble groups § Alt 2: Using different preamble groups and/or RO partitioning § Alt.3: Using UCI based indication § Alt. 4: Using different DMRS ports/sequences - At least up to two msgA PUSCH configurations are supported for RRC_CONNECTED state for Rel.16 o FFS details - FFS whether the MsgA PUSCH configurations are the same among different RRC states (IDLE, INACTIVE, CONNCETED) - FFS the rule or BS signaling the criterion for the UE’s selection of msgA PUSCH configuration

Hence, it seems preamble groups A and B would be supported for 2-step RA similarly to 4-step RA.

Furthermore, RAN2#107-Bis agreed the following:

Agreements:

- If the random access procedure is not successfully completed even after transmitting the msgA 'N' times, UE fallbacks to 4 step RACH procedure i.e. UE only transmits the PRACH preamble.

- Network can configure the number of times 'N', a UE can attempt to re-transmit msgA during the random access procedure.

Agreements

- TB size offered in UL grant in the fallback RAR shall be the same as the TB size offered for payload transmission in MsgA; otherwise, the UE behavior is not defined (i.e. it is up to UE implementation).

# 2	Discussion

## 2.1	Preamble groups for 2-step RACH

Given the RAN1 agreement to support at least 2 preamble groups for indication of used 2-step MsgA PUSCH configuration, it seems similar principle as applied currently for 4-step preamble groups A and B selection can be applied. The following is an excerpt of the corresponding 4-step preamble group selection from TS 38.321:

### 5.1.2 Random Access Resource selection (…) 2> if Msg3 has not yet been transmitted: 3> if Random Access Preambles group B is configured: 4> if the potential Msg3 size (UL data available for transmission plus MAC header and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB; or 4> if the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-Msg3SizeGroupA: 5> select the Random Access Preambles group B. 4> else: 5> select the Random Access Preambles group A. 3> else: 4> select the Random Access Preambles group A.

It should noted that in case UE selected 2-step RA type based on the agreed RSRP threshold, this may not necessarily mean the same cell quality for the Preamble group B selection. For the purpose of the data threshold, similar parameter to ra-Msg3SizeGroupA can be introduced for the 2-step RA, e.g., ra-MsgASizeGroupA.

Proposal 1: Introduce preambles group A and B for 2-step RACH.

Proposal 2: Apply the same selection formulas to select between 2-step preambles group A and B as specified for 4-step in Rel-15. For the purpose of data threshold, ra-MsgASizeGroupA parameter can be introduced.

## 2.2	Fallback from 2-step RA to 4-step RA

As the UE can fallback to 4-step RA procedure after ‘N’ retries over 2-step RACH, the agreement on TB size offered in UL grant for Msg3 via 4-step Msg2 shall also be the same as used for the MsgA PUSCH. This will limit the applicability of 2-step RACH in case the preamble groups A and B need to be of equal size in both the 2-step and 4-step RACH to ensure UE won’t end up receiving UL grant size in Msg3 different as used for the MsgA PUSCH when it falls back to 4-step after ‘N’ retries over 2-step RACH.

Observation 1: Applicability of 2-step RACH will be limited in case the preamble groups A and B need to result of equal size grants both in 2-step and 4-step RACH.

Given that the 2-step RACH was designed to support low latency as well as possibly be able to carry some small UP data already along with the MsgA, it seems to be a meaningful restriction in case the UL grant sizes for preamble groups A and B for both 2-step RACH and 4-step RACH need to be coordinated – meaning in practice 56 bits and 72 bits for group A and B, respectively, to allow maximum coverage within a cell with 4-step RA. Furthermore, as the RSRP threshold is applied for selecting the 2-step RACH, it already can be used to enable higher payload sizes to be used in MsgA transmission of 2-step RA than used in the corresponding Msg3 transmission of 4-step RA.

Observation 2: The RSRP threshold to select between 2-step and 4-step RA would allow higher payload sizes to be used for MsgA transmission of 2-step RA than used in the corresponding Msg3 transmission of 4-step RA.

To allow some flexibility for the NW to use different grant sizes for MsgA transmissions of 2-step than used for Msg3 transmissions of 4-step RA, it should be able to be configured the fallback from 2-step RA to 4-step RA after ‘N’ retries over 2-step RACH is only allowed for one of the preambles group A and B used in the 2-step RACH. For instance, if the NW used 72 bits grant for 4-step preamble group B and 2-step preamble group A, the UE would be allowed to fallback to 4-step RA only if it used preamble group A for 2-step RA. With this approach, a higher payload for MsgA with preamble group B could be allowed. Naturally, also an option where fallback to 4-step is not allowed at all by the NW should be supported in which case the grant sizes for MsgA in 2-step RA could be different for both preamble groups A and B than for the corresponding preamble groups A and B for 4-step RA.

Proposal 3: Support configuration where fallback from 2-step RA to 4-step RA after ‘N’ retries over 2-step RACH is only allowed for one of the preamble groups A or B in 2-step RA.

Proposal 4: Support configuration where fallback from 2-step RA to 4-step RA is not allowed, ie., when the number ‘N’ is not configured.

# 3	Conclusion

In this contribution, the need for preamble groups for 2-step RACH was discussed as well as the needed coordination between the preamble groups of 4-step RACH.

Proposal 1: Introduce preambles group A and B for 2-step RACH.

Proposal 2: Apply the same selection formulas to select between 2-step preambles group A and B as specified for 4-step in Rel-15. For the purpose of data threshold, ra-MsgASizeGroupA parameter can be introduced.

Proposal 3: Support configuration where fallback from 2-step RA to 4-step RA after ‘N’ retries over 2-step RACH is only allowed for one of the preamble groups A or B in 2-step RA.

Proposal 4: Support configuration where fallback from 2-step RA to 4-step RA is not allowed, ie., when the number ‘N’ is not configured.