3GPP TSG-RAN WG2 Meeting #107Bis	R2- 1913911

# Chongqing, China, 14 – 18 October 2019	                  (Revision of R2-1911310)

Agenda item:	6.9.2

Source:	Samsung

Title:	Single stack eMOB solution for 1Rx NR UEs

WID/SID:	NR_Mob_enh-Core - Release 16

Document for:	Discussion & Decision

# 1	Introduction

The following was agreed at RAN2#107 meeting [1]:

Agreements

1	Introduce a solution for HO interruption time reduction based on dual active protocol stack.

The dual active protocol stack (DAPS) solution agreed above is applicable to 2Rx NR UEs and not applicable to 1Rx NR UEs. The remaining issue is if interruption time is not reduced for 1Rx NR UEs then Rel-15 normal HO applies which does not meet ~0 ms interruption time requirement.

# 2	Discussion

2.1 	Overview of LTE MBB/eMBB solutions

Make-before-break (MBB) is a simple solution from Rel-14 LTE based on single protocol stack, where the UE maintains a logical connection with one cell at a time. The only fundamental difference when compared to the legacy handover is that the UE keeps exchanging data with its source node until it is ready to connect to the target node (and the exact moment of time when a UE actually switches is up to the UE hardware capabilities and limitations). Specification wise, MBB feature involving RAN2 and RAN3 impact, the overall specification impact was not high as we managed to leverage most of the existing mobility procedures in TS 36.300. It is worth noting that Rel-14 LTE MBB feature has the formal assumption of 1 Rx1 TX UE architecture, which is further governed by performance requirements and scenarios in which it can be applied. In other words, the 1Rx UE architecture assumption does not allow to realize the 0ms interruption time, neither it is possible to assume that MBB is used for the inter-frequency/inter-band handover. To overcome some of these limitations and restrictions, enhanced MBB solutions was extensively discussed in the corresponding Rel-16 LTE WI with the assumption of dual Rx UE architecture. At RAN2#106 meeting it was agreed to specify dual active protocol stack (DAPS) solution with specified capability coordination that does not have to be utilized by the network [2] for LTE eMBB. The single stack solution for LTE eMBB was ruled out. Therefore, in LTE the 1Rx UEs still can benefit from the Rel-14 MBB solution and the Rel-16 LTE eMBB solution based on DAPS applies to 2Rx UEs.

Observation#1: For enhanced MBB (i.e. LTE eMBB in Rel-16) RAN2 agreed dual active protocol solution with specified capability coordination to benefit the 2Rx LTE UEs.

Observation#2: For the 1RX LTE UEs the Rel-14 MBB solution applies with which it is still possible to achieve ~0ms interruption time and there are restrictions on scenarios where MBB can be applied.

2.2 	NR MBB solutions

Both LTE and NR RATs are positioned as candidate IMT-2020 technology in ITU which need to qualify the requirements set by ITU for IMT-2020 [3]. From an UE/NW implementation criteria perspective we believe the solutions selected for LTE and NR mobility enhancements should be aligned and harmonized as much as possible to ease the development efforts and specification impacts. Adopting aligned LTE and NR mobility enhancement solutions result in similar outcome at the application layer for both LTE and NR RATs from an IMT-2020 requirement perspective. With this principle at RAN2#107 meeting RAN2 agreed to adopt the DAPS solution for the NR UE architecture based on 2Rx, however there is no enhancement for the 1Rx NR UEs.

Observation#3: For NR in Rel-16 RAN2 agreed dual active protocol solution with specified capability coordination to benefit the 2Rx NR UEs but there is no solution to benefit the 1Rx NR UEs.

The NR UEs which are 1Rx capable cannot benefit from the DAPS solution to reduce the interruption time. The performance of such 1Rx NR UEs will be based on Rel-15 mobility procedure. It is desirable that performance of such UEs is enhanced both in terms of interruption reduction and mobility robustness. The conditional handover (CHO) solution brings mobility robustness regardless of the UE capability. However, interruption time reduction can be leveraged relying on the single stack solution wherein the UE continuous Tx/Rx with source after reception of reconfiguration with sync. In our companion paper [4], we had proposed to adopt the RACH-less HO in NR. When the RACH-less HO is combined with the single stack solution the interruption time can be reduced to close to 0 ms. In scenarios where RACH-less is not possible, the NW implementation ensures CFRA is configured during handover to reduce the interruption time. In our companion paper [5], the specification impacts to TS 38.331 and TS 38.323 are analyzed to specify the single stack solution.

Observation#4: Single Rx NR UEs cannot benefit from the DAPS solution.

Observation#5: For single RX NR UEs, when single stack solution is combined with RACH-less or CFRA is configured the interruption time can be reduced to close to 0ms.

Therefore, based on the above observations we propose RAN2 to adopt the single stack solution for NR 1Rx UEs.

Proposal: Single protocol stack is specified for single Rx NR UEs.

# 3	Conclusion

We conclude the paper with following observations and proposals:

Observation#1: For enhanced MBB (i.e. LTE eMBB in Rel-16) RAN2 agreed dual active protocol solution with specified capability coordination to benefit the 2Rx LTE UEs.

Observation#2: For the 1RX LTE UEs the Rel-14 MBB solution applies with which it is still possible to achieve ~0ms interruption time and there are restrictions on scenarios where MBB can be applied.

Observation#3: For NR in Rel-16 RAN2 agreed dual active protocol solution with specified capability coordination to benefit the 2Rx NR UEs but there is no solution to benefit the 1Rx NR UEs.

Observation#4: Single Rx NR UEs cannot benefit from the DAPS solution.

Observation#5: For single RX NR UEs, when single stack solution is combined with RACH-less or CFRA is configured the interruption time can be reduced to close to 0ms.

Proposal: Single protocol stack is specified for single Rx NR UEs.

# 4	References

[1] R2-19xxxx, RAN2 Chairman Notes for RAN2#107 (Intel).

[2] R2-1908108, Report from session on Rel-16 LTE Mobility Enhancements WI and NR Mobility Enhancements WI, Session Chair (Nokia).

[3] REPORT ITU-R M.2410-0, Minimum requirements related to technical performance for IMT-2020 radio interface(s)

[4] R2-1911292, Support of RACHless HO in NR, Samsung.

[5] R2-1911309, Specification Impacts to specify Single Stack solution for NR, Samsung.