3GPP TSG-RAN WG2 Meeting #108                                	R2-1914615

Reno, USA, 18th – 22nd November 2019

Source: 	Asia Pacific Telecom

Title: 	DRX ambiguous period enhancements for NTN

Agenda item:	6.6.3.1

Document for:	Discussion and Decision

# Introduction

In RAN2#105, the following agreements related to DRX has been made.

Agreement in RAN2#105 - If HARQ is supported by NTN, the drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL, should be modified to support NTN. Agreement in RAN2#105 - A modification of drx-LongCycleStartOffset, drx-StartOffset, drx-ShortCycle, drx-ShortCycleTimer, drx-onDurationTimer, drx-SlotOffset, and drx-InactivityTimer is NOT needed to support NTN. - A modification of drx-RetransmissionTimerDL and drx-RetransmissionTimerDL is NOT needed to support NTN.

The status can be summarised that the feature of DRX will be supported by NTN, and most modifications on timers are not needed because the timer values were inspected to accommodate the round-trip time (RTT) of NTN system, except the drx-RTT timers.

However, the long RTT may impact DRX more than the discussed timers. In this contribution, we raise an issue for the DRX ambiguous period due to the long RTT in NTN.

# Discussion

In NR, DRX ambiguous period refers to a time interval that UE and NW have no common understanding of whether DRX MAC CE command is applied by the UE. In NR, this period includes the following two parts.

- The PDSCH decoding time (around 1ms);

- DRX MAC CE processing time (around 3ms);

The DRX ambiguous period is set to 4ms as a good trade-off value between UE power-saving and network (NW) blind decoding complexity, e.g., CSI report and SRS.

For example, as shown in Figure 1, DRX MAC CE is received before the DRX ambiguous period. For legacy DRX in NR, although DRX MAC CE is received before DRX ambiguous period, for NW, there are two cases.

- Case 1: if K1 > 4 ms, NW cannot know before symbol n whether UE has received the DRX MAC CE successfully or not. Thus, during the time between symbol n and the time NW receives the HARQ feedback, NW still needs to perform blind decoding for CSI report or SRS transmission.

- Case 2: if K1 < 4 ms, NW knows in symbol n whether UE has received the DRX MAC CE successfully. If UE has received the DRX MAC CE correctly, NW does not need to decode CSI report or SRS transmission in/after symbol n.

Figure 1. An example of DRX ambiguous period when receiving DRX MAC CE.

To summarize, in the NR Uu interface, NW can avoid the blind decoding with a good scheduling manner, that ensures NW receives HARQ feedback within the DRX ambiguous period.

However, in NTN, due to a long propagation delay, the legacy 4 ms of the DRX ambiguous period is too short to cover any propagation delay in NTN scenarios, e.g., 270 ms for GEO, 12 ms for LEO-600, and 20 ms for LEO-1200 with transparent payloads. As a price, NW may have a penalty period for blind decoding.

The DRX ambiguous period of 4 ms is too short to cover any propagation delay in NTN scenarios, which leads to a penalty for blind decoding at the NW side.

For example, as shown in Figure 2, we illustrate four timelines between NW and UE, where the same slot number is used for these four timelines, but the corresponding time will be different after timing advance (TA) compensation. The ideal ambiguous period shall cover the time NW receives the HARQ feedback, meanwhile it shall be as short as possible to facilitate UE starting the DRX operation for power saving.

Figure 2. an example of the ideal DRX ambiguous period in NTN.

Some notations in Figure 2 are given below:

- TA: timing advance;

- Tp: propagation delay;

- n: slot number for receiving a DCI scheduling a PDSCH;

- m: slot number for receiving the PDSCH carrying a DRX MAC CE;

- k: slot number for sending a HARQ feedback corresponding to the DRX MAC CE;

- K_0 and K_1: the scheduling offset in the NR Uu interface;

- K_offset: the new scheduling offset agreed in RAN1#98, used for covering TA.

To summarize, in NR NTN, NW may NOT be able to avoid the blind decoding if there is no modification on the legacy 4 ms of DRX ambiguous period used in the NR Uu interface.

If DRX is supported by Rel-17 NTN NWI, the DRX ambiguous period should be modified to accommodate the RTT for NTN systems.

# Conclusion

In this contribution, we have the following observations and proposals.

Observation 1	The DRX ambiguous period of 4 ms is too short to cover any propagation delay in NTN scenarios, which leads to a penalty for blind decoding at the NW side.

Based on the above observations, we then have the following proposals.

Proposal 1	If DRX is supported by Rel-17 NTN NWI, the DRX ambiguous period should be modified to accommodate the RTT for NTN systems.

# Reference

Chairman’s notes in RAN2#105, Athens, Greece, Feb. 2019.

Chairman’s notes in RAN1#98, Prague, Czech, Aug. 2019.