3GPP TSG-RAN WG2 Meeting #108	                                                  R2-1914616

Reno, USA, 18th - 22th November 2019

Agenda item:		6.16.4

Source:			Asia Pacific Telecom co. Ltd

Title:		Discussion on SCell BFR procedure

Document for:	 	Discussion and decision

# Introduction

In last meeting, RAN2 has a preliminary discussion for the design of SCell BFR procedure and made the following agreements.

RAN2 #107bis Agreement

- The Scell beam failure detection is per cell.

- Each DL BWP of a SCell can be configured with an independent SCell BFR configuration (the content is FFS)

- One SR ID is configured for BFR within the same cell group.

- The SCell BFRQ MAC CE triggers a SCell BFRQ SR if there is no valid uplink grant which can accommodate the SCell BFRQ MAC CE.

- FFS whether the transmission of the SCell BFRQ MAC CE cancels the pending BFRQ SR of the failed SCell(s).(depends whether the MAC CE provides info for one or more Scells)

- When the number of the BFRQ SR transmission reaches the sr-TransMax, the UE triggers a RACH procedure (i.e. reuse Rel-15 behaviour)

On the other hand, an email discussion [1] was raised to discuss more details of SCell BFR procedure (e.g., beam failure detection, BFR-SR modelling, SCell BFR MAC CE design, and SCell BFR termination, etc.). However, there are still some issues were not covered in the email discussion.

In this contribution, we are eager to discuss the issues for termination of SCell BFR procedure. Based on the observations, we propose to introduce a time window to improve latency and reliability for SCell BFR procedure.

# Discussion

## R-15 BFR procedure for SpCell

In R-15, a response window (i.e., ra-ResponseWindow) is configured for the UE to monitor BFR response. After the UE transmits the BFRQ (i.e., via RA preamble), the UE will start the ra-ResponseWindow. While the ra-ResponseWindow is running, the UE should continuously monitor PDCCH for the BFR response. If the UE receives the BFR response during the ra-ResponseWindow, the UE considers the RA procedure and BFR procedure successfully completed. However, if the ra-ResponseWindow expires and the UE has not received the BFR response, the UE could retransmit the BFRQ with higher power and with another candidate beam. In a nutshell, there are two objectives for the ra-ResponseWindow.  The first objective is to control the delay of BFR response. When the NW receives the BFRQ, the NW should provide the BFR response during the ra-ResponseWindow. Otherwise, the UE will consider that the NW does not receive the BFRQ and then reattempts the BFRQ transmission. The second purpose is to ensure the UE continuously monitors PDCCH during the ra-ResponseWindow to avoid missing the possible BFR response.

[2] Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall: 1> if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity: 2> start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission; 2> monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running.

Observation 1: In R-15 BFR procedure for SpCell, ra-ResponseWindow is used to control the delay of BFR response and ensure the UE continuously monitors PDCCH to avoid missing the possible BFR response.

## 2.2	R-16 SCell BFR procedure

In R-16, a SCell BFR procedure will be introduced. One of the differences from the R-15 BFR procedure is that the BFRQ is not transmitted via RA preamble, but rather transmitted via two steps of BFRQ. The first step of BFRQ is to transmit the BFR-SR (via PUCCH) to request for an UL grant for the second step of BFRQ. The second step of BFRQ is to transmit the SCell BFR MAC CE (via PUSCH) to carry the failed SCell(s) ID and candidate beam information. It is noted that the BFR-SR transmission is not mandatory, but rather depends on whether there is an available UL grant for SCell BFR MAC CE transmission.

On the other hand, the modelling of BFR response in R-16 is also distinct. In R-15, the BFR response is modelled as a signal transmitted on a recovery search space via a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI. Nonetheless, no dedicated search spaced would be applied in R-16 SCell BFR procedure. The BFR response for SCell BFR procedure is modelled as a normal uplink grant to schedule a new transmission for the same HARQ process as the PUSCH carrying the SCell BFR MAC CE, which is the same as normal “ACK” for PUSCH [3].

In current mechanism, there is no restriction on timeline for the NW to schedule an “ACK” for PUSCH. Hence, the NW may schedule a new transmission with the same HARQ process as the PUSCH carrying the SCell BFR MAC CE only when it is needed, e.g., when there is incoming DL/UL data.  In other words, it is possible that the transmission of BFR response is deferred, which then prolongs the whole SCell BFR procedure indefinitely. Furthermore, after transmitting the SCell BFR MAC CE, the UE may not always keep in Active Time to monitor PDCCH. For example, if the SCell BFR MAC CE is transmitted via configured grant, the drx-InactivityTimer will not be (re-)started. In light of this, the UE will lose some opportunities for PDCCH monitoring, hence an expected delay for the termination of SCell BFR procedure is inevitable, which also degrades the reliability for BFR response reception.

Observation 2: In R-16 SCell BFR procedure, currently there is no restriction to control the delay of BFR response, and the UE may not always keep in Active Time to monitor the BFR response, which will cause a delay for the termination of SCell BFR procedure and degrade the reliability for BFR response reception.

In order to avoid the unnecessary delay of SCell BFR procedure, inheriting the approach of response window from R-15 would be a favorable way. It can be noted that the BFRQ for SCell BFR procedure is divided into two steps. One alternative is to start the response window when transmitting the BFR-SR, and another alternative is to start the response window when transmitting the SCell BFR MAC CE. Based on the reply LS [3], if the UE already has an UL grant which could accommodate the SCell BFR MAC CE, the UE is not required to signal the BFR-SR. In addition, the BFR response would be transmitted after the UE transmits the SCell BFR MAC CE, so the UE only needs to monitor the BFR response after that. According to the above reasons, to start the response window upon transmitting the SCell BFR MAC CE is a more reasonable choice.

An illustration is shown in Figure 1, the UE starts the response window upon transmitting the SCell BFR MAC CE. While the response window is running, the UE monitors PDCCH to receive the BFR response. If the response window expires, and the UE could not receive the BFR response, the UE considers the BFR response reception not successful. If the BFR response is not successfully received, the UE should re-try the step 2 BFRQ transmission, e.g., the UE can generate a new SCell BFR MAC CE to transmit on a new PUSCH resource. With such response window, the overall latency of SCell BFR procedure can be controlled and the reliability is improved as well.

Figure 1: SCell BFR procedure with a response window

Proposal 1: To avoid an endless waiting for BFR response, a response window (similar to ra-ResponseWindow) should be introduced. The response window is started upon the transmission of SCell BFR MAC CE. While the response window is running, the UE monitors PDCCH for BFR response.

Proposal 2: If the response window expires and the BFR response has not been received, the UE considers the BFR response reception not successful. Then the UE generates a new SCell BFR MAC CE and transmits it via a new PUSCH resource.

# Conclusion

Based on the analysis shown aforementioned, we have the following observations and proposals.

Observation 1: In R-15 BFR procedure for SpCell, ra-ResponseWindow is used to control the delay of BFR response and ensure the UE keep monitoring PDCCH to avoid missing the possible BFR response.

Observation 2: In R-16 SCell BFR procedure, currently there is no restriction to control the delay of BFR response, and the UE may not always keep in Active Time to monitor the BFR response, which will cause a delay for the termination of SCell BFR procedure and degrade the reliability for BFR response reception.

Proposal 1: To avoid an endless waiting for BFR response, a response window (similar to ra-ResponseWindow) should be introduced. The response window is started upon the transmission of SCell BFR MAC CE. While the response window is running, the UE monitors PDCCH for BFR response.

Proposal 2: If the response window expires and the BFR response has not been received, the UE considers the BFR response reception not successful. Then the UE generates a new SCell BFR MAC CE and transmits it via a new PUSCH resource.

# Reference

[1] R2-191xxxx Summary of [107bisx#61][NR eMIMO] Scell BFR MAC CR (Nokia, Samsung)

[2] 3GPP TS 38.321 V15.7.0; Medium Access Control (MAC) protocol specification

[3] R2-1912013 Reply LS on MAC CE design for SCell BFR