# 3GPP TSG-RAN WG2 Meeting #108						R2-1914653

# Reno, USA, 18th – 22nd November 2019

Agenda Item:	6.4.2

Source:	MediaTek Inc.

Title:	Remaining issues of SL Prioritization

Document for:     Discussion and Decision

# Introduction

In RAN2#107BIS[1], agreement related to prioritization is reached as follows:

Agreements on prioritization:

1: 	A separate LCH priority thresholds is configured for both NR-UL and NR-SL.

2:	For between SL-data and UL-data/SRB, the SL transmission is prioritized if the highest priority value of UL LCH(s) with available data is larger than the UL priority threshold and the highest priority value of SL LCH(s) with available data is lower than the SL priority threshold. Otherwise the UL transmission is prioritized.

3:	Prioritization between UL SR and SL data transmission could be based on priority of the UL LCH that triggered the UL SR and priority value(s) of SL LCH(s), similar as prioritization between NR UL data and NR SL data transmission.

Agreements on flexible BSR:

1: 	If SL-BSR is prioritized, and if the UL-grant size is not enough to carry “the BS of all prioritized SL-BSR entries + UL-BSR”, transmit SL-BSR. Otherwise, rely on legacy behavior.

2:	The rule for UL-data/SL-data prioritization is reused for defining prioritized SL-BSR/UL-BSR.

In this paper, we discuss issues below:

- Revisit the agreement for multiplexing UL BSR and SL BSR

- Prioritization between UL BSR and NR SL data

- Prioritization between NR-UL and LTE-SL, and between LTE-UL and NR-SL

# Discussion

## Revisit the multiplexing method for flexible BSR

Based on RAN2#107BIS agreement, the multiplexing method for flexible BSR is illustrated in Table 1:

| Agreement | UL BSR + SL prioritized BSR < UL grant size | UL BSR + SL prioritized BSR >= UL grant size |
| --- | --- | --- |
| SL BSR is prioritized | [Agreement]Transmit SL BSR [Problem] UL BSR is not transmitted | [agreement] Follow legacy [Problem] if PHR is triggered, SL prioritized BSR may not be transmitted because in legacy, PHR is prioritized over SL BSR |
| UL BSR is prioritized | [agreement] Follow legacy | [agreement] Follow legacy |

Table 1: Method to multiplex UL BSR and SL BSR

We think the agreement makes much sense, but some minor issues can be further updated. When SL BSR is prioritized, depending on the UL grant size there are two cases:

- Case 1: the BS of all prioritized SL-BSR entries + UL-BSR < UL grant size => rely on legacy behavior

  - The legacy behaviour has a priority order UL BSR > PHR > SL BSR. This mean if at the same time there is a pending PHR for transmission, UE will transmit UL BSR and PHR first, and SL BSR is included only when the UL grant has remaining room after including UL BSR and PHR MAC CE. This is not our intention.

- Case 2: the BS of all prioritized SL-BSR entries + UL-BSR < UL grant size => transmit SL BSR

  - We understand that the intention to send SL BSR only is because non-padding UL BSR cannot be truncated based on current spec, so it is impossible to send SL BSR and UL BSR at the same time. However, if we follow current agreement, even if the SL BSR may include only a few BS with higher priority than UL BSR and many BS with lower priority than the SL BSR, the UL BSR is stilled dropped.

Observation: Following current agreement for multiplexing UL BSR and SL BSR, there are two issues:

- First, when SL BSR is prioritized, and the BS of all prioritized SL-BSR entries + UL-BSR is more than the UL grant size, if we follow legacy behaviour (PHR is prioritized over SL BSR), the prioritized part of SL BSR may not be able to transmit if p

- Second, when SL BSR is prioritized, and the BS of all prioritized SL-BSR entries + UL-BSR is less than the UL grant size, UL BSR cannot be transmitted even though the SL-BSR include many BS entries with a lower priority than UL BSR.

To address the two issues, we consider corrections below:

- First, when SL BSR is prioritized, and the BS of all prioritized SL-BSR entries + UL-BSR is more than the UL grant size, instead of just saying we follow legacy behaviour, we explicitly specify that the prioritized SL BSR entry and UL BSR would be included in the UL grant

Proposal 1: When SL BSR is prioritized, and the BS of all prioritized SL-BSR entries + UL-BSR is more than the UL grant size, UL BSR and truncated SL BSR are included in the UL grant (instead of follow legacy behaviour).

- Second, when SL BSR is prioritized, and the BS of all prioritized SL-BSR entries + UL-BSR is less than the UL grant size, the prioritized SL BSR entry and truncated UL BSR is included in the UL grant. The intention of including truncated UL BSR is to ensure that UL BSR can be transmitted as early as possible, in which the truncated UL BSR is of course thought as padding BSR and will not cancel the pending non-padding BSR.

Proposal 2: When SL BSR is prioritized, and the BS of all prioritized SL-BSR entries + UL-BSR is less than the UL grant size, prioritized SL BSR entry and UL BSR are included in the UL grant (instead of follow legacy behaviour).

|  | UL BSR + SL prioritized BSR < UL grant size | UL BSR + SL prioritized BSR >= UL grant size |
| --- | --- | --- |
| SL BSR is prioritized | [Agreement]Transmit SL BSR [Proposal] Transmit prioritized SL BSR and truncated UL BSR (ensure prioritized BS entry can be transmit asap) | [agreement] Follow legacy [Proposal] Transmit UL BSR and truncated SL BSR (same intention but consider the case of PHR) |
| UL BSR is prioritized | [agreement] Follow legacy | [agreement] Follow legacy |

Table 2: Proposed method to multiplex UL BSR and SL BSR

## Prioritization between UL BSR and NR SL data

In last meeting, it’s agreed we use the same rule to perform prioritization between NR UL data and NR SL data, and between NR UL BSR and NR SL BSR (by applying the two thresholds for NR UL and NR SL). Then, it is straightforward to reuse the same rule for prioritization between NR UL BSR and NR SL data, i.e. the priority of a UL BSR is equal to the priority of highest-priority UL LCH whose buffer status is included in the UL BSR.

Proposal 3: The rule for UL-data/SL-data prioritization is reused for UL-BSR/SL-data prioritization.

## Prioritization between NR-UL and LTE-SL and between NR-SL and LTE-UL

In last meeting, the prioritization rule between NR-UL and NR-SL is specified, i.e., both NR-UL and NR-SL are configured with a threshold for comparison between UL and SL. Based on this we can consider the prioritization rule between NR-UL and LTE-SL and between LTE-UL and NR-SL:

- Prioritization between NR-UL and LTE-SL

  - Both NR-SL and LTE-SL have the threshold for prioritization, which mean we can directly reuse the method of NR-UL/NR-SL prioritization for NR-UL/LTE-SL prioritization. That is, LTE-SL transmission is prioritized if the highest priority value of NR UL LCH(s) with available data is larger than the NR UL priority threshold and the highest priority value of SL LCH(s) with available data is lower than the LTE SL priority threshold. Otherwise the NR-UL transmission is prioritized. By this way, no change is needed for LTE-V2X spec.

- Prioritization between LTE-UL and NR-SL

  - NR-SL and LTE-SL has threshold for UL/SL prioritization, while LTE-UL has no threshold for UL/SL prioritization like NR-UL does. So, LTE-UL/NR-SL prioritization can reuse the method for LTE-UL/LTE-SL prioritization, i.e., if the priority of NR-SL is below the threshold, LTE UL is prioritized; otherwise, NR SL is prioritized. By this way, no change is needed for LTE-Uu spec.

Proposal 4: The rule for NR-UL-data/NR-SL-data prioritization is reused for NR-UL-data/LTE-SL-data prioritization.

Proposal 5: The rule for LTE-UL-data/LTE-SL-data prioritization is reused for LTE-UL-data/NR-SL-data prioritization.

# 3 Conclusion

Observation: Following current agreement for multiplexing UL BSR and SL BSR, there are two issues:

- First, when SL BSR is prioritized, and the BS of all prioritized SL-BSR entries + UL-BSR is more than the UL grant size, if we follow legacy behaviour (PHR is prioritized over SL BSR), the prioritized part of SL BSR may not be able to transmit if p

- Second, when SL BSR is prioritized, and the BS of all prioritized SL-BSR entries + UL-BSR is less than the UL grant size, UL BSR cannot be transmitted even though the SL-BSR include many BS entries with a lower priority than UL BSR.

Based on our analysis, the proposals are below:

Proposal 1: When SL BSR is prioritized, and the BS of all prioritized SL-BSR entries + UL-BSR is more than the UL grant size, UL BSR and truncated SL BSR are included in the UL grant (instead of follow legacy behaviour).

Proposal 2: When SL BSR is prioritized, and the BS of all prioritized SL-BSR entries + UL-BSR is less than the UL grant size, prioritized SL BSR entry and UL BSR are included in the UL grant (instead of follow legacy behaviour).

Proposal 3: The rule for UL-data/SL-data prioritization is reused for UL-BSR/SL-data prioritization.

Proposal 4: The rule for NR-UL-data/NR-SL-data prioritization is reused for NR-UL-data/LTE-SL-data prioritization.

Proposal 5: The rule for LTE-UL-data/LTE-SL-data prioritization is reused for LTE-UL-data/NR-SL-data prioritization.

# 4 Reference

- RAN2#107bis Chairman’s note