# 3GPP TSG-RAN WG2 Meeting #108						  R2-1914902

# Reno, NV, USA, November 18 – November 22, 2019		 resubmission of R2-1913763

Agenda item:		6.20.2 (TEI16)

Source:	LG Electronics Inc.

Title: 	Reduced PDCCH monitoring in CA

Document for:	Discussion and Decision

1.	Introduction

This document discusses potential enhancement of DRX operation considering different purpose of Short DRX and Long DRX.

2.	Discussion

With current specification, when CA is configured, the UE configures multiple cells, and has to monitor PDCCH on multiple cells. The UE consumes its power in order to monitor multiple PDCCHs. The more the cells the UE needs to monitor, the more the UE consumes its power. This behaviour is shown in Figure 1.

Figure 1. Legacy DRX operation in CA

As the most power consumed in unused serving cell is coming from PDCCH monitoring, it is necessary to reduce power consumption in CA configuration by reducing PDCCH monitoring in unused serving cells.

To reduce power consumption when CA is configured, one potential enhancement is that the UE monitors PDCCH on only one specific cell, e.g. SpCell, among all activated cells during an On Duration, and extends PDCCH monitoring on all activated cells during Active Time within the DRX cycle if a PDCCH is received on the specific cell during the On Duration.

The proposed DRX operation is quite well aligned with bursty traffic characteristics where the traffic is small at the start or end of the burst while the traffic is large in the middle of the burst.

An example of the proposed DRX operation is shown in Figure 2.

Figure 2. Reduced PDCCH monitoring in CA

Thus, we propose that RAN2 considers the above explained method as a potential enhancement for DRX operation in CA.

Proposal: When CA is configured, the UE monitors PDCCH on only one specific cell, e.g. SpCell, among all activated cells during On Duration, and, if a PDCCH is received on the specific cell during the On Duration, extends PDCCH monitoring on all activated cells during Active Time within the DRX cycle.

3.	Proposal

In this document, we discuss potential enhancement of DRX operation in CA, and propose followings:

Proposal: When CA is configured, the UE monitors PDCCH on only one specific cell, e.g. SpCell, among all activated cells during On Duration, and, if a PDCCH is received on the specific cell during the On Duration, extends PDCCH monitoring on all activated cells during Active Time within the DRX cycle.

Potential changes for the NR MAC specification is shown below (assuming that a specific serving cell is SpCell).

5.7 Discontinuous Reception (DRX) The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for a SpCell or all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6]. RRC controls DRX operation by configuring the following parameters: - drx-onDurationTimer: the duration at the beginning of a DRX Cycle; - drx-SlotOffset: the delay before starting the drx-onDurationTimer; - drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity; - drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received; - drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received; - drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts; - drx-ShortCycle (optional): the Short DRX cycle; - drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle; - drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity; - drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity. When a DRX cycle is configured, the Active Time includes the time while: - drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in clause 5.1.5) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4); or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clause 5.1.4). When DRX is configured, the MAC entity shall: 1> if a MAC PDU is received in a configured downlink assignment: 2> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 2> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. 1> if a MAC PDU is transmitted in a configured uplink grant: 2> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission; 2> stop the drx-RetransmissionTimerUL for the corresponding HARQ process. 1> if a drx-HARQ-RTT-TimerDL expires: 2> if the data of the corresponding HARQ process was not successfully decoded: 3> start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL. 1> if a drx-HARQ-RTT-TimerUL expires: 2> start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL. 1> if a DRX Command MAC CE or a Long DRX Command MAC CE is received: 2> stop drx-onDurationTimer; 2> stop drx-InactivityTimer. 1> if drx-InactivityTimer expires or a DRX Command MAC CE is received: 2> if the Short DRX cycle is configured: 3> start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-InactivityTimer or in the first symbol after the end of DRX Command MAC CE reception; 3> use the Short DRX Cycle. 2> else: 3> use the Long DRX cycle. 1> if drx-ShortCycleTimer expires: 2> use the Long DRX cycle. 1> if a Long DRX Command MAC CE is received: 2> stop drx-ShortCycleTimer; 2> use the Long DRX cycle. 1> if the Short DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or 1> if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset: 2> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. 1> if the MAC entity is in Active Time: 2> if the Active Time is due to only drx-onDurationTimer running: 3> monitor the PDCCH on the SpCell as specified in TS 38.213 [6]; 2> else: 3> monitor the PDCCH on all the activated Serving Cells as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission: 3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. 2> if the PDCCH indicates a UL transmission: 3> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission; 3> stop the drx-RetransmissionTimerUL for the corresponding HARQ process. 2> if the PDCCH indicates a new transmission (DL or UL): 3> start or restart drx-InactivityTimer in the first symbol after the end of the PDCCH reception. 1> in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause: 2> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]. 1> if CSI masking (csi-Mask) is setup by upper layers: 2> in current symbol n, if drx-onDurationTimer would not be running considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause: 3> not report CSI on PUCCH. 1> else: 2> in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause: 3> not report CSI on PUCCH and semi-persistent CSI on PUSCH. Regardless of whether the MAC entity is monitoring PDCCH or not, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] when such is expected. The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).